3GPP TSG-RAN WG4 Meeting # 99-e	R4-2110698

Electronic Meeting, May. 19-27, 2021

Title:	TP to TR 38.717-04-02: Addition of CA_n25-n66-n71-n77

Source:	Nokia, T-Mobile USA

Agenda item:	8.14.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-04-01 to add CA_n25A-n66A-n71A-n77A with 2UL

************************************* Start of TP*****************************************

## 6.x	CA_n25-n41-n71-n77

### 6. x.1	Operating bands for CA

Table 6.7.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n25A-n66A-n71A-n77A | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

CA_n25A-n66A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n25A-n71A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n25A-n77A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n66A-n71A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n66A-n77A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n71A-n77A 2UL defined in 38.101-1 Table 5.5A.3.1-1

#### 6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported bandwidths per CA band combination for 4DL inter-band CA

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n66A-n71A-n77A | CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.x.3		Specific for 2 bands UL CA

#### 6.x.3.1	UE Co-existence studies

For 4DL/2UL NR CA, only the IMD issues due to dual uplink operation of two bands falling into the DL of the third and fourth band shall be verified.

Co-existence studies for dual uplink operation of two bands, i.e. CA_n25A-n66A, CA_n25A-n71A, CA_n25A-n77A, CA_n66A-n71A, CA_n66A-n77A and CA_n71A-n77A have been captured in TS 38.101-1, Table 6.5A.3.2.3-1 for spurious emission and Table 7.3A.5-1 interband reference sensitivity or provided below, where:

- For CA_n25A-n66A Protected bands list is already in specification

- TR 38.717-02-01-040 CA_n25A-n71A there are no IMD issues affecting own Rx frequencies.

- TR 38.717-02-01-040 CA_n25A-n77A, the 2nd and 4th order IMDs may fall into Rx frequencies of band n25.

Further, the 4th order and 5th order IMDs may fall into Rx frequencies of band n77.

Protected bands list is already in TR 38.717-02-01-040

- For CA_n66A-n71A two UL co-ex analysis is captured into 38.716-02-00 Table 6.4.2.1-1. and from there we can see that there is IMD4 falling on top of n66 DL. There is pretty similar combination in EN-DC, DC_66A_n71A where MSD is defined for B66. Because the IMD mechanisms and frequencies are exactly the same, we propose to reuse that MSD number for n66.

- For CA_n66A-n77A Protected bands list is already in specification

- For CA_n71A-n77A Protected bands list is already in specification

#### 6.x.3.2	REFSENS requirements

There are no additional MSD requirements for this band combination.

************************************* End of TP*****************************************