3GPP TSG-RAN WG4 Meeting#99      	R4-2110704

E-meeting, 19th – 27th  May, 2021

Title:	TP to TR 38.717-03-01: Addition of CA_n5-n14-n77

Source:	Nokia, AT&T

Agenda item:	8.11.2

Document for:	Approval

# Introduction

This is a TP into TR 38.717-03-01 to introduce CA_n5-n14-n77.

********************** Start of TP *********************************

## 6.x	CA_n5-n14-n77

### 6.x.1	Operating bands for CA

Table 6.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n5-n14-n77 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n14 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n5-n14-n77 | - | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.x.3	Co-existence studies

Table 6.x.3-1 summarizes frequency ranges where harmonics occur due to 3DL bands CA with 1 UL. There is 4th HAM from n5 UL to n77 DL. MSD is already defined for this in 38.101-1

Table 6.x.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 |
| 14 | 2305 | 2315 | 2350 | 2360 | 4610 | 4630 | 6915 | 6945 | 9220 | 9260 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

Table 6.x.3-2 gives harmonic mixing issue for the 3DL bands CA with 1 UL. There is 4th HAM from n5 UL to n77 DL. MSD is already defined for this in 38.101-1.

Table 6.x.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 |
| 14 | 2305 | 2315 | 2350 | 2360 | 4700 | 4720 | 7050 | 7080 | 9400 | 9440 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

For single uplink, the UE coexistence is already considered for these bands in TS 38.101-1.

### 6.x.4	∆TIB,c and ∆RIB,c values

It is proposed to re-use TIB,c and RIB,c  values from DC_13-n77, DC_5_n77 and CA_5-13.

Table 6.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n14-n77 | n5 | 0.5 |
|  | n14 | 0.3 |
|  | n77 | 0.8 |

Table 6.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n14-n77 | n5 | 0.2 |
|  | n14 | 0.2 |
|  | n77 | 0.5 |

### 6.x.5	REFSENS requirements

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

********************** End of TP *********************************