3GPP TSG-RAN WG4 Meeting #99-e	R4-2110893

Electronic Meeting, 19 – 27 May, 2021

Title: 	Additional link level simulation results for SRS-RSRP

Source: 	Huawei, HiSilicon

Agenda item:	6.5.2.3.2

Document for:	Discussion

Introduction

RRM accuracy requirements for gNB SRS-RSRP measurements have been discussed in RAN4#98-bis-e, and the conclusions are captured in the WF [1]. Based on [1] the baseline SRS-RSRP accuracy will be decided based on link level simulation results, and updated link level simulation assumptions are also approved in [2].

In this paper we will provide our additional simulation results on SRS-RSRP measurement performance. The new results are marked in red.

Discussion

Simulation assumption

The simulation assumption is same as in [2].

- the SINR side conditions are +3dB and -13dB

- the number of samples is 1

- the propagation channel is AWGN

Simulation results

In Table 1-5, we show the SRS-PSRP estimation accuracy for 2 different SNR level with different SCS, where SNR1=3dB and SNR3=-13dB. The results are for absolute SRS-RSRP accuracy, which is derived as the max(5%, 95%) value on the CDF curve for delta-RSRP.

Table 1: Link level simulation results for SRS-RSRP estimation with 15kHz

| RB | Comb and Symb | AWGN |  |
| --- | --- | --- | --- |
|  |  | SNR1 | SNR2 |
| 24 | 2+1 | 0.7 | 10.1 |
|  | 2 + 2 | 0.5 | 4.4 |
|  | 4 + 4 | 0.5 | 5.1 |
|  | 4+12 | 0.3 | 3.1 |
|  | 8 + 8 | 0.5 | 7.0 |
|  | 8 + 12 | 0.4 | 4.0 |
| 52 | 2+1 | 0.5 | 4.5 |
|  | 2 + 2 | 0.4 | 3.4 |
|  | 4 + 4 | 0.3 | 3.4 |
|  | 4+12 | 0.2 | 2.3 |
|  | 8 + 8 | 0.3 | 3.3 |
|  | 8 + 12 | 0.3 | 3.0 |
| 104 | 2+1 | 0.3 | 3.4 |
|  | 2 + 2 | 0.2 | 2.7 |
|  | 4 + 4 | 0.2 | 2.7 |
|  | 4+12 | 0.2 | 1.7 |
|  | 8 + 8 | 0.3 | 2.6 |
|  | 8 + 12 | 0.2 | 2.4 |
| 264 | 2+1 | 0.2 | 2.5 |
|  | 2 + 2 | 0.2 | 1.8 |
|  | 4 + 4 | 0.1 | 1.9 |
|  | 4+12 | 0.1 | 1.2 |
|  | 8 + 8 | 0.2 | 1.8 |
|  | 8 + 12 | 0.1 | 1.6 |

Table 2: Link level simulation results for SRS-RSRP estimation with 30kHz

| RB | Comb and Symb | AWGN |  |
| --- | --- | --- | --- |
|  |  | SNR1 | SNR2 |
| 48 | 2+1 | 0.5 | 4.5 |
|  | 2 + 2 | 0.4 | 3.5 |
|  | 4 + 4 | 0.4 | 3.4 |
|  | 4+12 | 0.2 | 2.4 |
|  | 8 + 8 | 0.4 | 3.5 |
|  | 8 + 12 | 0.3 | 3.2 |
| 132 | 2+1 | 0.3 | 3.2 |
|  | 2 + 2 | 0.2 | 2.4 |
|  | 4 + 4 | 0.2 | 2.5 |
|  | 4+12 | 0.1 | 1.6 |
|  | 8 + 8 | 0.2 | 2.4 |
|  | 8 + 12 | 0.2 | 2.1 |
| 272 | 2+1 | 0.2 | 2.4 |
|  | 2 + 2 | 0.2 | 1.8 |
|  | 4 + 4 | 0.1 | 1.8 |
|  | 4+12 | 0.1 | 1.2 |
|  | 8 + 8 | 0.2 | 1.8 |
|  | 8 + 12 | 0.1 | 1.6 |

Table 3: Link level simulation results for SRS-RSRP estimation with 120kHz

| RB | Comb and Symb | AWGN |  |
| --- | --- | --- | --- |
|  |  | SNR1 | SNR2 |
| 32 | 2+1 | 0.6 | 8.0 |
|  | 2 + 2 | 0.4 | 4.2 |
|  | 4 + 4 | 0.4 | 4.2 |
|  | 4+12 | 0.3 | 2.8 |
|  | 8 + 8 | 0.4 | 4.3 |
|  | 8 + 12 | 0.4 | 3.6 |
| 64 | 2+1 | 0.4 | 4.1 |
|  | 2 + 2 | 0.3 | 3.1 |
|  | 4 + 4 | 0.3 | 3.3 |
|  | 4+12 | 0.2 | 2.1 |
|  | 8 + 8 | 0.3 | 3.2 |
|  | 8 + 12 | 0.3 | 2.9 |
| 132 | 2+1 | 0.3 | 3.2 |
|  | 2 + 2 | 0.2 | 2.4 |
|  | 4 + 4 | 0.2 | 2.5 |
|  | 4+12 | 0.1 | 1.5 |
|  | 8 + 8 | 0.2 | 2.5 |
|  | 8 + 12 | 0.2 | 2.1 |

Table 4: Link level simulation results for SRS-RSRP estimation with 60kHz FR1

| RB | Comb and Symb | Channel |  |
| --- | --- | --- | --- |
|  |  | AWGN |  |
|  |  | SNR1 | SNR2 |
| 48 | 2+1 | 0.5 | 4.7 |
|  | 2 + 2 | 0.4 | 3.5 |
|  | 4 + 4 | 0.4 | 3.6 |
|  | 4+12 | 0.2 | 2.3 |
|  | 8 + 8 | 0.4 | 3.6 |
|  | 8 + 12 | 0.3 | 3.0 |
| 132 | 2+1 | 0.3 | 3.1 |
|  | 2 + 2 | 0.2 | 2.5 |
|  | 4 + 4 | 0.2 | 2.5 |
|  | 4+12 | 0.1 | 1.5 |
|  | 8 + 8 | 0.2 | 2.5 |
|  | 8 + 12 | 0.2 | 2.1 |

Table 5: Link level simulation results for SRS-RSRP estimation with 60kHz FR2

| RB | Comb and Symb | Channel |  |
| --- | --- | --- | --- |
|  |  | AWGN |  |
|  |  | SNR1 | SNR2 |
| 132 | 2+1 | 0.3 | 3.2 |
|  | 2 + 2 | 0.2 | 2.4 |
|  | 4 + 4 | 0.2 | 2.5 |
|  | 4+12 | 0.1 | 1.6 |
|  | 8 + 8 | 0.2 | 2.5 |
|  | 8 + 12 | 0.2 | 2.1 |
| 264 | 2+1 | 0.2 | 2.5 |
|  | 2 + 2 | 0.2 | 1.7 |
|  | 4 + 4 | 0.2 | 1.9 |
|  | 4+12 | 0.1 | 1.1 |
|  | 8 + 8 | 0.2 | 1.9 |
|  | 8 + 12 | 0.1 | 1.5 |

From the tables, we can observe that

Observation 1: The performance is very dependent on SNR conditions.

The difference in accuracy between +3dB and -13dB side condition can be 3~5dB. As the SRS BW increases, the gap becomes smaller.

Observation 2: There is a performance difference between different comb and symbol sizes.

It can be observed that the accuracy for comb+symbol 2+2, 4+4 and 8+8 are very similar. The reason is that for a given BW, the number of SRS REs for these combinations is same, so the processing gain is similar. The performance with 8+12 is in general better than the other cases due to more SRS REs.

The performance difference is more obvious when comparing results between 2+1 (6 REs) and 4+12 (36 REs), and the gap is very large at low Es/Iot condition and with small RB number. For 32 RB, the gap can be >5dB. With larger RB number the gap becomes smaller, but it is still >1dB with 272 RB.

Observation 3: The accuracy improves in proportion with BW in RB and the impact of SCS is small.

This is aligned with the expectation and aligned with SS-RSRP accuracy for UE. This observation is also confirmed by the new results for 60kHz SCS.

Conclusions

In this paper we provided our simulation results for SRS-RSRP measurement performance.

Observation 1: The performance is very dependent on SNR conditions.

Observation 2: There is a performance difference between different comb and symbol sizes.

Observation 3: The accuracy improves in proportion with BW in RB and the impact of SCS is small.

Reference

- R4-2105755, WF on gNB positioning measurement requirements, Ericsson

- R4-2105758,Updated link level simulation assumption for gNB positioning measurement performance, Huawei, HiSilicon