3GPP TSG-RAN WG4 Meeting #99-e	R4-2110905

Electronic Meeting, 19 – 27 May, 2021

Title: 	Discussion on CSI-SINR accuracy requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.6.2.2.3

Document for:	Discussion

Introduction

CSI-SINR accuracy requirements were discussed in RAN4#98-bis-e, and the outcomes are captured in the WF [1]. Based on [1] the following general issues are to be further discussed:

- Upper bound of Es/Iot for side condition #2

In this paper we will provide our views on the above open issue based on additional simulation results.

Discussion

- Specify CSI-SINR accuracy requirement based on the following conditions on timing offset between the reference measurement timing and the target CSI-RS (TΔ) and Es/Iot side condition(s) - Side condition #1: - ¦TΔ¦≤ CP/2 and Es/Iot ≤ 15 dB - Side condition #2 - ¦TΔ¦≤ CP and Es/Iot ≤ [6] dB - Note: the value of Es/Iot can be revisited based on additional simulation results in next meeting. - No dedicated test cases will be introduced for side condition #2

Based on RAN4#98-bis-e discussions, two side conditions on the timing offset will be defined for CSI-SINR accuracy requirements, namely |TΔ|≤ CP/2 and |TΔ|≤ CP. The accuracy values will be same for the two conditions, but the upper bound of Es/Iot will be different.

In our previous paper [2], we found that the accuracy requirements cannot be achieved with |TΔ|≤ CP and Es/Iot at 6dB. In this paper, we provide additional simulation results for Es/Iot < 6dB. The simulation results are based on assumptions in [3]. The 5%- and 95%-tile of CSI-SINR error are shown in Table 1~3 for 15kHz, 30kHz and 120kHz, respectively.

Table 1: CSI-SINR accuracy with 15kHz SCS

| Timing error | SNR | Channel |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | AWGN |  | ETU70 |  |
|  |  | 5% | 95% | 5% | 95% |
| CP | 6 | -2.71 | -1.22 | -3.07 | -0.63 |
|  | 5 | -2.34 | -0.80 | -2.73 | -0.43 |
|  | 4 | -2.03 | -0.42 | -2.44 | -0.14 |

Table 2: CSI-SINR accuracy with 30kHz SCS

| Timing error | SNR | Channel |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | AWGN |  | EPA5 |  |
|  |  | 5% | 95% | 5% | 95% |
| CP | 6 | -2.78 | -1.25 | -3.84 | -0.25 |
|  | 5 | -2.48 | -0.73 | -3.19 | 0.08 |
|  | 4 | -2.17 | -0.38 | -2.69 | 0.34 |

Table 3: CSI-SINR accuracy with 120kHz SCS

| Timing error | SNR | Channel |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDLA-30-3 |  | TDLC-30-30 |  |
|  |  | 5% | 95% | 5% | 95% |
| CP | 6 | -3.76 | -0.05 | -3.19 | -0.59 |
|  | 5 | -3.32 | 0.18 | -2.81 | -0.27 |
|  | 4 | -2.83 | 0.37 | -2.47 | 0.02 |

From the tables, it can be observed that accuracy requirements can be achieved with |TΔ|≤ CP at Es/Iot 4dB.

Proposal: The Es/Iot upper bound for side condition #2 in CSI-SINR accuracy requirements is 4dB.

Conclusions

In this paper we provided our views on remaining issues in CSI-SINR accuracy requirements.

Proposal: The Es/Iot upper bound for side condition #2 in CSI-SINR accuracy requirements is 4dB.

Reference

- R4-2105769, WF on performance requirements of CSI-RS based L3 measurement, CATT, OPPO

- R4-2105769, Discussion on CSI-SINR accuracy requirements, Huawei, HiSilicon

- R4-2103725, Updated link-level simulation assumption for CSI-RS based L3 measurement, CATT