3GPP TSG-RAN WG4 Meeting # 99-e	R4-2111043

# Electronic Meeting, May 19 - 27, 2021

Source: 	Huawei

Title: 	TP to TR 38.860: B1/B2 background

Agenda Item:	10.4.1

Document for:	Approval

# Introduction

In this TP to TR38.860 we provide background information on the B1 and B2 options, plus editorials corrections identified in the TR. TR template leftovers were also removed.

In case other options are to be captured in the TR (e.g. 2Ba), it is proposed to separate their descriptions from the feasibility analyses, for consistency with the approach to B1/B2.

It is proposed to agree on the attached TP to TR 38.860.

# TP to TR 38.860

The following TP to TR 38.860 is proposed for approval.

------------------------------ Modified section ------------------------------

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

----------------------------- Next modified section ------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]				RP-202143	LS on Frequency arrangements for IMT in the band 470-703MHz, APT

[3]	APT/AWG APT/AWG/REP-79: “APT Report on Frequency Arrangements for IMT in the Band 470-698 MHz”, Sep. 2017

[4]				Radio Regulation, ITU, Edition of 2020

[5]				APT report on frequency arrangements for IMT in the band 470 – 698 MHz, APT, 2017

[6]				3GPP TR 36.755: “US 600 MHz Band for LTE”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

APT	Asia-Pacific Telecommunity

AWG	APT Wireless Group

DTV	Digital Television

FDD	Frequency Division Duplex

RAS	Radio Astronomy Service

----------------------------- Next modified section ------------------------------

# 4	Background

## 4.1	General

Referring to the APT LS to 3GPP RAN and RAN4 in [2], AWG-26 agreed to undertake the study to revise the APT/AWG/REP-79 [3] to develop frequency arrangements in the band 470 - 703 MHz for APT Members that wish to implement both the APT700 and a 600 MHz frequency arrangements that is optimal for APT Members. In the LS [2], AWG asked 3GPP to provide feedback on the technical feasibility of B1 and B2 options, before the 27th APT Wireless Group (AWG-27) meeting, scheduled in Q1 2021.

Furthermore, for option B2 including four different suggested sizes of the second duplexer overlapping with the US600 band n71, feedback on different suggested sizes of the second duplexer from a feasibility and cost-efficient perspective were requested by APT.

Details of options B1 and B2 are based on inputs provided by APT LS in [2] and provided in clauses 4.2 and 4.3, respectively.

RAN4 feasibility analyses for options B1, B2 and B2a are provided in clauses 4.4 to 4.6, respectively. RAN4 decided not to provide any final recommendation on the frequency arrangements to apply in the frequency range in question. Decision on the frequency arrangement is to be taken by the AWG regulatory body.

## 4.2	Background: option B1 description

This option is based on an extension to band n71 and is shown on figure 4.2-1 below:

Figure 4.2-1: Harmonized band plan for 470 - 698 MHz band - Option B1

In this option the frequency arrangement in Option A (i.e. existing n71 frequency arrangement) is extended upwards and downwards by 5 MHz being a better utilization of spectrum that is achieved with existing APT band that is 703 - 748 (UL) paired with 758 - 803 MHz (DL) (otherwise the spectrum 698 - 703 MHz is not fully utilized).

Some countries in Region 3 have Radio Astronomy Service (RAS) on a co-primary basis (Radio Regulation No. 5.305 and No. 5.307, as captured in clause 5.1).

- Duplex direction: The extended band proposal is a reverse duplex and in the lower part of the FDD band (below the duplex gap) is down link as compared to up link that is the ordinary convention but this is better suited for co-existence with APT 700 MHz band (band n28) at the upper edge of the band but also with possible broadcasting at the lower edge.

- Duplex distance: The duplex distance is 51 MHz. The frequency plan can support full band or split duplexer as is already the case for band n28.

- Channel bandwidth: The revised proposal will result in 2 x 40 MHz of FDD spectrum.

- Carrier bandwidths supported: The extended bandwidth will support NR carrier bandwidths up to 30 MHz and LTE bandwidths up to 20 MHz.

## 4.3	Background: option B2 description

This option uses two duplexers as shown in Fig 4.3-1 and the duplex distance is 46 MHz. Also the bottom duplexer is the same as that of band n71, with an additional upper duplexer that should have as large possible overlap as possible with the lower duplexer but at the same time being able to handle the duplex gap of 6 MHz. The size of this upper duplexer needs to be studied and decided prior to being specified. It can be considered to have variable bandwidths ranging from 35 MHz to 10 MHz respectively. Like option B1, this arrangement B2 is also a reversed duplex arrangement.

Figure 4.3-1: Option B2 - two duplexers

Option B2 retains economies of scale advantages by adopting one duplexer and the duplex distance being the same as that of n71, but the duplex gap is now reduced to 6 MHz.

----------------------------- Next modified section ------------------------------

# 5	Regulatory landscape

## 5.1	General

The 470 - 694 MHz frequency range is allocated to the broadcasting service and mobile service on a co-primary basis in Region 3 [4]. The frequency band 470 - 698 MHz, or parts thereof, was identified by WRC-15 in 7 countries in Region 3 through footnote No. 5.296A for use by those administrations as listed wishing to implement terrestrial IMT systems. In addition, there was interest from other significant markets to do the same. Elsewhere, USA, Mexico and several other countries in Region 2 also identified this band for IMT through footnotes 5.295 and 5.308A. It is noted that resolves 2 of Resolution 224 (Rev.WRC-19) encourages the following:

encourages administrations to take into account results of the existing relevant ITU Radiocommunication Sector studies, when implementing IMT applications/systems in the frequency bands

- 694-862 MHz in Region 1,

- in the frequency band 470-806 MHz in Region 2,

- in the frequency band 790-862 MHz in Region 3,

- in the frequency band 470-698 MHz, or portions thereof, for those administrations mentioned in No. 5.296A, and

- in the frequency band 698-790 MHz, or portions thereof, for those administrations mentioned in No. 5.313A.

The regulatory landscape overview for all regions in frequency range 470 – 694 MHz is provided in table 5.1-1, based on ITU radio regulations [4].

Table 5.1-1: ITU-R frequency allocation to services [4]

----------------------------- Next modified section ------------------------------

## 5.2	ITU Region 3

ITU Region 3 specific regulatory aspects for the frequency range 470 – 694 MHz are provided in this clause.

Referring to the APT LS, AWG-26 agreed to undertake the study to revise the APT/AWG/REP-79 [3] to develop frequency arrangements in the band 470-703 MHz for APT Members that wish to implement both the APT700 and a 600 MHz frequency arrangements that is optimal for APT Members.

----------------------------- Next modified section ------------------------------

-

----------------------------- End of modified section ------------------------------