3GPP TSG-RAN WG4 Meeting #99-e	R4-2111237

# Online,  , May 19 - 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.174 | CR | draftCR | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 38.174 draftCR CSI reporting radiated performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Creation of IAB-MT CSI reporting radiated performance requirements in the 38.174 radiated conformance testing specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the IAB-MT CSI reporting radiated performance requirements, i.e., the sections of General, CQI, PMI, RI, as per agreed worksplit [R4-2103994]. As far as configurations PMI and RI requiremnts are not completely agreed yet, we cross over some text to show the difference from the UE specification. Additionally, yellow highlights are used for the missing references and ot emphasies important places. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No radiated conformance testing of CSI reporting performance requirements for IAB-MTs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.176-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Agenda item 6.3.6.3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 11	Radiated performance requirements

## 11.2	IAB-MT performance requirements

<<Start of change>>

### 11.2.3	CSI reporting requirements

#### 11.2.3.1	Performance requirements for IAB type 1-O

##### 11.2.3.1.1	Reporting of Channel Quality Indicator (CQI)

Apply the requirements defined in clause 8.2.3.1 for 2Rx.

##### 11.2.3.1.2	Reporting of Precoding Matrix Indicator (PMI)

Apply the requirements defined in clause 8.2.3.2 for 2Rx.

##### 11.2.3.1.3	Reporting of Rank Indicator (RI)

Apply the requirements defined in clause 8.2.3.3 for 2Rx.

#### 11.2.3.2	Performance requirements for IAB type 2-O

##### 11.2.3.2.1	General

This clause includes radiated requirements for the reporting of channel state information (CSI).

###### 11.2.3.2.1.1	Applicability of requirements

11.2.3.2.1.1.1	General

The minimum performance requirements are applicable to the FR2 operating bands defined in TS 38.101-2 [7], with FDL_high not exceeding 40000 MHz.

The minimum performance requirements in Clause 11 are mandatory for IAB-MT supporting NR operation, except test cases listed in Clause 11.2.3.2.1.1.3.

Unless otherwise stated, the minimum performance requirements in Clause 11.2.3.2.3 and 11.2.3.2.4  shall apply only, if PMI/RI reporting is declared to be supported (see D.XXX in table x.x-x).

11.2.3.2.1.1.2		Applicability of requirements for different number of RX antenna ports

IAB-MT shall support 2 RX ports for different RF operating bands. The UE requirements applicability is defined in Table 11.2.3.2.1.1.2-1.

Table 11.2.3.2.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports 2RX antenna | CQI | All tests in Clause 11.2.3.2.2 |
|  | PMI | All tests in Clause 11.2.3.2.3 |
|  | RI | All tests in Clause 11.2.3.2.4 |

11.2.3.2.1.1.3	Applicability of requirements for mandatory IAB-MT features with capability signalling

The performance requirements in Table 11.2.3.2.1.1.3-1 shall apply for IAB-MTs which support mandatory UE features with capability signalling only.

Table 11.2.3.2.1.1.3-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR2 | CQI | Clause 11.2.3.2.2.2 | The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed UE PDSCH MIMO layers capability |
|  |  | RI | Clause 11.2.3.2.4 |  |
| Support of 1 port PTRS (onePortsPTRS) | FR2 | CQI | Clause 11.2.3.2.2 |  |
|  |  | PMI | Clause 11.2.3.2.3 |  |
|  |  | RI | Clause 11.2.3.2.4 |  |

###### 11.2.3.2.1.2	Common test parameters

Parameters specified in Table 11.2.3.2.1.2-1 are applied for all test cases in this clause unless otherwise stated.

Table 11.2.3.2.1.2-1: Test parameters for CSI test cases

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| Duplex Mode |  |  |  | TDD |
| PTRS epre-Ratio |  |  |  | 0 |
| Actual carrier configuration |  | Offset between Point A and the lowest usable subcarrier on this carrier (Note 3) | RBs | 0 |
|  |  | Subcarrier spacing | kHz | 120 |
| DL BWP configuration #1 |  | Cyclic prefix |  | Normal |
|  |  | RB offset | RBs | 0 |
|  |  | Number of contiguous PRB | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-2 [7] for tested channel bandwidth and subcarrier spacing |
| Active DL BWP index |  |  |  | 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | DMRS ports indexes |  |  | {1000} for Rank1 {1000,1001} for Rank2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 2 |
| PTRS configuration | Frequency density (KPT-RS) |  |  | 2 |
|  | Time density (LPT-RS) |  |  | 1 |
|  | Resource Element Offset |  |  | 2 |
| NZP CSI-RS for CSI acquisition |  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
| Number of HARQ Processes |  |  |  | 8 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| Symbols for unused REs |  |  |  | OP.1 TDD as defined in Annex A.5.2.1 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Note 1: PDSCH is scheduled only on ‘D’ slots without CSI-RS resource and TRS allocated Note 2: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-2 [7] for tested channel bandwidth and subcarrier spacing. |  |  |  |  |

##### 11.2.3.2.2	Reporting of Channel Quality Indicator (CQI)

###### 11.2.3.2.2.1	General

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective conditions is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [11]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

Table 11.2.3.2.2.1-1: Test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| Default TDD UL-DL pattern (Note 1) |  |  |  | 3D1S1U |  |  |  |
| Special Slot Configuration |  |  |  | 10D+2G+2U |  |  |  |
| SNRBB |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 8/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 8/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 1 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 8/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 8/3 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-1, TBS.1-2 |  |  |  |
| Note 1: The same requirements are applicable to with different UL-DL patterns. Note 2: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 [x] are left up to test implementation, if transmitted or needed. |  |  |  |  |  |  |  |

###### 11.2.3.2.2.2	Minimum requirements

For the parameters specified in Table 11.2.3.2.1.1-1, and using the downlink physical channels specified in Annex TBA, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

##### 11.2.3.2.3	Reporting of Precoding Matrix Indicator (PMI)

###### 11.2.3.2.3.1	General

The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reports compared to the case when the transmitter is using random precoding, respectively. When the transmitter uses random precoding, for each PDSCH allocation a precoder is randomly generated and applied to the PDSCH. A fixed transport format (FRC) is configured for all requirements.

The requirements for transmission mode 1 with 2TX and higher layer parameter codebookType set to 'typeI-SinglePanel' are specified in terms of the ratio

In the definition of γ, for 2TX PMI requirements, is 90 % of the maximum throughput obtained at  using the precoders configured according to the UE reports, and is the throughput measured at with random precoding.

Table 11.2.3.2.3.1-1: Test parameters

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 100 | 100 |
| Subcarrier spacing |  | kHz | 120 | 120 |
| TDD DL-UL configuration |  |  | FR2.120-2 as specified in Annex A.1.3 | FR2.120-1 as specified in Annex A.1.3 |
| Default TDD UL-DL pattern (Note 1) |  |  | 3D1S1U | 3D1S1U |
| Special Slot Configuration |  |  | 10D+2G+2U | 10D+2G+2U |
| Propagation channel |  |  | TDLA30-35 | TDLA30-35 |
| Antenna configuration |  |  | 2 x 2 ULA Low | 2 x 2 ULA Low |
| Beamforming Model |  |  | As specified in Annex B.4.1 | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (8,-) | Row 4, (8,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) | (13,-) |
|  | CSI-RS interval and offset | slot | 8/1 | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3, (6,-) | Row 3, (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) | (13,-) |
|  | CSI-RS interval and offset | slot | Not configured 8/1 | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 1 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (8,13) | (8,13) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured | Not configured |
| ReportConfigType |  |  | Aperiodic Periodic | Aperiodic |
| CQI-table |  |  | Table 1 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 8 |
| csi-ReportingBand |  |  | 111111111 | 111111111 |
| CSI-Report interval and offset |  | slot | Not configured 8/3 | Not configured |
| Aperiodic Report Slot Offset |  |  | 6 Not configured | 8 |
| CSI request |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A |
|  | CodebookSubsetRestriction |  | 001111 | 001111 |
|  | RI Restriction |  | N/A | N/A |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| FRC/ CSI reference measurement channels (A.4) |  |  | M-FR2-A4-11 | R.PDSCH.5-7.1 TDD |
| Note 1: The same requirements are applicable to with different UL-DL patterns. Note 2: For random precoder selection, the precoder shall be updated in each slot (0.125 ms granularity). Note 3: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 are left up to test implementation, if transmitted or needed. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4)], this reported PMI cannot be applied at the gNB downlink before slot#(n+4)]. Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

###### 11.2.3.2.3.2	Minimum requirements

For the parameters specified in Table 11.2.3.2.3.1-1, and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified in Table 11.2.3.2.3.2-1.

Table 11.2.3.2.3.2-1: Minimum requirement

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
|  | 1.05 | 1.05 |

##### 11.2.3.2.4	Reporting of Rank Indicator (RI)

###### 11.2.3.2.4.1	General

The purpose of this test is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission.

The minimum performance requirement in Table 11.2.3.2.4.2-1 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

Table 11.2.3.2.4.1-1: Test parameters

| Parameter |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 | 100 | 100 |
| Subcarrier spacing |  |  | kHz | 120 | 120 | 120 |
| Duplex Mode |  |  |  | TDD | TDD | TDD |
| TDD Slot Configuration |  |  |  | FR1.120-2 | FR1.120-2 | FR1.120-2 |
| Default TDD UL-DL pattern (Note 1) |  |  |  | 3D1S1U | 3D1S1U | 3D1S1U |
| Special Slot Configuration |  |  |  | 10D+2G+2U | 10D+2G+2U | 10D+2G+2U |
| SNR |  |  | dB | 0 | 16 | 16 |
| Propagation channel |  |  |  | TDLA30-35 | TDLA30-35 | TDLA30-35 |
| Antenna configuration |  |  |  | ULA Low 2x2 | ULA Low 2x2 | XP High 2x2 |
| Beamforming Model |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 4, (8,-) | Row 4, (8,-) | Row 4, (8,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (13,-) | (13,-) | (13,-) |
|  | CSI-RS interval and offset |  | slot | 8/1 | 8/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Aperiodic Periodic | Aperiodic Periodic | Aperiodic Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 2 | 2 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig interval and offset |  | slot | Not configured 8/1 | Not configured 8/1 | Not configured 8/1 |
|  | aperiodicTriggeringOffset |  |  | 0 | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | Pattern 1 | Pattern 1 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8,13) | (8,13) | (8,13) |
|  | CSI-IM timeConfig interval and offset |  | slot | Not configured | Not configured | Not configured |
| ReportConfigType |  |  |  | Aperiodic Periodic | Aperiodic Periodic | Aperiodic Periodic |
| CQI-table |  |  |  | Table 1 | Table 1 | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | 8 | 8 |
| csi-ReportingBand |  |  |  | 111111111 | 111111111 | 111111111 |
| CSI-Report interval and offset |  |  | slot | Not configured 8/3 | Not configured 8/3 | Not configured 8/3 |
| Aperiodic Report Slot Offset |  |  |  | 7 Not configured | 7 Not configured | 7 Not configured |
| CSI request |  |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  |  | CodebookSubsetRestriction |  | 010000 for fixed rank 2, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank | 000011 for fixed rank 1, 010011 for following rank |
|  |  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 1.375 | 1.375 | 1.375 |
| Maximum number of HARQ transmission |  |  |  | 1 | 1 | 1 |
| RI Configuration |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI |
| FRC/ CSI reference measurement channels (A.4) |  |  |  | M-xx | M-xx | M-xx |
| Note 1: The same requirements are applicable to with different UL-DL patterns. Note 2: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 are left up to test implementation, if transmitted or needed. Note 2: Measurements channels are specified in Table A.4-1. TBS.1-1 is used for Rank 1 case. TBS.1-2 is used for Rank 2 case. |  |  |  |  |  |  |

###### 11.2.3.2.4.2	Minimum requirements

For the parameters specified in Table 11.2.3.2.4.1-1, and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified in Table 11.2.3.2.4.2-1.

Table 11.2.3.2.4.2-1: Minimum requirement

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | 1.05 | 1.05 |
| 2 | 1.0 | N/A | N/A |

<<END of change>>