3GPP TSG-RAN WG4 Meeting #99-e                                          R4-2111430

Electronic Meeting, May 19-27, 2021

Source: 	Huawei, HiSilicon

Title: 	On synchronization reference source

Agenda Item:	9.14.5.4

Document for:	Approval

# Introduction

In last meeting, there is a discussion about which one is used as the synchronization resource, i.e, GNSS based synchronization or network based synchronization for the case that SL and Uu operate in the same licensed band. Since there is no conclusion about the issue, we provide some further investigations based on the agreements from RAN1 in this contribution.

# Discussion

Based on RAN1 agreement in RAN1#96:

Agreements:

- Whether GNSS-based synchronization or gNB/eNB-based synchronization is used is (pre)-configured.

  - The following table is a working assumption

| GNSS-based synchronization | gNB/eNB-based synchronization |
| --- | --- |
| - P0: GNSS - P1: the following UE has the same priority: - UE directly synchronized to GNSS - P2: the following UE has the same priority: - UE indirectly synchronized to GNSS - P3: the remaining UEs have the lowest priority. | - P0: gNB/eNB - P1’: UE directly synchronized to gNB/eNB - P2’: UE indirectly synchronized to gNB/eNB - P3’: GNSS - P4’: UE directly synchronized to GNSS - P5’: UE indirectly synchronized to GNSS - P6’: the remaining UEs have the lowest priority. |

Based on RAN1 agreement in RAN1#98

Agreements:

- For confirmation of the working assumption of synchronization priority rules, eNB/gNB should be included into the priority order of GNSS-based synchronization.

| GNSS-based synchronization | gNB/eNB-based synchronization |
| --- | --- |
| • P0: GNSS • P1: UE directly synchronized to GNSS • P2: UE indirectly synchronized to GNSS • P3: gNB/eNB • P4: UE directly synchronized to gNB/eNB • P5: UE indirectly synchronized to gNB/eNB • P6: the remaining UEs have the lowest priority. | • P0’: gNB/eNB • P1’: UE directly synchronized to gNB/eNB • P2’: UE indirectly synchronized to gNB/eNB • P3’: GNSS • P4’: UE directly synchronized to GNSS • P5’: UE indirectly synchronized to GNSS • P6’: the remaining UEs have the lowest priority. |

Based on RAN2 specification in TS38.331：

### 5.8.12 DFN derivation from GNSS When the UE selects GNSS as the synchronization reference source, the DFN, the subframe number within a frame and slot number within a frame used for NR sidelink communication are derived from the current UTC time, by the following formulae: DFN= Floor (0.1*(Tcurrent –Tref–OffsetDFN)) mod 1024 SubframeNumber= Floor (Tcurrent –Tref–OffsetDFN) mod 10 SlotNumber= Floor ((Tcurrent –Tref–OffsetDFN)*2μ) mod (10*2μ) Where: Tcurrent is the current UTC time that obtained from GNSS. This value is expressed in milliseconds; Tref is the reference UTC time 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Thursday, December 31, 1899 and Friday, January 1, 1900). This value is expressed in milliseconds; OffsetDFN is the value sl-OffsetDFN if configured, otherwise it is zero. This value is expressed in milliseconds.

Based on RAN1 agreement, there are two standalone lists for synchronization priority as GNSS-based synchronization or gNB/eNB-based synchronization. When a UE is (pre)-configured with GNSS-based synchronization, the UE would prefer GNSS as synchronization resource. When a UE is (pre)-configured with gNB/eNB -based synchronization, the UE would prefer network as synchronization resource. Also based on RAN1 agreement, it is up to (pre)-configuration whether GNSS-based synchronization or gNB/eNB-based synchronization is used.

When SL and Uu operate in the same licensed band, it is up to network configuration to determine GNSS-based synchronization or gNB/eNB-based synchronization is used. On the one hand, it is nature that a UE can be configured by network with the network based synchronization for intra-band con-current V2X operating bands. On the other hand, when GNSS is known by network, it is still reasonable that a UE is (pre)-configured with GNSS-based synchronization for intra-band con-current V2X operating bands, because there is an ‘OffsetDFN’ defined in TS38.331 which is indicated by network to make sure that there is a common understanding for SL and Uu about which slot is used for SL transmission. Then it is up to network implementation to determine the synchronization list configuration for a SL UE, no more specification is needed.

Observation1: there is an agreement reached in RAN1#96 as follow:

- Whether GNSS-based synchronization or gNB/eNB-based synchronization is used is (pre)-configured.

Proposal 1: RAN4 follow the existing RAN1 design on sync reference source and the agreement as well as corresponding information is captured in the TR for NR-V2X.

# Conclusion

This contribution provides more information on synchronization reference source.

Observation1: there is an agreement reached in RAN1#96 as follow:

- Whether GNSS-based synchronization or gNB/eNB-based synchronization is used is (pre)-configured.

Proposal 1: RAN4 follow the existing RAN1 design on sync reference source and the agreement as well as corresponding information is captured in the TR for NR-V2X.

# References

[1] R4-2105404, WF on synchronization issue for SL and Uu operated in the same licensed band