3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2111489

Electronic Meeting, May 19-27, 2021

Source:	T-Mobile USA

Title:	TP for TR38.841: PC2 CA_n25A-n77A

Agenda item:	8.36.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 2DL/2UL CA_n25A-n77A as well as PC2 2DL/1UL DL CA_n25A-n77A UL n77A according to the request in [1].

The MSD is the same as that for CA_2A-n77A with PC2 for n77.

- Reference

[1] RP-210476, Revised basket WID: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2)

- Text Proposal

<Beginning of changes>

### 5.7.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n66A-n77A requirements are applied for PC2 CA_n66A-n77A.

## 5.x		CA_n25-n77

### 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n77A | CA_n25A-n77A | n25 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | n77 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |

### 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n25A-n77A | CA power class | Carrier n25 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 5.x.3	REFSENS requirements

According to the PC3 CA_n25A-n77A study, the 2nd harmonic mixing products from band 2 may fall into band n77 UL frequency range. And, the 2nd, 4th and 5th order IMD products generated by dual uplink from both band 2 and band n77 may fall into band 2 Rx frequency range. Thus additional MSD should be considered to mitigate the impact of the interference for PC2 CA_n25A-n77A combination.

#### 5.x.3.1	Power class 2 Case A

The MSD due to receiver harmonic mixing for PC2 Case A are same as PC3 CA_n25A-n77A.

The additional MSD due to intermodulation for PC2 CA_n25A-n77A are defined in table 5.x.3.1-1. These were copied from the PC2 MSD for CA_2A-n77A in 5.5.

Table 5.x.3.1-1: MSD test points for PCell due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 32.10 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 19.10 | FDD | IMD44 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.x.3.2	Power class 2 Case B

The additional MSD due to receiver harmonic mixing for Case B are defined in table 5.x.3.2-1.

Table 5.x.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for NR CA  in NR FR1

|  | NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band |  | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 |  | n25 | 9.1 | 8.0 | 7.0 | 6.7 |  |  |  |  |  |  |  |  |  |

The additional MSD due to intermodulation for PC2 Case B CA_n25A-n77A are same as the Case A defined in table 5.x.3.1-1.

### 5.x.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n25A-n77A requirements are applied for PC2 CA_n25A-n77A.

<End of changes>