3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2111491

Electronic Meeting, May 19-27, 2021

Source:	T-Mobile USA

Title:	TP for TR38.841: PC2 CA_n71A-n77A

Agenda item:	8.36.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 2DL/2UL CA_n71A-n77A as well as PC2 2DL/1UL DL CA_n17A-n77A UL n77A according to the request in [1].

- Reference

[1] RP-210476, Revised basket WID: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2)

- Text Proposal

<Beginning of changes>

### 5.11.4	Void

## 5.x		CA_n71-n77

### 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71A-n77A | CA_n71A-n77A | n71 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n71A-n77A | CA power class | Carrier n71 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n71A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 5.x.3	REFSENS requirements

According to the PC3 CA_n71A-n77A study, there is MSD due to harmonic mixing and cross band isolation.

#### 5.x.3.1	Power class 2 case a

Comparing with PC3 CA_n71A-n77A, no additional MSD are expected for this PC2 CA_n71A-n77A with dual or single uplink carrier.

#### 5.x.3.2	Power class 2 case b

Comparing with PC3 CA_n71A-n77A, additional MSD is expected for this PC2 CA_n71A-n77A with dual uplink due to higher Tx power in n77. The MSD was calculated by starting with 5.5 dB MSD for PC3, calculating the interference noise power, and then adding 3 dB to the interference noise power.

Table 5.x.3.2-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n71-n77 | n71 | 671 | 5 | 25 | 625 | 7.8 | FDD | IMD5 |
|  | n77 | 3300 | 10 | 50 | 3300 | N/A | TDD | N/A |

### 5.x.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n71A-n77A requirements are applied for PC2 CA_n71A-n77A.

<End of changes>