3GPP TSG-RAN WG4 Meeting RAN4#99-e     				R4-2111495 Online

April 19 – April 27, 2021

Agenda Item:	6.7.1.1

Source:	Lenovo, Motorola Mobility

Title:	On Defining EVM for Transmit Diversity using the Pseudo-Inverse

Document for:  Discussion

- Introduction

In RAN4#98-e-bis, the following EVM definition was agreed for transmit diversity

EVM=P1 ∙EVM1+P2 ∙EVM2P1+P2

with the understanding that the pseudo-inverse proposed in [1] could be further considered. The pseudo-inverse was also considered previously in [2].  In this contribution, we discuss two issues with respect to the use of the pseudo-inverse to define EVM for transmit diversity.

- Concerns with the Pseudo-Inverse

Issue 1: Channel Dependence

The first issue with using the pseudo-inverse for transmit diversity is that the resulting EVM will be channel dependent.  As mentioned in several previous contributions[3-5], the EVM definition should be independent of the channel between the transmitter and receiver so that the EVM can always be achieved regardless of the channel between the transmitter and a noiseless receiver.

From [2], the signal received at a noiseless gNB receiver is given by

y=H w x+n ,

where w=w1 w2T is the rank-1 precoding vector, x is the data symbol, and n=n1 n2T is the transmitter noise at the two antenna connectors. Because H w has dimension 2x1, its inverse is not uniquely defined. If the gNB receiver applies the pseudo inverse receiver given by

HwHHwHHw

then the estimate of the received symbol is given by

x=HwHHwHHw y

=x+v

where v is the noise at the output of the receiver.

The noise v can be expressed as

v=wHHHHwHHHHw n ,

where n denotes the noise vector at the output of the transmitter.  From Figure 1 below, it can be observed that the noise vector measured by the test equipment is given by

n'=W-1n

where

W=w100w2 .

With this, the noise v at the output of the receiver can also be expressed as

v=wHHHH WwHHHHw n'=dH n',

where

d=WHHHH wwHHHHw .

Figure 1: EVM Measurement

If the pseudo-inverse receiver is used to define EVM for transparent transmit diversity, then the EVM is given by

EVM=100Ev2

=100dHΣ'd

where

Σ'=n' n'H= σ12εε*σ22 .

and

EVM1=100σ12         EVM2=100σ22 .

From this expression, it should be apparent that this EVM definition depends on the channel H between the transmitter and the receiver, and thus is not channel independent.  To demonstrate this fact, we consider a few examples.

Example 1:  The channel H and the precoder w are given by

H=10.910.7  and  w=1-12 .

From this, we have

d=WHHHH wwHHHHw=4-3

and thus

EVM=100dHΣ'd=10016 σ12-24 Reε+9 σ22 .

If σ12=σ22 and ε=0, then

EVM=10025 σ12=5 EVM1=5 EVM2 ,

and the EVM measured at the output of the pseudo-inverse receiver is 5 times greater than the EVM at the first and second antenna connectors.

Example 2:  For the second example, the channel H and the precoder w are given by

H=10.91-0.7  and  w=1-12 .

From this, we have

d=0.62070.3793

and thus

EVM=100dHΣ'd=1000.3853 σ12+0.4708 Reε+0.1439 σ22 .

If σ12=σ22 and ε=0, then

EVM=1000.5291 σ12=0.7274 EVM1=0.7274 EVM2 .

So, by changing 0.7 to -0.7 in H, the EVM is reduced from 5 EVM1 to 0.7274 EVM1.

Example 3:  For the third example, the channel H and the precoder w are given by

H=1001  and  w=1-12 .

From this, we have

d=0.50.5

and thus

EVM=100dHΣ'd=1000.25 σ12+0.5 Reε+0.25 σ22 .

If σ12=σ22 and ε=0, then

EVM=1000.5 σ12=0.707 EVM1=0.707 EVM2 .

As can be seen from the examples above, the EVM definition is highly dependent on the channel between the transmitter and the receiver. In the examples above, the EVM varies by a factor of 7.07 (=5/.707) from the largest to the smallest.  Though for conducted measurements, the channel H will always be approximately

H=1001 ,

or alternatively

H=ejθ100ejθ2 ,

it is our view that the EVM definition should not depend on the channel between the transmitter and the receiver.

Observation 1:  If the pseudo-inverse is used to define and measure EVM, the resulting EVM definition

will depend on the channel between the transmitter and receiver.

Issue 2: Correlated Transmitter Noise

As can be seen from Example 3, a second issue with the use of the pseudo-inverse is that it may yield an EVM which is significantly less than  EVM1 and  EVM2.  For a conductive test, the transmitter noise is likely to be uncorrelated so that ε=0 and

Σ'=n' n'H= σ12εε*σ22= σ1200σ22 .

Additionally, for a conductive test, the test channel is approximately given by

H=1001

or alternatively, by

H=ejθ100ejθ2 .

With w=1 -1T, ε=0 and either of these two test channels, the EVM is given by

EVM=100dHΣ'd=1000.25 σ12+0.5 Reε+0.25 σ22

=0.5 EVM12+EVM22

If EVM1=EVM2, then as in Example 3 above

EVMU=0.707 EVM1=0.707 EVM2 .

as in Example 3 above, where EVMU denotes the EVM with uncorrelated noise.

When the UE is transmitting through its antennas, the transmitter noise is likely to be highly correlated due to antenna coupling and reverse intermodulation.  If we consider the worst case in which ε=σ1σ2 and the channel H is the same as above, we have

EVM=100dHΣ'd=1000.25 σ12+0.5 σ1σ2+0.25 σ22

=0.5 EVM1+EVM2 .

If EVM1=EVM2, then

EVMC=EVM1=EVM2 ,

where EVMC denotes the EVM with correlated noise. It can be noted that EVMC with worst case correlation of the transmitter noise is 1.4 times greater than EVMU with uncorrelated transmitter noise.

Observation 2:  With worst-case correlation of the transmitter noise, the EVM measured using the

pseudo-inverse may underestimate the EVM by a factor as large as 1.4.

- Transmit Diversity EVM Agreement from RAN4#98-e-bis

In RAN4#98-e-bis, the following EVM definition was agreed for transmit diversity

EVM=P1 ∙EVM1+P2 ∙EVM2P1+P2

with the understanding that the pseudo-inverse proposal in [1] could be further considered.  A few observations can be made with respect to this agreed definition.

Observation 3:  The EVM definition agreed in RAN4#98-e-bis does not depend on the channel between

the transmitter and receiver.

As derived in [3,4], the EVM definition assumes that the channel is inverted so that the channel has no impact on the EVM definition so long as the channel H is invertible.

Observation 4:  The EVM definition agreed in RAN4 #98-e-bis assumes the worst-case correlation of

the transmitter noise.  Thus, the definition will not underestimate the EVM even though

it is based on conductive measurements.

The agreed EVM definition from RAN4#98-e-bis does not depend on the correlation of the transmitter noise because it assumes the maximum correlation with the worst-case phase. If we consider the case above in which w=1 -1T2 , then P1=P2, so that

EVM=EVM1+EVM22 .

In the case that EVM1=EVM2, we have

EVM=EVM1=EVM2 .

Note that this EVM value matches the EVM for the pseudo-inverse with w=1 -1T2, H given by

H=1001

and worst-case correlation of the transmitter noise.

- Summary

In RAN4#98-e-bis, the following EVM definition was agreed for transmit diversity

EVM=P1 ∙EVM1+P2 ∙EVM2P1+P2

with the understanding that the pseudo-inverse proposed in [1] could be further considered. In this contribution, we have the following observations with respect to the pseudo-inverse proposal and the as well as the EVM definition agreed in RAN4#98-e-bis.

Observation 1:  If the pseudo-inverse is used to define and measure EVM, the resulting EVM definition

will depend on the channel between the transmitter and receiver.

Observation 2:  With worst-case correlation of the transmitter noise, the EVM measured using the

pseudo-inverse may underestimate the EVM by a factor as large as 1.4.

Observation 3:  The EVM definition agreed in RAN4#98-e-bis does not depend on the channel between

the transmitter and receiver.

Observation 4:  The EVM definition agreed in RAN4 #98-e-bis assumes the maximum possible

correlation of the transmitter noise with worst-case phase.  Thus, the definition will not

underestimate the EVM even though it is based on conductive measurements.

Because defining EVM using the pseudo-inverse will yield an EVM definition which is fundamentally depends on the propagation channel, and because the pseudo-inverse does not address the correlation of the transmitter noise that may not be present in conductive measurements but will be present when the UE transmits through its antennas, we have the following proposal.

Proposal:  Keep the existing agreement in which the EVM for transparent transmit diversity is defined as

EVM=P1 ∙EVM1+P2 ∙EVM2P1+P2

where EVM1 and EVM2 denote the EVM measured at the first and second antenna connectors and P1 and P2 denote the power measured at the first and second antenna connectors.

# References

- R4-2107112, ‘Discussion on FR1 Tx Diversity EVM measurements’, Rohde and Schwartz, 3GPP TSG-RAN WG4 Meeting #98-e-bis

- R4-2008276, ‘Considerations on the EVM Definition for an Antenna Port or a Single MIMO Layer’, Motorola Mobility, 3GPP TSG-RAN WG4 Meeting #95-e.

- R4-2107369, ‘On the EVM Definition for Transmit Diversity and Single Layer Transmission’, Lenovo, Motorola Mobility, TSG-RAN WG4 Meeting #98-e-bis.

- R4-2102917, ‘On the EVM Definition for Transmit Diversity and Antenna Ports,’ Lenovo, Motorola Mobility, TSG-RAN WG4 Meeting #98-e.

- R4-2011519, ‘Further Considerations on the EVM Definition for Antenna Ports Including Transparent Transmit Diversity’, Lenovo, Motorola Mobility,3GPP TSG-RAN WG4 Meeting #96-e.