3GPP TSG-RAN WG4 (Radio) Meeting #73-AH-UE-RF	R4-73AH-0016

Oulu, Finland, 13 – 16 January 2015

Agenda Item:	4.1.1

Source: 	Alcatel-Lucent, Sprint

Title: 	TP for Rel-13 2DL TR 36.8xx: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (25 + 26)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced Carrier Aggregation (CA) of Band Combination (25 + 26) was approved in RAN#66 [1]. One objective of the WI is to specify the band-combinations specific Radio Frequency (RF) requirements for inter-band CA of this combination.

The impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting CA of this band combination to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the Inter-band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<New section>

##### 6.1.X.1.2.1	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 25 and Band 26 DL carriers can be calculated as shown in Table 6.1.X.1.2.1-1 below:

Table 6.1.X.1.2.1-1: Band 25 and Band 26 DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 859 | 894 | 1930 | 1995 |
| 2nd harmonics frequency limits (MHz) | 1718 | 1788 | 3860 | 3990 |
| 3rd harmonics frequency limits (MHz) | 2577 | 2682 | 5790 | 5985 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 1036 | 1136 | 2789 | 2889 |
| 3rd order IMD products | (f2-low – 2*f1-high) | (f2-high – 2*f1-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 142 | 277 | 2966 | 3131 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 3648 | 3783 | 4719 | 4884 |
| 3rd order IMD products | (f1-low – f2-high + f2-low) | (f1-high + f2-high – f2-low) | (f2-low – f1-high + f1-low) | (f2-high + f1-high – f1-low) |
| IMD frequency limits (MHz) | 794 | 959 | 1895 | 2030 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 839 | 914 | 1915 | 2010 |

It can be seen from Table 6.1.X.1.2.1-1 that the 2nd harmonics of BS transmitting in Band 26 may fall into the BS receive band of Bands 3, 4, 9 and 10, and the 3rd harmonics may fall into the BS receive band of Bands 38 and 41, while the 3rd IMD products may fall into the BS receive band of Bands 1, 2, 5, 6, 8, 14, 18, 19, 20, 23, 25, 26, 27, 33, 34, 35, 36, 37, 39, 43, and 44.  Note that the calculation in Table 6.1.X.1.2.1-1 (except the last row) assumes the BS is transmitting with the whole 65 MHz DL frequency of Band 25 and the whole 35 MHz DL frequency of Band 26. If the BS is only transmitting up to 20 MHz DL in Band 25 and up to 15 MHz DL in Band 26 as stated in the WIDS, then the 3rd IMD products will not fall into the BS receive band of Band 2, 14, 18, 25, 27, 34, 35 or 44 as shown in the last row in Table 6.1.X.1.2.1-1, and the 3rd IMD products will not fall into the BS own (up to 15 MHz) receive block within Band 26, and may only fall into the BS receive band of certain frequency range within Bands 5 and 26 under the transmit configurations shown in Table 6.1.X.1.2.1-2 below.

Table 6.1.X.1.2.1-2: Band (25 + 26) BS transmit configurations with 3rd IMD within Bands 5 and 26 BS receive band

| Band 26 DL channel bandwidth (MHz) | Lower edge of Band 26 DL frequency block (MHz) | Band 25 DL channel bandwidth (MHz) | Lower edge of IMD frequency limits (MHz) |
| --- | --- | --- | --- |
| 5, 10 or 15 | 859 – 863.9 | 15 | 844 – 848.9 |
| 5, 10 or 15 | 859 – 868.9 | 20 | 839 – 848.9 |

Note that Bands 1, 3, 6, 8, 9, 19, 20, 33, 38 and 39 are not intended for use in the same geographical area as Bands 25 and 26. Moreover, co-location of Band (25 + 26) transmitter and Band 36 or 37 transceiver implies FDD/TDD co-location on adjacent frequencies which requires the use of certain site-engineering solutions to avoid mutual interference. Therefore, the focus here is on the harmonics and IMD falling into Bands 4, 5, 10, 23, 26, 41 and 43.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 4, 10, 41 or 43 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 25 and 26 BS transmitters do not share the same antenna with Band 4, 10, 41 or 43 BS receiver. But there is not a large frequency gap between Band 25 DL and Band 23 UL, and hence Band 23 BS receiver desensitization may still be an issue.

Therefore, it is recommended that Bands 25 and 26 BS transmitters should not share the same antenna with Band 4, 10, 23, 41 or 43 BS receiver, or with Band 5 or 26 BS receiver for the affected frequency ranges if the aforementioned BS transmit configurations are used, in order to prevent BS receiver desensitization, unless the antenna path meets very stringent 2nd and 3rd order PIM specification so that the PIM will not cause Band 4, 5, 10, 26, 41 or 43 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of section>

# References

[1]	RP-142020, “New WID: LTE Advanced inter-band Carrier Aggregation of Band 25 and Band 26”, Sprint.

[2]	R4-73AH-0015, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (25 + 26)”, Alcatel-Lucent, Sprint.

[3]	R4-147551, “Rel-13 2DL TR 36.8xx: LTE-A Inter-band Carrier Aggregation v0.2.0”, Ericsson.