TSG-RAN Working Group 4 (Radio) meeting #73 UE RF AH	R4-73AH-0085

Oulu, Finland, 13 - 16 January 2015

Source:	Nokia Networks, Nokia Corporation

Title:	Uplink channel bandwidth combinations for 3DL/2UL CA

Agenda item:	4.6	[LTE Advanced Inter-band Carrier Aggregation 3DL/2UL pairing]

4.3	[2UL non-contiguous intra-band CA requirements]

3		[LTE_CA_C_B40_3DL-Core], [LTE_CA_C_B41_3DL-Core]

Document for:	Discussion

# Introduction

2UL contiguous CA was in the scope of 3DL contiguous CA WIs for Band 40 and 41 [1, 2].

Since the supported bandwidth combination set is not signalled separately for DL and UL in UE capability signalling, it is unclear what bandwidth combinations are specified for 2UL CCs in the current RAN4 specifications.

In general, this issue is not only the contiguous CA but also for inter-band CA and non-contiguous intra-band CA once 3DL/2UL CAs (or even 4DL/2UL) are introduced. In this paper, we discuss a wayforard to resolve the issue.

# Discussion

## Contiguous CA 3DL/2UL (DL CA_xD, UL CA_xC)

The following table is the bandwidth combination sets defined for contiguous CA for Band 40 and Band 41.

Currently up to 2 CCs are supported for uplink in 3DL contiguous CA class D, thus Class C is applied in uplink. The bandwidth combinations for uplink is not clear enough in the current specification, since the supported bandwidth combination sets are not signalled separately for UL and DL unlike the CA bandwidth class. This is actually indicated in NOTE 2 of the table. The note was introduced in [8] to explain such RAN2 signalling behaviour. At that time, there was no case that UL and DL CA configurations are different, i.e., either UL CA is the same as DL CA or it is configured as a single carrier uplink, so the supported UL configurations were clear in the 3GPP specifications. However, the note is not clear any more.

According to the WID for CA_41D [2], the assumption is to use the configuration of CA_41C for uplink. However, regarding the bandwidth combinations, CA_41C has now two bandwidth combination sets after the set 1 was added for CA_41A-41C and CA_41C-41A [3,7]. So it is not clear which combination set is applied for uplink, although set 0 was the only one when the CA_41D WI was created.

Furthermore, the existing CA_41C channel bandwidth combinations do not cover all the fallback configurations of 3DL configuration CA_41D [4]. For example, 10+15 and 15+10 are not defined within the CA_41C combinations, but they are the possible fallback modes of CA_41D. Therefore further clarification is required to explain if these configurations are supported or not in the uplink.

We also observe the similar issue for CA_40D, in which the bandwidth combination sets for uplink is not clear enough in the current specification. For Band 40, the fallback combination set (i.e., set 1) has been defined in CA_40C [5, 6], therefore this set 1 could be referred as uplink configuration, but it is not clearly specified how uplink channel bandwidth combinations are defined.

Table 2.1

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_40C | CA_40C | 10 | 20 |  | 40 | 0 |
|  |  | 15 | 15 |  |  |  |
|  |  | 20 | 10, 20 |  |  |  |
|  |  | 10, 15 | 20 |  | 40 | 1 |
|  |  | 15 | 15 |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |
| CA_40D | CA_40C | 10, 15, 20 | 20 | 20 | 60 | 0 |
|  |  | 20 | 10, 15 | 20 |  |  |
|  |  | 20 | 20 | 10, 15 |  |  |
| CA_41C | CA_41C | 10 | 20 |  | 40 | 0 |
|  |  | 15 | 15, 20 |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |
|  |  | 5, 10 | 20 |  | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |
| CA_41D | CA_41C | 10 | 20 | 15 | 60 | 0 |
|  |  | 10 | 15, 20 | 20 |  |  |
|  |  | 15 | 20 | 10, 15 |  |  |
|  |  | 15 | 10, 15, 20 | 20 |  |  |
|  |  | 20 | 15, 20 | 10 |  |  |
|  |  | 20 | 10, 15, 20 | 15, 20 |  |  |
| NOTE 1: The CA configuration refers to an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 3: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |  |  |  |

Here’s the four possible ways forward to clarify this issue.

- Introduce a new UE capability signalling in the RAN2 specification to indicate which bandwidth combination sets are supported in the uplink.

- Reuse the 2DL/2UL channel bandwidth combination sets for the uplink CA part of 3DL/2UL CA. This means CA_40C set 1 is used for 2UL with 3DL CA_40D. A new CA_41C set needs to be defined to include the missing fallback modes. Without RAN2 signalling, the supported channel bandwidth combinations in the uplink needs to be defined within the RAN4 specification somehow.

- Clarify in the RAN4 specification that all the 2 CC fallback bandwidth combinations of the supported bandwidth combination sets in 3CC CA are supported in the 2 UL CA.[Note: This means that we do not refer CA_41C’s bandwidth combination sets from CA_41D.]

- Apply the fallback signalling, i.e., the supported bandwidth combination sets in DL CA_40C is applied to UL CA_41C (when DL is CA_41D). [This probably means we need further clarification in RAN2 and/or RAN4 spec, but the signalling itself does not need to be modified.]

The alternative 3 seems to be the simplest solution in terms of the impact to RAN4 specifications. So the draft CR for this alternative is attached in Annex. If this is acceptable in the group, a CR can be submitted in Athens meeting. Nevertheless, we would like to discuss in the Adhoc if other solutions should also be considered further or not.

In the following sections, other cases of 3DL/2UL CAs are discussed because the wayforward should be generic to other 3DL/2UL CA cases as well.

## Inter-band CA 3DL/2UL (DL CA_xA-yA-zA; UL CA_xA-yA, CA_yA-zA or CA_xA-zA)

Currently there is no 3DL/2UL CA defined for inter-band CA. The following table for CA_1A-3A-5A is taken as an example from Table 5.6A.1-2a of TS36.101.

Table 2.2

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 5) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-5A | - | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 40 | 1 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |

Let us now assume we introduce 3DL/2UL CA for this example. We can have 2UL CA such as CA_1A-3A, CA_1A-5A or CA_3A-5A for the uplink. For such CA, the uplink channel bandwidth combinations can be easily defined using the table above, because the channel bandwidths are specified separately for each band.

For example, if only one bandwidth combination set, i.e., set 1, is supported for 3DL CA, the channel bandwidth for band 1 is 5 or 10MHz; the channel bandwidth for band 3 is 5, 10, 15, or 20MHz. So this would be the possible bandwidth combinations for CA_1A-3A in the uplink.

Therefore, we can conclude that we do not need to refer the bandwidth combination set defined in 2 CC CA, i.e., CA_1A-3A in Table 5.6A1-2 when 3DL/2UL CA is introduced; we can just rely on the 3-band table, 5.6A1-2a, for the supported uplink 2CC bandwidth combinations.

## Inter-band CA 3DL/2UL (DL CA_xA-yC; UL CA_xA-yA or CA_yC)

Currently there is no 3DL/2UL CA defined for this case. The following table for CA_1A-42C is taken as an example of 3DL/1UL CA from Table 5.6A.1-2 of TS36.101.

Table 2.3

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 4) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set | Dual uplink CA capability |
| CA_1A-42C | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 | No |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |

The possible 2UL CA configurations would be CA_42_C and CA_1A-42A. For the case of CA_42C in the uplink, the supported bandwidth combinations in the uplink are trivial as you can see in the Table 5.6A.1-1. However, the supported bandwidth combination set for the case of CA_1A-42A may not be trivial.

There is only one bandwidth combination set in Table 5.6A.1-2 is defined today for CA_1A-42A; so there would be no question of what bandwidth combinations are supported even when such CA is introduced. However, it is better to clarify how the uplink bandwidth combinations are derived because more bandwidth combination sets could be added in the future.

## Non-contiguous CA 3DL/2UL (UL CA_xA-xC; UL CA_xA-xA or CA_xC)

Currently there is no 3DL/2UL CA defined for the non-contiguous CA. The following table for CA_41A-41C is taken as an example of 3DL/1UL CA from Table 5.6A.1-3 of TS36.101.

Table 2.4

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set | uplink CA capability |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |  |
| CA_41A-41C | - | 5, 10, 15, 20 | See Table 5.6A.1-1 |  | 60 | 0 | No |

The possible 2UL CA configurations would be CA_41_C and 41A-41A. For CA_41C, the supported uplink bandwidth combinations would be taken from Table 5.6A.1-1 as mentioned in the table. However, the bandwidth combination set for the case of CA_41A-41A is not trivial. This needs be clarified when such UL CA is introduced.

## Non-contiguous CA 3DL/2UL (DL CA_xA-xA-xA; UL CA_xA-xA)

Currently non-contiguous CA with three distinct blocks is not defined in RAN4 specifications. For this case, CA_xA-xA would be the only possible configuration for 2UL CA. Such case needs to be taken into account in our discussion to check if the wayforward can be generic enough or not.

# Conclusion

It has been discussed that the uplink channel bandwidth combinations are not clearly specified in the current RAN4 specifications for the case of 2 UL and 3 DL. Several possible wayfoward has been presented in Section 2.1. The draft CR to TS36.101 is attached in Annex based on one of the proposed alternatives. This wayfoward is generic enough to be applied to other CA cases such as inter-band CA and non-contiguous CA.

If this wayfoward is acceptable in the group, a CR can be submitted in Athens meeting.

# Reference

- RP-140100	Revised WID:  LTE Advanced Intra-band Contiguous Carrier Aggregation in Band 41 for 3 DL, Sprint

- RP-140948	Revised WID Proposal: LTE Advanced intra-band contiguous Carrier Aggregation in Band 40 for 3DL, CMCC

- RP-140101	Revised WID:  LTE Advanced intra-band Non-contiguous Carrier Aggregation in Band 41 for 3 DL, Sprint

- RP-140943 (R4-142437)	CR for TS 36.101 on introduction CA_41D, Sprint

- RP-140603	CR for TS 36.101 on introduction CA_40D, CMCC, ZTE

- RP-141560 (R4-144082)	Adding 15MHz channel BW to B40 3DL and new bandwidth combination set for the 2DL, TeliaSonera

- RP-140944 (R4-143954)	CR to TS 36.101 on introduction of 3DL intra-band non-contiguous CA requirements, Samsung, Sprint, Alcatel-Lucent

- RP-121304 (R4-124834)	Bandwidth combination sets for intra-band and inter-band carrier aggregation, Ericsson, ST-Ericsson, Nokia Corporation

ANNEX: Draft CR

3GPP TSG- RAN WG4 Meeting #73 UE RF AH	R4-73AH-xxxx

# Oulu, Finland, 13-16 January 2015

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR |  | rev |  | Current version: | 12.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| d |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Uplink channel bandwidth combinations for 3DL/2UL CA |  |  |  |  |  |  |  |  |  |
| Source to WG: | Nokia Networks, Nokia Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_C_B40-Core, LTE_CA_C_B41-Core |  |  |  |  |  |  | Date: |  | 2015-01-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | 2UL CA is supported in 3DL contiguous CA for Band 40 and 41. However, it is unclear which channel bandwidth combinations are supported in uplink. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The notes are modified to indicate that UL channel bandwidth combinations are constructed as fallback of 3DL channel bandwidth combinations. It is expected 3DL/2UL CAs are also introduced in inter-band and intra-band non-contigous CAs. Therefore, the note is also introduced in a generic way for all the channel bandwidth combination set tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear which UL CC bandwidth combinations are supported by UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 5.6A.1	Channel bandwidths per operating band for CA

The requirements for carrier aggregation in this specification are defined for carrier aggregation configurations with associated bandwidth combination sets. For inter-band carrier aggregation, a carrier aggregation configuration is a combination of operating bands, each supporting a carrier aggregation bandwidth class. For intra-band contiguous carrier aggregation, a carrier aggregation configuration is a single operating band supporting a carrier aggregation bandwidth class.

For each carrier aggregation configuration, requirements are specified for all bandwidth combinations contained in a bandwidth combination set, which is indicated per supported band combination in the UE radio access capability. A UE can indicate support of several bandwidth combination sets per band combination.

Requirements for intra-band contiguous carrier aggregation are defined for the carrier aggregation configurations and bandwidth combination sets specified in Table 5.6A.1-1. Requirements for inter-band carrier aggregation are defined for the carrier aggregation configurations and bandwidth combination sets specified in Table 5.6A.1-2 and Table 5.6A.1-2a. Requirements for intra-band non-contiguous carrier aggregation are defined for the carrier aggregation configurations and bandwidth combination sets specified in Table 5.6A.1-3.

The DL component carrier combinations for a given CA configuration shall be symmetrical in relation to channel centre unless stated otherwise in Table 5.6A.1-1, Table 5.6A.1-2 and Table 5.6A.1-2a.

Table 5.6A.1-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_1C | CA_1C | 15 | 15 |  | 40 | 0 |
|  |  | 20 | 20 |  |  |  |
| CA_2C |  | 5 | 20 |  | 40 | 0 |
|  |  | 10 | 15, 20 |  |  |  |
|  |  | 15 | 10, 15, 20 |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |
| CA_3C | CA_3C | 5, 10, 15 | 20 |  | 40 | 0 |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |
| CA_7C | CA_7C | 15 | 15 |  | 40 | 0 |
|  |  | 20 | 20 |  |  |  |
|  |  | 10 | 20 |  | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |
| CA_12B | - | 5 | 5, 10 |  | 15 | 0 |
| CA_23B | - | 10 | 10 |  | 20 | 0 |
|  |  | 5 | 15 |  |  |  |
| CA_27B | - | 1.4, 3, 5 | 5 |  | 13 | 0 |
|  |  | 1.4, 3 | 10 |  |  |  |
| CA_38C | CA_38C | 15 | 15 |  | 40 | 0 |
|  |  | 20 | 20 |  |  |  |
| CA_39C | CA_39C | 5,10,15 | 20 |  | 35 | 0 |
|  |  | 20 | 5, 10, 15 |  |  |  |
| CA_40C | CA_40C | 10 | 20 |  | 40 | 0 |
|  |  | 15 | 15 |  |  |  |
|  |  | 20 | 10, 20 |  |  |  |
|  |  | 10, 15 | 20 |  | 40 | 1 |
|  |  | 15 | 15 |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |
| CA_40D | CA_40C | 10, 15, 20 | 20 | 20 | 60 | 0 |
|  |  | 20 | 10, 15 | 20 |  |  |
|  |  | 20 | 20 | 10, 15 |  |  |
| CA_41C | CA_41C | 10 | 20 |  | 40 | 0 |
|  |  | 15 | 15, 20 |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |
|  |  | 5, 10 | 20 |  | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |
| CA_41D | CA_41C | 10 | 20 | 15 | 60 | 0 |
|  |  | 10 | 15, 20 | 20 |  |  |
|  |  | 15 | 20 | 10, 15 |  |  |
|  |  | 15 | 10, 15, 20 | 20 |  |  |
|  |  | 20 | 15, 20 | 10 |  |  |
|  |  | 20 | 10, 15, 20 | 15, 20 |  |  |
| CA_42C | CA_42C | 5, 10, 15, 20 | 20 |  | 40 | 0 |
|  |  | 20 | 5, 10, 15 |  |  |  |
| NOTE 1: The CA configuration refers to an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal if CA configurations are the same for downlink and uplink. For the cases that uplink CA is in lower order than downlink, all the downlink channel bandwidth combinations are supported in the uplink. NOTE 3: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |  |  |  |

Table 5.6A.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (two bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 4) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set | Dual uplink CA capability |
| CA_1A-3A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 | Yes |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_1A-5A | - | 1 |  |  |  | Yes |  |  | 20 | 0 | Yes |
|  |  | 5 |  |  |  | Yes |  |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 1 | Yes |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
| CA_1A-7A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 | Yes |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-8A | - | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 0 | Yes |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 | Yes |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 2 | No |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |  |
| CA_1A-11A | - | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 0 | No |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |  |
| CA_1A-18A | - | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 | No |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 | No |
|  |  | 18 |  |  | Yes | Yes |  |  |  |  |  |
| CA_1A-19A | - | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 | Yes |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |  |
| CA_1A-20A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 | No |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_1A-21A | - | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 | Yes |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
| CA_1A-26A | - | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 | No |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 | No |
|  |  | 26 |  |  | Yes | Yes |  |  |  |  |  |
| CA_1A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
| CA_1A-41A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |  |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_1A-41C | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6.A.1-1 |  |  |  |  |  |  |  |  |
| CA_1A-42A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 | No |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_1A-42C | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 | No |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |
| CA_2A-4A | - | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 | Yes |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 | Yes |
|  |  | 4 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 2 | Yes |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_2A-4A-4A | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 | No |
|  |  | 4 | See CA_4A-4A in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |
| CA_2A-5A | - | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 | No |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
| CA_2A-2A-5A | - | 2 | See CA_2A-2A in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
| CA_2A-12A | - | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 | No |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 1 | No |
|  |  | 12 |  | Yes | Yes | Yes |  |  |  |  |  |
| CA_2A-12B | - | 2 |  |  | Yes | Yes | Yes | Yes | 35 | 0 | No |
|  |  | 12 | See CA_12B in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |
| CA_2A-13A | - | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 | Yes |
|  |  | 13 |  |  |  | Yes |  |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 | Yes |
|  |  | 13 |  |  |  | Yes |  |  |  |  |  |
| CA_2A-2A-13A | - | 2 | See CA_2A-2A in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 | No |
|  |  | 13 |  |  |  | Yes |  |  |  |  |  |
| CA_2A-17A | - | 2 |  |  | Yes | Yes |  |  | 20 | 0 | No |
|  |  | 17 |  |  | Yes | Yes |  |  |  |  |  |
| CA_2A-29A | - | 2 |  |  | Yes | Yes |  |  | 20 | 0 | No |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 | No |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 2 | No |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |  |
| CA_2C-29A | - | 2 | See CA_2C in table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |  |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |  |
| CA_2A-30A | - | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 | No |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |  |
| CA_3A-5A | - | 3 |  |  |  | Yes | Yes | Yes | 30 | 0 | Yes |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  | 20 | 1 | Yes |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 2 | Yes |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
| CA_3A-7A | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 | Yes |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |  |
| CA_3A-7C | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 | No |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |
| CA_3C-7A | - | 3 | See CA_3C in table 5.6A.1-1 |  |  |  |  |  | 60 | 0 | No |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_3A-8A | - | 3 |  |  |  | Yes | Yes | Yes | 30 | 0 | Yes |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  | 20 | 1 | Yes |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 2 | No |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |  |
| CA_3A-19A | - | 3 |  |  | Yes | Yes | Yes | Yes | 35 | 0 | Yes |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |  |
| CA_3A-20A | - | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 0 | Yes |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 1 | Yes |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_3A-26A | - | 3 |  |  | Yes | Yes | Yes | Yes | 35 | 0 | Yes |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes |  |  | 20 | 1 | Yes |
|  |  | 26 |  |  | Yes | Yes |  |  |  |  |  |
| CA_3A-27A | - | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 0 | No |
|  |  | 27 |  |  | Yes | Yes |  |  |  |  |  |
| CA_3A-28A | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 | No |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_4A-5A | - | 4 |  |  | Yes | Yes |  |  | 20 | 0 | No |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 1 | No |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
| CA_4A-4A-5A | - | 4 | See CA_4A-4A in table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
| CA_4A-7A | - | 4 |  |  | Yes | Yes |  |  | 30 | 0 | Yes |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_4A-4A-7A | - | 4 |  |  | Yes | Yes |  |  | 40 | 0 |  |
|  |  | 4 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_4A-12A | - | 4 | Yes | Yes | Yes | Yes |  |  | 20 | 0 | Yes |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 4 | Yes | Yes | Yes | Yes | Yes | Yes | 30 | 1 | Yes |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 2 | Yes |
|  |  | 12 |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes |  |  | 20 | 3 | Yes |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 4 | Yes |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |  |
| CA_4A-4A-12A | - | 4 | See CA_4A-4A in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 | No |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |  |
| CA_4A-12B | - | 4 |  |  | Yes | Yes | Yes | Yes | 35 | 0 | No |
|  |  | 12 | See CA_12B in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |
| CA_4A-13A | - | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 0 | Yes |
|  |  | 13 |  |  |  | Yes |  |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes |  |  | 20 | 1 | Yes |
|  |  | 13 |  |  |  | Yes |  |  |  |  |  |
| CA_4A-4A-13A | - | 4 | See CA_4A-4A in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 | No |
|  |  | 13 |  |  |  | Yes |  |  |  |  |  |
| CA_4A-17A | - | 4 |  |  | Yes | Yes |  |  | 20 | 0 | Yes |
|  |  | 17 |  |  | Yes | Yes |  |  |  |  |  |
| CA_4A-27A | - | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 0 | No |
|  |  | 27 |  | Yes | Yes | Yes |  |  |  |  |  |
| CA_4A-29A | - | 4 |  |  | Yes | Yes |  |  | 20 | 0 | No |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes |  |  | 20 | 1 | No |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 2 | No |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |  |
| CA_4A-30A | - | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 0 | No |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |  |
| CA_5A-7A | - | 5 | Yes | Yes | Yes | Yes |  |  | 30 | 0 | Yes |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |  |
| CA_5A-12A | - | 5 |  |  | Yes | Yes |  |  | 20 | 0 | Yes |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |  |
| CA_5A-13A | - | 5 |  |  | Yes | Yes |  |  | 20 | 0 |  |
|  |  | 13 |  |  |  | Yes |  |  |  |  |  |
| CA_5A-17A | - | 5 |  |  | Yes | Yes |  |  | 20 | 0 | Yes |
|  |  | 17 |  |  | Yes | Yes |  |  |  |  |  |
| CA_5A-25A | - | 5 |  |  | Yes | Yes |  |  | 30 | 0 | No |
|  |  | 25 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_5A-30A | - | 5 |  |  | Yes | Yes |  |  | 20 | 0 | No |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |  |
| CA_7A-8A | - | 7 |  |  |  | Yes | Yes | Yes | 30 | 0 |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |  |
| CA_7A-12A | - | 7 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |  |
| CA_7A-20A | - | 7 |  |  |  | Yes | Yes | Yes | 30 | 0 | Yes |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes | 40 | 1 | Yes |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_7A-28A | - | 7 |  |  | Yes | Yes | Yes | Yes | 35 | 0 | Yes |
|  |  | 28 |  |  | Yes | Yes | Yes |  |  |  |  |
| CA_8A-11A | - | 8 |  |  | Yes | Yes |  |  | 20 | 0 | No |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |  |
| CA_8A-20A | - | 8 |  |  | Yes | Yes |  |  | 20 | 0 | No |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  | 20 | 1 | No |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |  |
| CA_8A-40A | - | 8 |  |  | Yes | Yes |  |  | 30 | 0 | No |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_11A-18A | - | 11 |  |  | Yes | Yes |  |  | 25 | 0 | No |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |  |
| CA_12A-25A | - | 12 |  |  | Yes | Yes |  |  | 30 | 0 | No |
|  |  | 25 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_12A-30A | - | 12 |  |  | Yes | Yes |  |  | 20 | 0 | No |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |  |
| CA_18A-28A | - | 18 |  |  | Yes | Yes | Yes |  | 25 | 0 | No |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
| CA_19A-21A | - | 19 |  |  | Yes | Yes | Yes |  | 30 | 0 | Yes |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
| CA_19A-42A | - | 19 |  |  | Yes | Yes | Yes |  | 35 | 0 | No |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_19A-42C | - | 19 |  |  | Yes | Yes | Yes |  | 55 | 0 | No |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |
| CA_20A-32A | - | 20 |  |  | Yes | Yes |  |  | 30 | 0 | No |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_23A-29A | - | 23 |  |  | Yes | Yes | Yes | Yes | 30 | 0 | No |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 23 |  |  | Yes | Yes |  |  | 20 | 1 | No |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |  |
| CA_25A-41A | - | 25 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |  |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_25A-41C | - | 25 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |
| CA_26A-41A | - | 26 |  |  | Yes | Yes | Yes |  | 40 | 0 |  |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_26A-41C | - | 26 |  |  | Yes | Yes | Yes |  | 55 | 0 |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6.A.1-1 |  |  |  |  |  |  |  |  |
| CA_29A-30A | - | 29 |  |  | Yes | Yes |  |  | 20 | 0 | No |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |  |
| CA_39A-41A | - | 39 |  |  |  | Yes | Yes | Yes | 40 | 0 | Yes |
|  |  | 41 |  |  |  |  |  | Yes |  |  |  |
| CA_39A-41C | - | 39 |  |  |  | Yes | Yes | Yes | 60 | 0 |  |
|  |  | 41 |  |  |  |  |  | Yes |  |  |  |
|  |  | 41 |  |  |  |  |  | Yes |  |  |  |
| CA_39C-41A | - | 39 | See CA_39C in Table 5.6A.1-1 |  |  |  |  |  | 55 | 0 |  |
|  |  | 41 |  |  |  |  |  | Yes |  |  |  |
| CA_41A-42A | - | 41 |  |  |  | Yes | Yes | Yes | 40 | 0 | No |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal if CA configurations are the same for downlink and uplink. For the cases that uplink CA is in lower order than downlink, all the downlink channel bandwidth combinations are supported in the uplink. NOTE 4: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |  |  |  |  |  |  |  |  |

Table 5.6A.1-2a: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (three bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 5) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-5A | - | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 40 | 1 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-3A-8A | - | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 40 | 1 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes |  | 40 | 2 |
|  |  | 3 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
| CA_1A-3A-19A | - | 1 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-3A-26A | - | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 26 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-3A-20A (NOTE 4) | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-5A-7A | - | 1 |  |  | Yes | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
| CA_1A-7A-20A (NOTE 4) | - | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-18A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 45 | 0 |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 18 |  |  | Yes | Yes |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-19A-21A | - | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
| CA_2A-4A-5A | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-4A-12A | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-4A-13A | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_2A-4A-29A | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-5A-12A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-5A-13A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_2A-5A-30A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-12A-30A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-29A-30A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-7A-20A | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_4A-5A-12A | - | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-5A-13A | - | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_4A-5A-30A | - | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-7A-12A | - | 4 |  |  | Yes | Yes |  |  | 40 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-12A-30A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-29A-30A | - | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_7A-8A-20A | - | 7 |  |  |  | Yes | Yes | Yes | 40 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal if CA configurations are the same for downlink and uplink. For the cases that uplink CA is in lower order than downlink, all the downlink channel bandwidth combinations are supported in the uplink. NOTE 4: A terminal which supports a DL CA configuration shall support all the lower order fallback DL CA combinations and it shall support at least one bandwidth combination set for each of the constituent lower order DL combinations containing all the bandwidths specified within each specific combination set of the upper order DL combination. NOTE 5: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |  |  |  |  |  |  |  |

Table 5.6A.1-3: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with two sub-blocks)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set | uplink CA capability |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |  |
| CA_2A-2A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 | No |
| CA_3A-3A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 | No |
| CA_4A-4A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 | Yes |
| CA_7A-7A | - | 5 | 15 |  | 40 | 0 | No |
|  |  | 10 | 10, 15 |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |
| CA_23A-23A | - | 5 | 10 |  | 15 | 0 | No |
| CA_25A-25A | - | 5, 10 | 5, 10 |  | 20 | 0 | No |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 1 | No |
| CA_41A-41A | - | 10, 15, 20 | 10, 15, 20 |  | 40 | 0 | No |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 1 | No |
| CA_41A-41C | - | 5, 10, 15, 20 | See Table 5.6A.1-1 |  | 60 | 0 | No |
| CA_41C-41A | - | See Table 5.6A.1-1 |  | 5, 10, 15, 20 | 60 | 0 | No |
| CA_42A-42A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 | No |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 2: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal if CA configurations are the same for downlink and uplink. For the cases that uplink CA is in lower order than downlink, all the downlink channel bandwidth combinations are supported in the uplink. |  |  |  |  |  |  |  |

< end of changes >