3GPP TSG-RAN4 Meeting #75 OTA TRP/TRS Ad Hoc     R4-75AH-TRPS-0004

# Venice, Italy, July 1-3 2015

Source: 	SONY Mobile

Title: 		TRP/TRS data from 2 tablets (core bands).

Agenda Item: 	7.1

Document for:	Discussion

# 1.		Introduction R4-125719

At RAN 4 #75 in Fukuoka a request was raised for more UE tablets TRP/TRS measurement data to be provided by members in addition to the data provided by NTT Docomo in[1]. Below data from two different tablet models is given at free space condition. These are GSM/UMTS/LTE devices. All bands are considered core bands. Data is obtained by SGS test-house.

2.	TRP/TRS measured data by test-house SGS.

| Band | Free space | Tablet A, dBm | Tablet B, dBm |
| --- | --- | --- | --- |
| UMTS 1 | TRP | 22.1 | 21.4 |
|  | TRS | -109.8 | -109.8 |
| UMTS 2 | TRP | 22.2 | N/A |
|  | TRS | -110.1 | N/A |
| UMTS 5 | TRP | 20.7 | N/A |
|  | TRS | -107.3 | N/A |
| UMTS 8 | TRP | 21.4 | 19.8 |
|  | TRS | -108.6 | -108.8 |
| LTE 3 10 MHz BW | TRP | 19.6 | 19.9 |
|  | TRS primary | -95.5 | -95.9 |
|  | TRS secondary | -92.0 | -94.1 |
| LTE 7 10 MHz BW | TRP | 19.0 | 19.7 |
|  | TRS primary | -93.8 | -93.7 |
|  | TRS secondary | -90.1 | -91.2 |
| LTE 20 10 MHz BW | TRP | 19.8 | 19.5 |
|  | TRS primary | -89.2 | -95.3 |
|  | TRS secondary | -88.0 | -92.1 |

3.	Conclusions

In response to the R4 #75 request for more tablet UE:s measured data on free space TRP/TRS the above table is provided revealing data from two different models.

5.	References

- R4-152691, NTT Docomo, Tablet requirement of TRP/TRS for UMTS band I and XIX.