3GPP TSG-RAN WG4 Meeting #77 NB-IoT Ad-Hoc            			R4-77AH-IoT-0115

# Budapest, Hungay, 20-22 Jan. 2016

Title:	Simulation result on co-existence between GSM and NB-IoT

Source:	CMCC

Agenda Item:	3.3

Document for:	Discussion

- Introduction

In this paper, we provide simulation results on co-existence between NB-IoT and GSM.

- Simulation results

In the last meeting, a way forward on coexistence simulation assumptions [1] as well as a way forward on how to apply ACLR and ACS on coexistence study for standalone [2] case was agreed.

Following we provide simulation results of interference from NB-IoT DL to GSM DL and from NB-IoT UL to GSM UL respectively.

Figure 1. NB-IoT DL to GSM DL

Table 1. GSM DL outage caused by NB-IoT DL

| ACLR/baseline | 40dB | 45dB | 50dB | 55dB | 60dB | baseline |
| --- | --- | --- | --- | --- | --- | --- |
| Outage% | 4.1 | 2.0 | 1.1 | 1.0 | 0.7 | 0.45 |

- Reference

[1] R4-158207	Way forward on coexistence simulation assumptions, Intel

[2] R4-158210	Way forward on how to apply ACLR and ACS on coexistence study for standalone case, Intel, Ericsson, Huawei, Nokia, Qualcomm