3GPP TSG-RAN WG4#78-NB-IoT	 R4-78AH-0163

Kista, SE, 3 - 4 May 2016

Agenda Item:	6.7

Source: 	Nokia

Title: 		PRACH RRM Requirements for NB-IoT UEs

Document for:	Approval

# Introduction

NB-IoT PRACH transmission uses a 3.75 kHz sub-carrier spacing and consists of symbol groups with frequency hopping between symbol groups. Each symbol group has a cyclic prefix and a preamble. Symbol groups hop by one or six sub-carriers in frequency, and repetitions of groups of symbol groups hop by a pseudo-random number of sub-carriers in frequency. There are two possible cyclic prefix lengths for the random access transmission symbol groups, suitable for different maximum cell sizes [1]. For NB-IoT, PRACH is performed on the anchor carrier on which the UE has received NPSS/NSSS, NPBCH and SIB transmission.

In this paper, we discuss the RRM requirements for NB-IoT UEs.

# Comparison of eMTC PRACH and NB-IoT PRACH procedures

In the running CR for TS 36.321 [2], it is shown that the random access procedure for NB-IoT is actually very similar to that for eMTC, although depending on different RRC-parameters. Thus, from RRM requirement point of view, we may develop the NB-IoT PRACH RRM requirements based on the eMTC PRACH RRM requirements. The following Table 1 shows the comparison of the eMTC and NB-IoT parameters.

Table 1: Comparison of eMTC and NB-IoT PRACH parameters [1]

| Parameters | eMTC | NB-IoT |
| --- | --- | --- |
| powerRampingStep | {dB0, dB2,dB4, dB6} |  |
| powerRampingParameters [3] |  | TBD? |
| preambleTransMax | {3, 4, 5, 6, 7, 8, 10, 20, 50, 100, 200} | {3, 4, 5, 6, 7, 8, 10, 20, 50, 100, 200} |
| preambleInitialReceivedTargetPower | {dBm-120, dBm-118, dBm-116, dBm-114, dBm-112, dBm-110, dBm-108, dBm-106, dBm-104, dBm-102,dBm-100, dBm-98, dBm-96, dBm-94, dBm-92, dBm-90} |  |
| DELTA_PREAMBLE | Defined in 36.321 | 0 |
| maxHARQ-Msg3Tx | {1, 2, 3, 4, 5, 6, 7, 8} | Not support because of the asynchronou and adaptive UL HARQ process, as analyzed in [3] |
| preambleMappingInfoList | Designed by RAN1 | Not support |
| RSRP-ThresholdsPrachInfoList | Maximum 3 values for maximum 4 CE Levels | Maximum [2] values for maximum 3 CE Levels |
| maxNumPreambleAttemptNPRACH | {3, 4, 5, 6, 7, 8, 10} | Re-use: {3, 4, 5, 6, 7, 8, 10} |
| nprach-NumRepetitions | {1, 2, 4, 8, 16, 32, 64, 128} | {1, 2, 4, 8, 16, 32, 64, 128} |
| ra-ResponseWindowSize | LTE: sf {2, 3, 4, 5, 6, 7, 8, 10} eMTC: sf {20, 50, 80, 120, 180, 240, 320, 400} | Using PRACH period instead of sf directly, re-use the value in LTE: pp {2, 3, 4, 5, 6, 7, 8, 10} |
| mac-ContentionResolutionTimer | LTE: sf {8, 16, 24, 32, 40, 48, 56, 64} eMTC: sf {80, 100, 120, 160, 200, 240, 480, 960} | Using pp instead of sf directly, re-use the value in LTE: pp {8, 16, 24, 32, 40, 48, 56, 64} |

The similarity and difference in the Random Access Resource selection between eMTC and NB-IoT are shown more clear from the PRACH procedure in TS 36.321 as illustrated as follows [2]:

If the UE is an eMTC UE:

-	select the Random Access Preambles group, ra-ResponseWindowSize, mac-ContentionResolutionTimer and PRACH resource corresponding to the selected CE level;

If the UE is an NB-IoT UE:

-	select a specific time-frequency PRACH resource (specific IE FFS), ra-ResponseWindowSize and mac-ContentionResolutionTimer based on the corresponding CE level and based on the support for multi-tone transmission.

Therefore, similar with eMTC UEs, we should introduce additional RRM performance requirements for NB-IoT UEs.

# NB-IoT PRACH RRM performance requirements

For eMTC PRACH RRM performance, the new requirements introduced in Section 6.2.3 in TS 36.133 are as follows [4]:

### 6.2.3	Requirements for Cat-M1 UEs

In addition to the requirements defined in 6.2.1 and 6.2.2, a Cat-M1 UE shall also execute the random access procedure defined in clause 5.1 in TS 36.321 [17] using the PRACH configuration contained in PRACH-ConfigSIB in TS 36.331 [2]

-	Determines the enhanced coverage level based on the RSRP measurement and the configured criterion (RSRP-ThresholdsPrach [2]) as defined in section 5.1.1, TS 36,321 [17],

-	Selects PRACH resources [2] configured for the corresponding enhanced coverage level as determined in the previous step and;

-	Transmits or re- transmits PRACH preamble using the selected PRACH resources and PRACH configuration.

Since NB-IoT PRACH procedure is very similar with eMTC PRACH procedure, we propose to introduce the following NB-IoT PRACH RRM requirements into TS 36.133, where the name for the new NPRACH RRC-parameters, i.e., PRACH-ConfigSIB-NB, provided in TS 36.331 running CR [3] are used (See the Appendix for the parameter details of PRACH-ConfigSIB-NB).

### 6.2.4	Requirements for NB-IoT UEs

In addition to the requirements defined in 6.2.1 and 6.2.2, an NB-IoT UE shall also execute the random access procedure defined in clause 5.1 in TS 36.321 [17] using the PRACH configuration contained in PRACH-ConfigSIB-NB in TS 36.331 [2]

-	Determines the enhanced coverage level based on the RSRP measurement and the configured criterion (RSRP-ThresholdsPrach in PRACH-ConfigSIB-NB [2]) as defined in section 5.1.1, TS 36,321 [17],

-	Selects PRACH resources (PRACH-Parameters in PRACH-ConfigSIB-NB [2]) configured for the corresponding enhanced coverage level as determined in the previous step and;

-	Transmits or re- transmits PRACH preamble using the selected PRACH resources and PRACH configuration.

# NB-IoT PRACH transmission power

For legacy UEs, the PRACH transmission requirements are defined in Section 6.2.2 as follows: “The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula defined in TS 36.213[3] and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as specified in table 6.3.5.1.1-1 of TS 36.101[5]. The relative power applied to additional preambles shall have an accuracy as specified in table 6.3.5.2.1-1 of 36.101[5].”

In 36.101 [5], it states “In the case of a PRACH transmission, the absolute tolerance is specified for the first preamble. The absolute power tolerance includes the channel estimation error (the absolute RSRP accuracy requirement specified in subclause 9.1 of TS 36.133)”. At this moment, DL RSRP measurements for NB-IoT are not finalized. Thus, we may also need to check if we can keep same requirements as stated above after RSRP measurement requirements are defined.

Observation 1: Once DL RSRP measurements are defined, we may want to recheck if Section 6.2.2 requirements for PRACH transmission power applies still apply to NB-IoT UEs.

# Summary

In this paper, we discussed the NPRACH RRM requirements for NB-IoT UEs. Based on the discussion, it was proposed to introduce the following NPRACH RRM requirements for NB-IoT UEs.

### 6.2.4	Requirements for NB-IoT UEs

In addition to the requirements defined in 6.2.1 and 6.2.2, an NB-IoT UE shall also execute the random access procedure defined in clause 5.1 in TS 36.321 [17] using the PRACH configuration contained in PRACH-ConfigSIB-NB in TS 36.331 [2]

-	Determines the enhanced coverage level based on the RSRP measurement and the configured criterion (RSRP-ThresholdsPrach in PRACH-ConfigSIB-NB [2]) as defined in section 5.1.1, TS 36,321 [17],

-	Selects PRACH resources (PRACH-Parameters in PRACH-ConfigSIB-NB [2]) configured for the corresponding enhanced coverage level as determined in the previous step and;

-	Transmits or re- transmits PRACH preamble using the selected PRACH resources and PRACH configuration.

# References

- R2-16xxxx,  36.300 Running CR “Introduction of NB-IoT”, Huawei (Rapporteur)

- R2-16xxxx, “36.321 running CR to capture agreements on NB-IoT”, Ericsson

- R2-16xxxx, “36.331 Running CR to capture agreements on NB-IoT”, Huawei (Rapporteur)

- 3GPP TS 36.133

- 3GPP TS 36.101

# Appendix PRACH-ConfigSIB-NB in TS 36.331[3]

The IE PRACH-ConfigSIB-NB is used to specify the PRACH configuration in the system information.

PRACH-Config-NB information elements

PRACH-ConfigSIB-NB-r13 ::=		SEQUENCE {

prach-CPLength-r13					ENUMERATED {us66dot7, us266dot7 },

rsrp-ThresholdsPrachInfoList-r13	RSRP-ThresholdsPrachInfoList-r13,

prach-ParametersList-r13   			PRACH-ParametersList-r13

}

PRACH-ParametersList-r13 ::=	SEQUENCE (SIZE (1..maxCE-Level-NB-r13)) OF PRACH-Parameters-NB-r13

PRACH-Parameters-NB-r13::=  	SEQUENCE {

prach-Periodicity-r13 				ENUMERATED {ms40, ms80, ms160, ms240,

ms320, ms640, ms1280, ms2560 },

prach-StartTime-r13					ENUMERATED {ms8, ms16, ms32, ms64,

ms128, ms256, ms512, ms1024 },

prach-SubcarrierOffset-r13			ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1 },

prach-NumSubcarriers-r13			ENUMERATED { n12, n24, n36, n48},

prach-SubcarrierRangeStart-r13		ENUMERATED { zero, oneThird, twoThird, one},

prach-MaxNumPreambleAttempt-r13		ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1},

prach-NumRepetitions-r13			ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128},

pdcch-NumRepetitions-RA-r13			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1 },

pdcch-startSF-CSS-RA-r13			ENUMERATED { v1dot5, v2, v4, v8, v16, v32, v48, v64}

}

RSRP-ThresholdsPrachInfoList-r13 ::= SEQUENCE (SIZE(1.. maxCE-Level-NB-r13)) OF RSRP-Range