3GPP TSG-RAN WG4 Meeting #79AH	R4-79AH-0157

Hong Kong, China, 28 - 30 June 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR |  | rev | 0 | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NPDSCH RMCs for NB-IOT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  |  | Date: |  | 2016-06-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | RMCs for NB-IOT UEs are currently missing in the specificaiton, and these will be needed in various test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: NPDSCH RMC for normal coverage Change #2: NPDSCH RMC for enhanced coverage |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to specify realistic test cases to verify the new NB-IOT speicifc core requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.512-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---START OF CHANGE 1---

### A.3.1.x	NPDSCH Reference Channel for UE Category NB1

#### A.3.1.x.1	 HD-FDD in Normal Coverage

Table A.3.1.x.1-1: NPDSCH Reference Channel for UE category NB1 HD-FDD in Normal Coverage

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| PDSCH Reference channel |  |  | R.14 HD-FDD | R.15 HD-FDD |
| Carrier bandwidth |  | MHz | 10 | 10 |
| Number of NRS ports |  |  | 1 | 2 |
| Number of CRS ports |  |  | 2 | 2 |
| Allocated resource blocksNote1 |  | PRBs | 1 | 1 |
| Allocated subframes per Radio Frame |  | subframes | 10 | 10 |
| Modulation |  |  | QPSK | QPSK |
| Target coding rate |  |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |
| Sub-Frames 0 ~ 9 |  | Bits | 88 | 88 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |
| Sub-Frames 0 ~ 9 |  |  | 1 | 1 |
| Maximum number of repetitions |  |  | 8 | 8 |
| NPDCCH starting position |  | symbols | 3 | 3 |
| Cell ID |  |  | Note 2 | Note 2 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |  |

---END OF CHANGE 1---

---START OF CHANGE 2---

#### A.3.1.x.2	 HD-FDD in Enhanced Coverage

Table A.3.1.x.2-1: NPDSCH Reference Channel for UE category NB1 HD-FDD in Enhanced Coverage

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| PDSCH Reference channel |  |  | R.14 HD-FDD | R.15 HD-FDD |
| Carrier bandwidth |  | MHz | 10 | 10 |
| Number of NRS ports |  |  | 1 | 2 |
| Number of CRS ports |  |  | 2 | 2 |
| Allocated resource blocksNote1 |  | PRBs | 1 | 1 |
| Allocated subframes per Radio Frame |  | subframes | 10 | 10 |
| Modulation |  |  | QPSK | QPSK |
| Target coding rate |  |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |
| Sub-Frames 0 ~ 9 |  | Bits | 88 | 88 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |
| Sub-Frames 0 ~ 9 |  |  | 1 | 1 |
| Maximum number of repetitions |  |  | 16 | 16 |
| NPDCCH starting position |  | symbols | 3 | 3 |
| Cell ID |  |  | Note 2 | Note 2 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |  |

---END OF CHANGE 2---