3GPP TSG-RAN WG4 Meeting #AH1801	R4-1800061

San Diego, US, 22nd – 26th Jan 2018

Source: 	Nokia

Title: 					TP for TR 37.863-01-01: DC_(n)71B

Agenda Item: 			4.1.2.2

Document for:	Approval

# 1		Introduction

This contribution corrects the acronymn to be aligned with TS 38.101-3. Similarly MSD and SEM clauses are aligned with technical specification.

****************************** Start of the TP **************************************

## 6.69	DC_(n)71B

### 6.69.1	Operating bands for DC

Table 6.69.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit | BS transmit / UE receive |  |
|  |  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| DC_(n)71B | 71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
|  | n71 |  |  |  |

### 6.69.2	Channel bandwidths per operating band for DC

Table 6.69.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | Maximum aggregated bandwidth [MHz] |
| DC_(n)71B | 71 | 15 | Yes | Yes | Yes | 20 |
|  | n71 | 15 | Yes | Yes | Yes |  |
|  |  | 30 |  | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes |  |

### 6.69.3	Co-existence studies

DC_(n)71B is a contiguous allocation thus only uplink harmonic analysis is needed.

Table 6.69.3-1: DC_(n)71B UL harmonics

| Uplink band |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 71 | 663 | 698 | 1326 | 1396 | 1989 | 2094 |

From Table 6.69.3-1 it can be seen that uplink harmonics 3rd harmonic overlaps with bands 3, 25, 34 and 70.

### 6.69.4	∆TIB and ∆RIB values

No dTib or dRib is required for DC_(n)71B as it is intraband DC.

### 6.69.5	MSD

Table 6.69.5-1: MSD for the DC_(n)71B configuration

| MSD / DC bandwidth class B |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation [LCRB] | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)71B | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | [0] | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | [1.8] |  |
| DC_(n)71B | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | [0] |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | [1.6] |  |
| DC_(n)71B | 71 | 668 | 10 | 10 (RBend = 49) | 622 | [0] |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | [1.7] |  |

### 6.69.6	SEM for DC_(n)71B

FCC has defined spectrum emission maks that is relevant for operation on band 71 frequency range. SEM is presented in Table 6.69.6-1.

6.69.6-1: SEM for band 71 frequency range

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.085 MHz | 0.015 MHz f_offset < 0.1 MHz | -13 | 30 kHz |
| 0.05 MHz f < ENBW – 0.05 MHz | 0.1 MHz f_offset < ENBW MHz | -13 | 100 kHz |
| (ENBW – 0.5) MHz f < (ENBW + 4.5) MHz | ENBW MHz f_offset < (ENBW + 5) MHz | -25 | 1 MHz |
| NOTE 1: ENBW is the aggregated bandwidth in MHz of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. NOTE 2: The measurement filter -3dB point is that closest to the aggregated block. |  |  |  |

### 6.69.7	A-MPR

FFS

****************************** End of the TP **************************************