3GPP TSG-RAN WG4 Meeting #AH1801	R4-1800062

San Diego, US, 22nd – 26th Jan 2018

Source: 	Nokia, Nokia Shanghai Bell

Title: 					n41 A-MPR simulations results

Agenda Item: 			4.3.2.9

Document for:	Discussion

# 1		Introduction

In previous RAN4 meeting we presented initial A-MPR results for band n41. This contribution provides complete set of necessary backoff results for all agreed channel bandwidths on two locations which are lowest position on the band and in a position where additional spurious emission requirement does not affect the results. Also delta triangles against tentative MPR proposal [2] are presented which tell the necessary A-MPR in case [2] is agreed.

# 2	Discussion

## 2.1	Emission requirements

In addition to general spurious emission requirements following requirement apply.

### 2.1.1	Spectrum emission mask

Band 41 has special emission mask for LTE due to FCC requirements, see below Table 1.

Table 1: Band 41 LTE SEM

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-9 | -25 | -13 | -13 | -13 | 1 MHz |
| 9-10 | -25 | -25 | -13 | -13 | 1 MHz |
| 10-13.5 |  | -25 | -13 | -13 | 1 MHz |
| 13.5-15 |  | -25 | -25 | -13 | 1 MHz |
| 15-18 |  |  | -25 | -13 | 1 MHz |
| 18-20 |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  | -25 | 1 MHz |

What is noticeable in band 41 SEM is that the breakpoint from -13 dBm/1 MHz to -25 dBm / 1 MHz is

- 6 MHz for 5 MHz Channel

- 9 MHz for 10 MHz Channel

- 13.5 MHz for 15 MHz Channel

- 18 MHz for 20 MHz Channel

Apart from 5 MHz channel this break point corresponds to LTE transmission bandwidth and comes from the way FCC has defined the mask requirement.

Band 41 emission mask for NR needs to be defined in similar manner by taking into account the actual transmission bandwidths of the signals when setting the breakpoint form -13 dBm/1 MHz to -25 dBm / 1 MHz. Actual NR transmission bandwidths are presented in Table 2. Also RAN4 needs to consider what is the requirement for first 1 MHz outside than channel bandwidth and one possibility is to use same requirement as for NR general emission mask. Hence the proposed n41 SEM is presented in Table 3.

Table 2: NR transmission bandwidths

| SCS [kHz] | Channel bandwidths [MHz] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| 15 | 4.5 | 9.36 | 14.22 | 19.08 | 23.94 | 38.88 | 48.6 | N.A | N.A | N.A |
| 30 | 3.96 | 8.64 | 13.68 | 18.36 | 23.40 | 38.16 | 47.88 | 58.32 | 78.12 | 98.28 |
| 60 | N.A | 7.92 | 12.96 | 17.28 | 22.32 | 36.72 | 46.8 | 56.88 | 77.04 | 97.20 |

Table 3: n41 SEM

| ΔfOOB MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± 0 - 1 | -15 | -18 | -20 | -21 | -22 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1 - 5 | -10 |  |  |  |  |  |  |  |  |  | 1 MHz |
| ± 5 – 6 | -13 | -10 |  |  |  |  |  |  |  |  |  |
| ± 6 - X | -13 |  |  |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) | -25 |  |  |  |  |  |  |  |  |  |  |
| Note 1: X is defined in Table 2 |  |  |  |  |  |  |  |  |  |  |  |

### 2.1.2	Additional spurious emissions

In addition to band 41 specific emission mask UE needs to full fill additional spurious emission requirement as presented in Table 4.

Table 4: Additional spurious emission requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | All channel bandwidths |  |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0 < f < 2490.5 | -25 | 1 MHz |

### 2.1.2	ACLR

Standard NR ACLR requirement applies as in Table 5 and 6.

Table 5: NR ACLR measurement bandwidth

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| NR ACLR measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 38.895 | 48.615 | 58.35 | 78.15 | 98.31 |

Table 6: NR ACLR requirement

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NRACLR |  | 31 dB | 30 dB |

UTRAACLR does not apply for band 41.

## 2.2. Simulation assumptions

The channel was placed at the lower edge of band n41. This maximizes the required A-MPR due to the additional spurious limit.

Simulation assumptions were as follows.

- PA calibration point: QPSK 100 RB DFT-s-OFDM signal with 0 dB MPR

- LO leakage and IQ-Image = 28 dBc

- NR ALCR = 30 dBc

- n41 SEM according to Table 3

- n41 additional spurious emission limit according to Table 4

- MPR assumption as in Table 6

Table 6: MPR assumption

|  |  | Tentative MPR [dB] |  |
| --- | --- | --- | --- |
| WF type | Modulation | Outer allocation | Inner allocation |
| DFT-S-OFDM | pi/2-BPSK | 0 | 0 |
|  | QPSK | 1 | 0 |
|  | 16-QAM | 1.5 | 0.5 |
|  | 64-QAM | 2 |  |
|  | 256-QAM | 4.5 |  |
| CP-OFDM | QPSK | 3 | 0.5 |
|  | 16-QAM | 3 | 1.5 |
|  | 64-QAM | 3 |  |
|  | 256-QAM | 6 |  |

## 2.3	Simulation results

### 2.3.1	Carrier in a lowest position of the band

In Figure 1 we present examples of delta triangles which present the difference of necessary backoff and MPR, in other words A-MPR. Carrier placement is at lowest position thus the additional spurious emission requirements is closest and we can observe a necessity of A-MPR also in part of the carrier which is closest to lower edge of the band (in red).

Figure 1: Examples of delta triangles in lowest position of the band

Full set of results can be found from attached delta_zeroOffset.zip-file.

#### 2.3.1.1	Output power backoff results

In this clause results are presented in a form of output power backoff which can be used to determine the necessary A-MPR once the MPR is agreed, example plots in Figure 2.

Figure 2: Examples of OBO triangles in lowest position of the band

Full set of results can be found from attached backoff_zeroOffset.zip-file.

### 2.3.2	Carrier in a lowest position where additional spur does not affect

First we studied what is necessary A-MPR in a position that additional spur does not affect and found out that no A-MPR is needed on top of MPR assumption in Table 6, see example delta triangles in Figure 3.

Figure 3: Examples of delta triangles away from lower band edge

Full set of results can be found from attached delta_largeOffset.zip-file.

Necessary offset between lower edge of the signal and lower edge of the band is presented in Figure 4 for the smallest specified SCS of each channel bandwidth. When the offset is equal or larger than in Figure 4 then no A-MPR is necessary. However as can be seen for example from Figure 1, even if the carrier is in A-MPR region not all allocations require A-MPR.

Figure 4: Necessary offset for no A-MPR

# 3	Conclusion

In this contribution we have provided full set of A-MPR results for n41. We have also presented the necessary offset from low edge of the band when no A-MPR is necessary due to additional spurious emission requirement.

# 4	References

[1] R4-1712448, Band n41 A-MPR, Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc., RAN4#85

[2], Sub-6 MPR WF, Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc, RAN4 AH1801