3GPP TSG-RAN WG4 AH1801 Meeting                      	 R4-1800115

San Diego, CA, USA, January 21st – 26th, 2018

Agenda Item:	4.6.5.1

Source:	MediaTek Inc.

Title:	Discussion on SBI acquisition requirement for NR

Document for:		Discussion

- Introduction

In the RAN4 #85 meeting the way forward on the SSB time index acquisition time was agreed [1]. The proposed simulation observation are shown as follow.

- For FR1, only DMRS acquisition is needed for SSB time index acquisition. Simulation results show that [TBD] attempt are needed at -6dB. - For FR2, both PBCH-DMRS acquisition and PBCH NR-MIB decoding are needed for SSB time index acquisition. Simulation results show that [TBD] attempts are needed at -6dB. - PBCH Soft-combining is not mandatory and shall be up to implementation.

In RAN1 #90bis meeting, a new working assumption was agreed on the NR-PBCH design are stated.

- For both sub-6 and above-6, reduce the PBCH to X PRB per symbol and add Y PBCH PRBs to SSS symbol with no other redesign - Working assumption: - X=20 - Y=2*(24-X) - Same mapping rule(frequency-first, time-second) for data and DMRS is applied - Working assumption: The same DMRS density in the PBCH PRB in the SSS symbols is assumed - (working assumption) NR-PBCH has a payload size of 56 bits (including CRC)

In RAN4 #85 meeting, the new simulation configuration based on these new working assumption was approved [2]. In this paper, we discuss the reference UE behaviors, update our simulation results based on RAN4 newest agreement and propose the requirements for SBI acquisition.

- NR-PBCH DMRS Detection and Acquisition Time

In this section we provide link level simulation results of mis-detection rate and number of acquisition attempts for DMRS SBI acquisition. The scenario for no Tx/Rx beamforming and no soft combination for multiple PBCHs were evaluated. The results are shown in the table below.

Table 1: DMRS SBI acquisition time

| Propagation condition | Subcarrier Spacing | SNR [dB] | Mis-Detection Rate | Number of acquisition attempts |
| --- | --- | --- | --- | --- |
| EPA5 | 15KHz | -10 | 0.1338 | 2 |
|  |  | -8 | 0.0584 | 2 |
|  |  | -6 | 0.0233 | 2 |
|  |  | -4 | 0.0089 | 1 |
|  |  | -2 | 0.0017 | 1 |
|  | 30KHz | -10 | 0.1435 | 2 |
|  |  | -8 | 0.0526 | 2 |
|  |  | -6 | 0.0146 | 2 |
|  |  | -4 | 0.0027 | 1 |
|  |  | -2 | 0.0005 | 1 |
| ETU70 | 15KHz | -10 | 0.1558 | 2 |
|  |  | -8 | 0.0637 | 2 |
|  |  | -6 | 0.0183 | 2 |
|  |  | -4 | 0.0051 | 1 |
|  |  | -2 | 0.0011 | 1 |

Observation 1: At least 2 attempts is needed for SS block index acquisition via DMRS detection in FR1, targeting 1% mis-detection probability.

- NR-PBCH Decoding BLER and Acquisition Time

In this section we provide link level simulation results of BLER and number of acquisition attempts for PBCH Decoding. The scenario for no Tx/Rx beamforming and no soft combination for multiple PBCHs were evaluated. The results are shown in the table below.

Table 2: NR-PBCH decoding and SBI acquisition times

| Propagation condition | Subcarrier Spacing | SNR [dB] | BLER | Number of acquisition attempts |
| --- | --- | --- | --- | --- |
| EPA5 | 15KHz | -10 | 0.323 | 6 |
|  |  | -8 | 0.159 | 4 |
|  |  | -6 | 0.057 | 2 |
|  |  | -4 | 0.0195 | 2 |
|  |  | -2 | 0.0047 | 1 |
|  | 30KHz | -10 | 0.292 | 5 |
|  |  | -8 | 0.106 | 3 |
|  |  | -6 | 0.033 | 2 |
|  |  | -4 | 0.0084 | 1 |
|  |  | -2 | 0.0018 | 1 |
| ETU70 | 15KHz | -10 | 0.48 | 5 |
|  |  | -8 | 0.165 | 3 |
|  |  | -6 | 0.0328 | 3 |
|  |  | -4 | 0.0042 | 1 |
|  |  | -2 | 0.0002 | 1 |
| TDL-C 100ns 3km/h | 120KHz | -10 | 0.48 | 9 |
|  |  | -8 | 0.158 | 5 |
|  |  | -6 | 0.032 | 2 |
|  |  | -4 | 0.0026 | 1 |
|  |  | -2 | 0.0002 | 1 |
|  | 240KHz | -10 | 0.58 | 17 |
|  |  | -8 | 0.26 | 9 |
|  |  | -6 | 0.063 | 3 |
|  |  | -4 | 0.0072 | 1 |
|  |  | -2 | 0 | 1 |

Observation 2: At least 3 attempts is required for SS block index acquisition via BPCH decoding in FR2 without considering additional delay due to Rx Beam sweeping, targeting 1% BLER.

- Intra-frequency SSB time index requirement without measurement gaps

In RAN1’s agreement, SMTC periodicities should be {5ms, 10ms, 20ms, 40ms, 80ms, 160ms}. In RRM, UE is not expected to perform measurement frequently when SMTC period is small. The wireless channels between two samples are strongly correlated in a short period of time. Detecting SBI for every SMTC occasion does not always guarantee performance gain in return, but consumes additional UE power. To trade-off the measurement timing and power consumption, it’s better also to add a lower bound on SMTC periodicity in the requirement similar as PSS/SSS detection and measurement requirement. In our opinion, the same as cell detection and measurement, 40ms is a reasonable value to be chosen as a lower bound on SMTC periodicity in the delay requirement.

Proposal 1: In the intra-frequency SSB time index acquisition requirement without measurement gaps, a lower bound, e.g., 40 ms, should be applied to SMTC periodicity.

After that, considering the RF loss and fixed point implementation, it is proposed to add one or two more SMTC periodicity to give enough margin to UE.

Proposal 2: In the intra-frequency SSB time index acquisition requirement without measurement gaps, 2 more SMTC periodicities should be applied to the requirement.

For DRX operation in RRC_Connected state, we can also re-use LTE design rules for PSS/SSS detection to define the performance requirement. It is desirable to follow same principle to have a low rate of activity for long DRX periods and keep non-DRX requirements when DRX period roughly equals RRM-related activity.

Proposal 3: In the intra-frequency SSB time index acquisition requirement, it should consider the power saving under large DRX scenarios.

In the following, we provide the potential performance requirements. When the UE can perform intra-frequency SS block identification without measurement gaps, the UE shall be able to identify a new SS block within TSSB_time_index for FR1and FR2 (without considering additional delay RX beam sweeping) as shown in Table 1, and 2, respectively.

Proposal 4: In the intra-frequency SSB time index acquisition without measurement gaps, the requirement table is shown as follow.

Table 1. SS block identification requirement without measurement gaps for frequency range FR1

| DRX cycle(s) | TSSB_time_index |
| --- | --- |
| No DRX, or DRX cycle ≤0.04 | max{ 4 x SMTC period Note 1, 160 ms} |
| 0.04 < DRX cycle≤ 2.56 | 4 x DRX period |
| Note 1 : If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 2. SS block identification requirement without measurement gaps for frequency range FR2

| DRX cycle(s) | TSSB_time_index |
| --- | --- |
| No DRX, or DRX cycle ≤0.04 | max{ 5 x SMTC period Note 1, 200 ms} Note 2 |
| 0.04 < DRX cycle≤ 2.56 | 5 x DRX period Note 2 |
| Note 1 : If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified Note 2: Not yet consider Rx beam sweeping in FR2 |  |

For multi-CCs cases, it could also happen that the SMTC occasions of all CCs collide with each other. In these multi-CCs’ cases, we propose also to scale the delay requirements by the parameter of configured CCs (NNR_CC,FR1 and NNR_CC,FR2). Details explanation could refer on another Tdoc [3].

Proposal 5: TSSB_time_index without measurement gap requirement is defined as follow.

In FR1, TSSB_time_index,per CC = max(160ms, 4 x SMTC period);

In FR2, TSSB_time_index,per CC = max(200ms, 5 Note x SMTC period);

when multiple CCs are configured: TSSB_time_index = TSSB_time_index,per CC x(NNRcc,FR12+NNRcc,FR2), for No DRX, or DRX cycle <0.04s, where NNR_CC,FR1 is the number of configured NR FR1 CCs to the UE, NNR_CC,FR2 is the number of configured NR FR2 CCs to the UE. (Note: Not yet consider Rx beam sweeping in FR2.)

- Summary

In this paper, we provided simulation results for SBI acquisition time in NR, for a number of parameter combinations according to the simulation assumptions agreed in [2]. At the same time, we also propose the intra-frequency SBI acquisition requirement without measurement gap.

Observation 1: PBCH-DMRS time index reading should be at least 3 attempts for SS block index acquisition targeting 1% mis-detection probability in frequency range FR1.

Observation 2: PBCH reading should be at least 3 attempts for SS block index acquisition targeting 1% BLER in frequency range FR2 without Rx Beamforming.

Proposal 1: In the intra-frequency SS/PBCH block time index acquisition requirement without measurement gaps, a lower bound, e.g., 40 ms, should be applied to SMTC periodicity.

Proposal 2: In the intra-frequency SS/PBCH block time index acquisition requirement without measurement gaps, 2 more SMTC periodicities should be applied to the requirement.

Proposal 3: In the intra-frequency SS/PBCH block time index acquisition requirement, it should consider the power saving under large DRX scenarios.

Proposal 4: In the intra-frequency SS/PBCH block time index acquisition without measurement gaps, the requirement table is shown as follow.

Table 1. SS block identification requirement without measurement gaps for frequency range FR1

| DRX cycle(s) | TSSB_time_index |
| --- | --- |
| No DRX, or DRX cycle ≤0.04 | max{ 4 x SMTC period Note 1, 160 ms} |
| 0.04 < DRX cycle≤ 2.56 | 4 x DRX period |
| Note 1 : If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 2. SS block identification requirement without measurement gaps for frequency range FR2

| DRX cycle(s) | TSSB_time_index |
| --- | --- |
| No DRX, or DRX cycle ≤0.04 | max{ 5 x SMTC period Note 1, 200 ms} Note 2 |
| 0.04 < DRX cycle≤ 2.56 | 5 x DRX period Note 2 |
| Note 1 : If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified Note 2: Not yet consider Rx beam sweeping in FR2 |  |

Proposal 5: TSSB_time_index without measurement gap requirement is defined as follow.

In FR1, TSSB_time_index,per CC = max(160ms, 4 x SMTC period);

In FR2, TSSB_time_index,per CC = max(200ms, 5 Note x SMTC period);

when multiple CCs are configured: TSSB_time_index = TSSB_time_index,per CC x(NNRcc,FR12+NNRcc,FR2), for No DRX, or DRX cycle <0.04s, where NNR_CC,FR1 is the number of configured NR FR1 CCs to the UE, NNR_CC,FR2 is the number of configured NR FR2 CCs to the UE. (Note: Not yet consider Rx beam sweeping in FR2.)

- References

- R4-1714251, “Way forward on the SSB time index acquisition time”, Huawei, HiSilicon, Ericsson, Intel, MediaTek, Samsung, Qualcomm

- R4-1713437, “Updated PBCH Simulation assumptions after redesign”, Huawei, HiSilicon

- R4-1800114, “Discussion on cell detection requirement for NR”, Mediatek

- Annex A: Simulation Assumptions

Table 1: Parameters for basic SI reading (PBCH acquisition)

| Parameter | Unit | Value |
| --- | --- | --- |
| Carrier frequency | GHz | 4GHz; 30GHz |
| Subcarrier spacing | KHz | 15 kHz, 30KHz; 120KHz, 240KHz |
| Number of Tx antennas | - | 1 Assuming only 1Tx port is used |
| Number of Rx antennas | - | 2 (Without receiver beamforming) |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 |
| SS burst set periodicity | ms | 20 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| RB Utilization | % | 0 |
| PBCH symbols within the SS block |  | PSS-PBCH-SSS and PBCH-PBCH |
| Data and Control Power offset with respect to PSS and SSS | dB | 0 |
| PBCH power offset with respect to PBCH-DMRS | dB | 0 |
| PBCH-DMRS power offset with respect to PSS and SSS | dB | To be indicated |
| PSS and SSS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS RE positions within the PBCH resource | - | According to the RAN1 agreements |
| PBCH Modulation | - | QPSK |
| PBCH Payload (including the CRC) | bits | 56bit(CRC=24bit) |
| PBCH SNR | dB | -10 : 0 dB, at least including -6dB,-8 dB |
| Propagation Condition | - | EPA5, ETU70, TDL-C with desired RMS delay spread 100ns Note1 |
| UE speed and Doppler Shift | - | 3km/h speed |
| Detection Method |  | one shot detection |
| NOTE 1: The channel models of CDL and TDL are used for simplified link level evaluations, which are defined in TR 38.900. |  |  |