3GPP TSG-RAN WG4 Meeting AH-201801                                                     R4-1800215

San Diego, US, 22- 26 Jan, 2018

Agenda Item:   4.2.2.3

Source:        Samsung

Title:          Discussion on NR sync raster for initial cell search

Document for:  Discussion

Introduction

As agreed in [1] the synch raster agreement are summarized as below:

- GSCN: The synchronization raster indicates the frequency positions of the synchronization block that can be used by the UE for system acquisition when explicit signalling of the synchronization block position is not present. - SS location: A global synchronization raster is defined for all frequencies. The frequency position of the SS block is defined as SSREF with corresponding number GSCN. The GSCN are numbered in increasing frequency order. - Granularity of global SS raster: Frequency range; SS block frequency position SSREF; Range of GSCN. 0 – 2650 MHz; N*900kHz+ M*5kHz, N=1:[2944], M=-1:1; 1 – [8832]. 2400 – 24250 MHz; 2400MHz+N*1.44MHz,N= 0:[15173]; [8833-24006]. 24250 – 100000 MHz; [24250.08]MHz+ N*[17.28]MHz, N= 0:[4383]; [24007 – 28390]. - Band specific SS raster: - For FR1 <2.6GHz: Raster entry=N*900kHz+ M*5kHz, N=1:[2944], M=-1:1 - Raster applies to bands with 100kHz channel raster and 15kHz channel raster with 5MHz minimum bandwidth - For 10MHz minimum channel bandwidth a down selection factor of [6] - For FR1>2.4GHz:2400MHz+N*1.44MHz,N= 0:[15173] - Raster applies to bands with 15kHz channel raster and 10MHz minimum bandwidth - For 10MHz minimum channel bandwidth and 15kHz SS block SCS a down selection factor of [3] is applied - For 40MHz minimum channel bandwidth and 30kHz SS block SCS a down selection factor of [21] - For FR2: Raster entry=[24250.08] MHz+ N*[17.28]MHz - Raster applies to bands with 60kHz channel raster in this frequency range - For 100MHz minimum channel bandwidth and 240kHz SS block SCS a down selection factor of [2]

Based on above agreement and assumption, start position and end of position of candidate SS channels (initial cell search only) for each band can be calculated corresponding SS raster and global GSCN. In this contribution, further discussion and calculation were carried for the GSCN of each operating band.

Discussion

SS frequency location and corresponding number GSCN for initial cell search without system assist information were defined and numbering as globally covering all frequencies based on SS raster in different frequencies ranges. As in show in figure 1,

- For 0~2650MHz with 100kHz channel raster, SS raster as 900kHz with 3 times shift; SS_REF = ceil(GSCN/3)*900kHz + (mod(GSCN-1,3)-1)*5kHz

- For 2.4GHz ~24.250GHz, SS raster as 1.44MHz for 15kHz channel raster, SS_REF=(GSCN-8833)*1.44MHz+2400MHz

- For 24.250GHz above, SS raster as 17.28MHz for 60 kHz channel raster, SS_REF=(GSCN-24007)*17.28MHz+24250MHz

Then for each operating band, possible SS block positions and corresponding GSCN can be calculated based on frequency ranges, minimum channel bandwidths and default SCS for SS block. For specific band, when determining corresponding GSCN, following mapping rules can be used:

- Start of SS_REF>= F_start+ F-offset

- End of SS_REF <=F_end-F-offset

F_start and F_end present the start and of end of frequency ranges for certain band, SS_BW corresponding to bandwidth of SS/PBCH block:

- F-offset = SS_BW/2+Guardband of minimum CHBW =20RB*12*SCS/2+ Guard band of minimum CHBW

According SCS of SS block, SS_BW can be calculated as below:

- 15kHz SCS: with minimum BW 5MHz: F-offset =4.085MHz/2; with minimum BW 10MHz: F-offset = 4.225MHz/2

- 30kHz SCS: with minimum BW 10MHz: F-offset = 8.490MHz/2; with minimum BW 40MHz:F-offset = 9.010MHz/2

- 120kHz SCS: with minimum BW 50MHz:F-offset =32.60 MHz/2

- 240kHz SCS: with minimum BW 100MHz:F-offset=65.20MHz/2 (assume same SU as the case of 50MHz,120kHz)

Observation1: When determining proper range of GSCN for certain band,  not only all possible SS raster locations should be within the frequency ranges of such band, meanwhile corresponding whole SS/PBCH block need to be inside of frequency ranges with enough guard band.

Taking band n1 as example, as show in figure 2 below, SS raster with GSCN=7033 is inside of frequency ranges of such band, however the whole SS/PBCH transmission block exceeds of low bound of this band. The first SS block which whole transmission bandwidth inside of band n1 is SS raster with GSCN = 7039. On the upper bound of band N1, SS raster with GSCN=7224 is full inside of band n1. Based on above analysis, the valid GSCN for band n1 is 7039~7224.

Then when UE perform initial cell search in band n1, UE will follow the predefined GSCN range in the specs for band n1 to blind detect SS transmission among all the valid candidate SS block locations in frequency domain.

Followed above principles, we do further mathematical analysis for GSCN mapping tables of all the bands defined in Table 5.4.3.3-1 (FR1) and 5.4.3.3-2 (FR2) in TS 38.104(corresponding table 5.4.3.3-1 in TS38.101-1 and TS38.101-2).

As summarized in below table 1 and table 2 for FR1 and FR2, we observed some differences between defined values of GSCN and our analysis:

Observation 2: For FR1, GSCN ranges of bands need to be corrected to ensure the whole transmission bandwidth of SS blocks inside of frequency ranges of corresponding band as summarized in table 1.

Observation 3: For band n66, in LTE upper bound 2180~2200MHz can only be used as SDL, which means cannot be deployed as SA. Further clarification needed whether in NR such frequency range can be used for SA or not.

Observation 4: band n75, n76 are SDL band, which cannot be deployed by SA.

Observation 5: For band 41, with 10MHz minimum channel bandwidth and 15kHz SS SCS, SS raster is 3 times of basic SS raster (step size =3)

Observation 6: For band 41, with 40MHz minimum channel bandwidth and 30kHz SS SCS, SS raster is 21 times of basic SS raster (step size =21)

Observation 7: For FR2, GSCN ranges are not insistent with frequency ranges of bands.

| NR Operating Band | SS Block SCS | Range of GSCN in 38.104 (First – <Step size> – Last) | Range of GSCN (proposed) (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 15kHz | 7033 – <1> – 7224 | 7039-<1>-7224 |
| n2 | 15kHz | 6433 – <1> – 6624 | 6439 – <1> – 6624 |
| n3 | 15kHz | 6016 – <1> – 6258 | 6022 – <1> – 6258 |
| n5 | 15kHz | 2896 – <1> – 2973 | 2902 – <1> – 2973 |
|  | 30kHz | 2911 – <1> – 2961 | 2911 – <1> – 2964 |
| n7 | 15kHz | 8734 – <1> – 8958 | 8988 – <1> – 9032 |
| n8 | 15kHz | 3082 – <1> – 3192 | 3091 – <1> – 3192 |
| n20 | 15kHz | 2635 – <1> – 2730 | 2644 – <1> – 2727 |
| n28 | 15kHz | 2527 – <1> – 2670 | 2533 – <1> – 2667 |
| n38 | 15kHz | 8566 – <1> – 8724 | 8953– <1> – 8984 |
| n41 | 15kHz | 8899 – <1> – 9030 | 8902 – <3> – 9031 |
| n50 | 15kHz | 4774 – <1> – 5049 | 4780 – <1> – 5049 |
| n51 | 15kHz | 4756 – <1> – 4764 | 4762 – <1> – 4764 |
| n66 | 15kHz | 7033 – <1> – 7326 | 7039 – <1> – 7257 (7326) Note1 |
|  | 30kHz | 7048 – <1> – 7317 | 7048 – <1> – 7251(7317) Note 1 |
| n70 | 15kHz | 6649 – <1> – 6726 | 6655 – <1> – 6726 Note 3 |
| n71 | 15kHz | 2056 – <1> – 2166 | 2062 – <1> – 2166 |
| n74 | 15kHz | 4915 – <1> – 5052 | 4924 – <1> – 5052 |
| n75 | 15kHz | 4774 – <1> – 5049 | 4780 – <1> – 5049 N/A Note2 |
| n76 | 15kHz | 4756 – <1> – 4764 | 4762 – <1> – 4764 N/A Note2 |
| n77 | 30kHz | 9460 – <1> –10079 | 9461 – <1> –10080 |
| n78 | 30kHz | 9460 – <1> – 9801 | 9461 – <1> –9802 |
| n79 | 30kHz | 10245 - <1> - 10613 | 10226 - <21> - 10625 |
| Note 1: for band 66 in LTE, frequency range of 2180~2200MHz only used for SDL, which cannot be deployed by SA. Note 2: band n75, n76 are SDL band, which cannot be deployed by SA. Note 3: Further clarification may be needed for band 70, for DL operation on 2010MHz~2020MHz |  |  |  |

| NR Operating Band | SS Block SCS | Range of GSCN in 38.104 (First – <Step size> – Last) | Range of GSCN (proposed) (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n258 | 120 kHz | 24057 – <1> – 24243 | 24008 – <1> – 24194 |
| n257 | 120 kHz | 24188 – <1> – 24358 | 24139 – <1> – 24309 |
|  | 240 kHz | 24190 – <2> – 24356 | 24140 – <2> – 24308 |
| n260 | 120 kHz | 24795 – <1> – 24966 | 24746 – <1> – 24917 |
|  | 240 kHz | 24797 – <2> – 24964 | 24747 – <2> – 24915 |

Conclusion

In this contribution we further analyze the sync raster as have below observations:

Observation 1: When determining proper range of GSCN for certain band,  not only all possible SS raster locations should be within the frequency ranges of such band, meanwhile corresponding whole SS/PBCH block need to be inside of frequency ranges with enough guard band.

Observation 2: For FR1, GSCN ranges of bands need to be corrected to ensure the whole transmission bandwidth of SS blocks inside of frequency ranges of corresponding band as summarized in table 1.

Observation 3: For band n66, in LTE upper bound 2180~2200MHz can only be used as SDL, which means cannot be deployed as SA. Further clarification needed whether in NR such frequency range can be used for SA or not.

Observation 4: band n75, n76 are SDL band, which cannot be deployed by SA.

Observation 5: For band 41, with 10MHz minimum channel bandwidth and 15kHz SS SCS, SS raster is 3 times of basic SS raster (step size =3)

Observation 6: For band 41, with 40MHz minimum channel bandwidth and 30kHz SS SCS, SS raster is 21 times of basic SS raster (step size =21)

Observation 7: For FR2, GSCN ranges of bands need to be corrected to ensure the whole transmission bandwidth of SS blocks inside of frequency ranges of corresponding band as summarized in table 2.

Reference

[1] R4-1714514, WF on SS Raster for NR

[2] TS 38.104 15.0.0

[3] TS 38.101-1 15.0.0

[4] TS38.101-2 15.0.0