3GPP TSG-RAN WG4 Meeting AH-1801                 	R4-1800375

San Diego, USA, 22 – 26 January 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial dynamic simulation results for NR Mobility

Agenda item:	4.6.2.1

Document for:	Discussion

# 1	Introduction

RAN4 has been discussing the number of cells and SSBs the UE shall be able to monitor during many meetings without consensus. Companies have provided system level simulation result as input to the discussion to facilitate the discussion. In this paper, we continue the discussion and present initial and draft dynamic system level simulation results.

# 2	Discussion

In earlier papers, we have been providing static simulation results in order to analyse the number of cell and SSBs the UE would be able to detect in static conditions. The results are captured in [1] and indicates that the UE could potentially detect a large number of cells and SSBs when considering higher carrier frequencies.

Such static simulation are valuable for determining number of cells and beams the UE may be able to detect when placed at certain spot under certain condition. However, they do not account for any mobility aspects. To evaluate the impact of mobility, dynamic simulations are needed. From dynamic simulations, more information related to the dynamics of the system will become known. E.g. how does the measurement sampling period, the L1 measurement period and the overall measurement latency impact.

## 2.1	Simulation setup

In the simulations, we have used the baseline simulation assumptions as agreed in [2] changing the necessary parameters to apply cell and SSB settings according to [3]. The network layout is illustrated in figure 1.

Figure 1 Illustration of the network layout used in the simulation.

We have used an Urban Macro layout with 7x4-sector sites, 28 sectors total and 200ms ISD. Sector aperture in the simulations is 90 degrees. In this paper, we have included dynamic simulations results using 48 SSBs per cell using 3 elevations. The hexagonal grid is only for illustrative purposes and have no real meaning.

In the simulations, the UEs are moving with 30 km/h and there is 28 UEs active at a time with full buffer traffic (one UE per sector). An SSB burst periodicities of 20 or 40 ms have been used which are also used as the UE sampling rate (L1 sampling interval). I.e. the UE measurement rate is 20 or 40ms. More detailed assumptions can be found in Appendix A.

In simulations, L1 measurement filters of 3 or 4 samples have been applied, which together with the SSB transmission offset reduces the interference seen by the UE when performing SSB based measurements. 3-sample filter is used in the case of measurement periodicity of 40 ms and 4-sample filter with 20 ms.

At the moment, the modeling of measurement report and handover command transmissions are ideal and there is no radio link monitoring active.

We have applied an offset between the SSB transmissions to reduce SSB transmissions collisions from different cells. By applying a random offset between the SSB transmissions, the beam collisions during measurement are randomized.

## 2.2	Simulation results

The goal of the simulation was to investigate the moving UEs capability of tracking and camping on the best cell and beam, based on the SSB-based RSRP measurement (SS-RSRP).

From analyzing the SS-RSRP difference between the overall best beam and the serving beam it is possible to evaluate the UEs ability to track beams at certain UE velocity. E.g. if the L1 measurements latencies would be lead to challenges in mobility.

In the case there will be large difference between the overall strongest beam and the serving beam this could be an indicator that UE has mobility problems due to measurement latencies. Next figure shows the initial results of the dynamic simulations.

Figure 2 Difference in SS-RSRP between overall best beam and serving cell best beam (30kmh).

The results are based on a 2Rx panel UE. We can see from the results that in 30kmh case the UE will in 80% of the time be camped on the beam with the best SS-RSRP – i.e. UE is served by best cell and beam.

We also investigated if there would be any impact from the UE measurement interval. I.e. would it have an impact whether UE L1 measurement interval would be 20ms or 40ms. These initial results show that at least in 30kmh scenario it makes no significant impact.

However, it should be noticed that these simulations are very initial and do not include impacts from:

- Signaling latencies

  - Measurement reporting

  - Handover command

Next steps will be to analyze the impact the impacts from signaling latencies, once the signaling details are defined in RAN2. Additionally, the impact from adding different levels of traffic to the system will need to be analyzed.

# 3	Conclusion

In this paper, we continue the discussion and present initial and draft dynamic system level simulation results. The simulations are very initial and no conclusions are drawn. However, in the next is analyze more the various system impact and how this potentially influences the mobility.

# References

- R4-1800374, SLS on Number of cells and SS-Blocks, Nokia, Nokia Shanghai Bell

- R4-1709903, System level simulation assumptions in NR RRM, Ericsson

- R4-1713151, Network Cell and Beam Dimensioning discussion, Nokia, Nokia Shanghai Bell

# A	Simulation Parameters