3GPP TSG-RAN WG4 Meeting AH-1801                                                         R4-1800437

# San Diego , US, 22 - 26 Jan, 2018

Agenda item:	4.2.1

Source: 	CMCC

Title: 	Channel bandwidths for NR band 34

Document for:	Approval

Introduction

In last RAN plenary, New NR band request for adding band 34 was proposed in RAN #78[1]. In this paper, we propose to add channel bandwidths for NR band 34.

| Frequency range /LTE band | REL-indep. From | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- | --- |
| Band 34 | REL-15 | SHAO Zhe CMCC | shaozhe@chinamobile.com | CATT, Huawei, ZTE | new |

Discussion

We propose to add Channel bandwidths for NR band 34 in case of SCS = 15 kHz, 30 kHz and 60 kHz. The channel bandwidths are defined as both BS channel bandwidths and UE channel bandwidths. All channel bandwidths for band 34 are shown below.

Table 1 channel bandwidths for NR band 34

| NR band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 102,3 MHz | 153 MHz | 203 MHz | 253 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n34 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |

Proposal 1: Table 1 channel bandwidths should be supported as BS and UE channel bandwidths for NR band34.

Conclusions

In this paper, we have the following proposal:

Proposal 1: Table 1 channel bandwidths should be supported as BS and UE channel bandwidths for NR band34.

Reference

- RP-172834, “Revision of WI: New Radio Access Technology”, NTT DOCOMO