3GPP TSG-RAN WG4 AH-1801	R4-1800724

San Diego, USA, 22 – 26 January 2018

Source:	Ericsson, Telstra

Title:	TP to TR 37.863-01-01: protected bands for DC_28A_n258A

Agenda item:	4.1.2.2

Document for:	Approval

# 1. Introduction

This is a text proposal for 37.863-01-01 for correcting typos and including protected bands for DC_28A_n258A.

# 2. Text Proposal for TR 37.863-01-01

##### ---Start of changes---

## 6.48	DC_28A_n258A

### 6.48.1	Operating bands for DC

Table 6.48.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A_n258A | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n258 | 24.25 GHz | – | 27.5 GHz | 24.25 GHz | – | 27.5 GHz | TDD |

### 6.48.2	Channel bandwidths per operating band for DC

Table 6.48.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A_n258A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  | n258 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | [Yes] |  |  | Yes | Yes | Yes |  |

### 6.48.3	Co-existence studies

Table 6.48.3-1 lists Band 28 +Band n258 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.48.3-1: Band 28 and Band n258 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 |  | 24250 | 27500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 |  | 1496 | 48500 | 55000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 |  | 2244 | 72750 | 82500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 23502 | 26797 |  | 24953 | 28248 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 26094 |  | 22754 | 47752 | 54297 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25656 |  | 28996 | 49203 | 55748 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 303 |  | 1148 | 24230 | 27520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 25391 | 22006 |  | 72002 | 81797 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 53594 |  | 47004 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 26359 |  | 29744 | 73453 | 83248 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 49906 |  | 56496 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 109297 |  | 96252 | 21258 | 24688 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 81094 |  | 71254 | 46256 | 52891 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 97703 |  | 110748 | 27062 | 30492 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 74156 |  | 83996 | 50609 | 57244 |

Based on Table 6.48.3-1;

- 3rd order IMD may fall into own Rx of Band n258 and Rx frequencies of bands 5, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68 and n257.

- 4th order IMD may fall into own Rx of Band n258 and Rx frequencies of band n257.

- 5th order IMD may fall into own Rx of Band n258 and Rx frequencies of band n257.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.48.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.48.3-2: 2UL Band 28 +Band n258 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.48.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.48.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28A_n258A | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception of -36 dBm/MHz for 718-748 MHz |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception of -38 dBm/MHz for 718-748 MHz |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |

### 6.48.4	∆TIB and ∆RIB values

For DC_28A_n258A, the TIB,c and RIB values are given in the tables below.

Table 6.48.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A_n258A | 28 | 0 |
|  | n258 | 0 |

Table 6.48.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28A_n258A | 28 | 0 |
|  | n258 | 0 |

##### ---End of changes---