3GPP TSG-RAN WG4 Meeting AH-1801	R4-1800894

## San Diego, CA, USA, 22 – 26 Jan, 2018

Source: 	Huawei

Title:	TP to TR 38.817-02: relations between single core and separate conducted/OTA test requirements (5.2)

Agenda Item:	 4.4.1.1

Document for:	Approval

# 1. Introduction

During previous RAN4#85 meeting, TP to TR 38.817-02 was proposed in R4-1712650 (TP to TR 38.817-02: relations between single core and conducted/OTA test requirements). This TP was proposed to address the fact that the relationship among the core and conformance requirements for NR BS will be not be the same as for the legacy BS specifications. Namely, for selected NR BS FR1 requirements, a single core requirements will be translated into two different test requirements, i.e. one for conducted conformance, and the second one for radiated conformance purposes. This is due to the fact, that Measurement Uncertainties for the conducted and radiated test setups are expected to differ.

Furthermore, during previous meeting comments were received claiming that the proposed description of relations among single core requirement and the conducted/OTA requirements is not applicable to all requirements. Therefore in this CR appropriate corrections were incorporated to adddress the above comment.

The followign changes were introduced:

-	Section 2: removal of TS 38.141 reference, as this specification was split into two parts and individual references to -1 and -2 parts are already listed in TR in [17] and [18].

-	Section 5.2: text update to address relation for the case of single core requirement and two different test requirements.

This TP is a mirror of the Draft CR to TS 38.104 in R4-1800895.

# 2. TP to TR 38.817-02

------------------------------ Modified section ------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Recommendation ITU-R M.1036-5 (10/2015), “Frequency arrangements for implementation of the terrestrial component of International Mobile Telecommunications (IMT) in the bands identified for IMT in the Radio Regulations (RR)”.

[3]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

[4]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[5]	ITU-R recommendation SM.328: "Spectra and bandwidth of emissions".

[6]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.

[7]	3GPP TS 37.105: “Active Antenna System (AAS) Base Station (BS) transmission and reception”.

[8]	3GPP TR 37.842: “	Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”.

[9]	3GPP TR 37.843: “Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) radiated requirements”.

[10]	R4-1700305, "LS on Characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz".

[11]	Code of Federal Regulations, Title 47, Part 30.203, Upper Microwave Flexible Use Service; Emission limits, Federal Communications Commission.

[12]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[13]	void

[14]	ETSI EN 301 489: “Electromagnetic compatibility and Radio spectrum Matters (ERM); ElectroMagnetic Compatibility (EMC) standard for radio equipment and services”

[15]	3GPP TS 38.113: “NR; Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)”.

[16]	3GPP TS 37.114: “Active Antenna System (AAS) Base Station (BS) Electromagnetic Compatibility (EMC)”.

[17]	3GPP TS 38.141-1: "NR; Base Station (BS) conformance testing; Part 1: Conducted conformance testing"

[18]	3GPP TS 38.141-2: "NR; Base Station (BS) conformance testing; Part 2: Radiated conformance testing"

[19]	3GPP TS 37.104: “E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception”

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

------------------------------ Next modified section ------------------------------

## 5.2	Relationship between minimum requirements and test requirements

While 3GPP TS 38.104 [3] describe the core requirements, the conformance to the requirements is demonstrated by fulfilling the test requirements specified in the conformance specification 3GPP TS 38.141-1 [17] for conducted testing and 3GPP TS 38.141-2 [18] for radiated testing. An allowance for measurement uncertainty through application of test tolerances is used to create the conformance test requirements using the shared risk principle.

This is explained through subclause 4.2 in 3GPP TS 38.104 [3], in a way aligned with the specifications for E-UTRA in 3GPP TS 37.104 [6] and for AAS BS in 3GPP TS 37.105 [7]. The shared risk principle is defined in recommendation ITU-R M.1545 [12].

Selected radiated FR1 requirements are based on the same limits as defined for conducted requirements. Measurement uncertainty values for the conducted test setup and OTA test setup may differ for those requirements due to different test setups. Therefore in case of selected FR1 requirements, a single limit used for both conducted and radiated requirements may result in two different test requirements for conducted and OTA testing, as captured in 3GPP TS 38.141-1 [5] and 3GPP TS 38.141-1 [6].

------------------------------ End of modified sections ------------------------------