3GPP TSG-RAN WG4 Meeting AH 1801	  R4-180956

San Diego, California, US, 22 Jan – 26 Jan 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	BS EVM window length for NR

Agenda item:	4.4.2.4.1.1

Document for:	Approval

# 1.	Introduction

Consideration on BS EVM window reduction [3] was discussed during the last RAN#85 meeting held in Reno, US. Similar discussions were done also for UE EVM window length in [4]. There is no decisions neither for BS EVM window length nor UE EVM window length.

In this contribution, we would like to discuss reduction of BS EVM window length for NR with different BW and SCS combinations with high spectrum utilization. We also provide proposal for BS EVM window length.

# 2.	Discussion

The EVM shall be measured at the point after the FFT and a zero-forcing (ZF) equalizer in the receiver, according to TS 36.104 [1], Annex E. The same reference point will be also selected for NR. It should be noted that before measuring the EVM, time and frequency synchronization must be carried out. The test equipment then computes the EVM for two extreme values of sample timing difference between the FFT processing window and the nominal timing of the ideal signal; these two extreme values correspond to the beginning and end of the EVM window, the length of which is expressed as a percentage of the Cyclic Prefix (CP) length.

Figure 1 illustrates the time window positions for FFT and the inter-symbol interference, coming from the delay spread of adjacent symbols. Typically, the filters have a symmetric impulse response; hence the optimum FFT window with lowest impact of inter-symbol interference is located at the centre position between the symbol boundaries.

Specification TS 36.104 defines an offset of WEVM/2 from that optimum position in both directions. The values of WEVM are copied from 3GPP TS36.104 and provided in Table 1 for E-UTRA and in Table 2 for NB-IoT.

OFDM symbol

n-1

OFDM symbol

n

FFT window 1

OFDM symbol

n+1

CPn+1

FFT window 2

Optimum FFT window

CPn

WEVM

Inter-symbol

interference

Figure 1: FFT time window position in OFDM symbol n, which is suffering from inter-symbol interference from adjacent symbols n+1 and n-1. For symmetric delay spread, the centered green FFT window constitutes the optimum. Both 3GPP defined FFT windows (cyan) experience higher inter-symbol interference. Cyclic prefix (CPn and CPn+1) boundaries are marked with dashed lines.

Table 1: EVM window length for LTE with normal CP as copied from 3GPP TS36.104 [1]

| Channel Bandwidth MHz | FFT size |  | Cyclic prefix length for symbols 0 in FFT samples | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6* [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 1.4 | 128 |  | 10 | 9 | 5 | 55.6 |
| 3 | 256 |  | 20 | 18 | 12 | 66.7 |
| 5 | 512 |  | 40 | 36 | 32 | 88.9 |
| 10 | 1024 |  | 80 | 72 | 66 | 91.7 |
| 15 | 1536 |  | 120 | 108 | 102 | 94.4 |
| 20 | 2048 |  | 160 | 144 | 136 | 94.4 |
| * Note: These percentages are informative and apply to symbols 1 through 6. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |  |  |

Table 2: EVM window length for NB-IoT with normal CP as copied from 3GPP TS36.104 [1]

| FFT size | Cyclic prefix length for symbols 0 in FFT samples | Cyclic prefix length for symbols 1-6 in FFT samples | EVM window length W | Ratio of W to total CP for symbols 1-6(Note 1) [%] |
| --- | --- | --- | --- | --- |
| 128 | 10 | 9 | 3 | 33.3 |
| Note 1: These percentages are informative and apply to symbols 1 through 6. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |

Please note that the EVM window length of NB-IoT (see Table 2) is 60% of the LTE 1.4 MHz EVM window length, though NB-IoT has the same sample rate, FFT size and CP length as LTE 1.4 MHz. However, NB-IoT has a narrower guard band than LTE 1.4 MHz.

For LTE 10 MHz, WEVM is 66 out of 72 cyclic prefix samples. This leaves just 3 samples on either side to protect from inter-symbol interference. Consequently, some inter-symbol interference will leak into the FFT window and generate EVM.

While this was not that much of an issue at the time the EVM definition was established for LTE, it creates severe problems in NR standard specification:

- Around 1 percent of EVM contribution have not been a problem for modulation schemes up to 64QAM. However, for modulation schemes as high as 256QAM or 1024QAM, this constitutes a problem due to more stringent requirements for EVM.

- Bandwidth efficiency above 90% [2] requires steeper filters with longer filter impulse response. As a result, more inter-symbol interference is expected to leak from adjacent OFDM symbols.

For these reasons, a shorter EVM window length is recommended.

Observation 1: The reuse of current EVM window based on 36.104 specification in NR creates several problems especially for high order modulation schemes (256QAM and 1024QAM) and high spectral utilization (above 90%).

The drawback of shorter EVM window length could be that more of the CP is utilized for the base station and less is remaining for channel delay spread or UE synchronization offset. This applies even more for larger subcarrier spacing (i.e. shorter CP). Therefore, the reduction of EVM window could take into account also this adverse effect.

Taking into account above considerations we propose to reduce EVM window length to around 60% of its original value (based on LTE). Thus, we propose to specify window length ‘W’ used in EVM calculation as 3.5 % of FFT length for various numerologies.

Proposal 1: Window length ‘W’ used in EVM calculation shall be 3.5% of FFT length for various numerologies.

Proposal 2: It is proposed to adopt table 3 to table 7 for EVM window length ‘W’ for normal CP.

Table 3 to table 5 present EVM window length ‘W’ for frequency range 1 (FR1) for normal CP and SCS 15 kHz, 30 kHz, 60 kHz respectively.

Table 3: EVM window length ‘W’ for FR1 for normal CP and SCS 15 kHz

| Channel Bandwidth (MHz) | Nominal FFT size | Sampling Rate (MHz) | for CP ≈ 7% FFT Size | EVM window length W |
| --- | --- | --- | --- | --- |
| 5 | 512 | 7.68 | 36 | 18 |
| 10 | 1024 | 15.36 | 72 | 36 |
| 15 | 1536 | 23.04 | 108 | 54 |
| 20 | 2048 | 30.72 | 144 | 72 |
| 25 | 2048 | 30.72 | 144 | 72 |
| 30 | 3072 | 46.08 | 216 | 108 |
| 40 | 4096 | 61.44 | 288 | 144 |
| 50 | 4096 | 61.44 | 288 | 144 |

Table 4: EVM window length ‘W’ for FR1 for normal CP and SCS 30 kHz

| Channel Bandwidth (MHz) | Nominal FFT size | Sampling Rate (MHz) | for CP ≈ 7% FFT Size | EVM window length W |
| --- | --- | --- | --- | --- |
| 5 | 256 | 7.68 | 18 | 8 |
| 10 | 512 | 15.36 | 36 | 18 |
| 15 | 768 | 23.04 | 54 | 26 |
| 20 | 1024 | 30.72 | 72 | 36 |
| 25 | 1024 | 30.72 | 72 | 36 |
| 30 | 1536 | 46.08 | 108 | 54 |
| 40 | 2048 | 61.44 | 144 | 72 |
| 50 | 2048 | 61.44 | 144 | 72 |
| 60 | 3072 | 92.16 | 216 | 108 |
| 70 | 3072 | 92.16 | 216 | 108 |
| 80 | 4096 | 122.88 | 288 | 144 |
| 90 | 4096 | 122.88 | 288 | 144 |
| 100 | 4096 | 122.88 | 288 | 144 |

Table 5: EVM window length ‘W’ for FR1 for normal CP and SCS 60 kHz

| Channel Bandwidth (MHz) | Nominal FFT size | Sampling Rate (MHz) | for CP ≈ 7% FFT Size | EVM window length W |
| --- | --- | --- | --- | --- |
| 10 | 256 | 15.36 | 18 | 8 |
| 15 | 384 | 23.04 | 27 | 13 |
| 20 | 512 | 30.72 | 36 | 18 |
| 25 | 512 | 30.72 | 36 | 18 |
| 30 | 768 | 46.08 | 54 | 26 |
| 40 | 1024 | 61.44 | 72 | 36 |
| 50 | 1024 | 61.44 | 72 | 36 |
| 60 | 1536 | 92.16 | 108 | 54 |
| 70 | 1536 | 92.16 | 108 | 54 |
| 80 | 2048 | 122.88 | 144 | 72 |
| 90 | 2048 | 122.88 | 144 | 72 |
| 100 | 2048 | 122.88 | 144 | 72 |

Table 6 to table 7 present EVM window length ‘W’ for frequency range 2 (FR2) for normal CP and SCS 60 kHz, 120 kHz respectively.

Table 6: EVM window length ‘W’ for FR2 for normal CP and SCS 60 kHz

| Channel Bandwidth (MHz) | Nominal FFT size | Sampling Rate (MHz) | for CP ≈ 7% FFT Size | EVM window length W |
| --- | --- | --- | --- | --- |
| 50 | 1024 | 61.44 | 72 | 36 |
| 100 | 2048 | 122.88 | 144 | 72 |
| 200 | 4096 | 245.76 | 288 | 144 |

Table 7: EVM window length ‘W’ for FR2 for normal CP and SCS 120 kHz

| Channel Bandwidth (MHz) | Nominal FFT size | Sampling Rate (MHz) | for CP ≈ 7% FFT Size | EVM window length W |
| --- | --- | --- | --- | --- |
| 50 | 512 | 61.44 | 36 | 18 |
| 100 | 1024 | 122.88 | 72 | 36 |
| 200 | 2048 | 245.76 | 144 | 72 |
| 400 | 4096 | 491.52 | 288 | 144 |

Table 8 presents EVM window length ‘W’ for frequency range 1 (FR1) for extended CP and SCS 60 kHz.

Proposal 3: It is proposed to adopt table 8 to table 9 for EVM window length ‘W’ for extended CP.

# Table 8: EVM window length for extended CP for FR1 and 60 kHz SCS

| Channel Bandwidth (MHz) | Nominal FFT size | Sampling Rate (MHz) | for CP = 25% FFT Size | EVM window length W |
| --- | --- | --- | --- | --- |
| 10 | 256 | 15.36 | 64 | 54 |
| 15 | 384 | 23.04 | 96 | 82 |
| 20 | 512 | 30.72 | 128 | 110 |
| 25 | 512 | 30.72 | 128 | 110 |
| 30 | 768 | 46.08 | 192 | 164 |
| 40 | 1024 | 61.44 | 256 | 220 |
| 50 | 1024 | 61.44 | 256 | 220 |
| 60 | 1536 | 92.16 | 384 | 330 |
| 70 | 1536 | 92.16 | 384 | 330 |
| 80 | 2048 | 122.88 | 512 | 440 |
| 90 | 2048 | 122.88 | 512 | 440 |
| 100 | 2048 | 122.88 | 512 | 440 |

Table 9 below specifies EVM window length (W) for extended CP for FR2, the cyclic prefix length  is 25% from Nominal FFT size for extended CP.

# Table 9: EVM window length for extended CP for FR2 and 60 kHz SCS

| Channel Bandwidth (MHz) | Nominal FFT size | Sampling Rate (MHz) | for CP = 25% FFT Size | EVM window length W |
| --- | --- | --- | --- | --- |
| 50 | 1024 | 61.44 | 256 | 220 |
| 100 | 2048 | 122.88 | 512 | 440 |
| 200 | 4096 | 245.76 | 1024 | 880 |

# 3.	Conclusion

In this contribution, we discussed the possible reduction of EVM window length for NR with different BW and SCS combinations with high spectrum utilization. We have made following observation and proposal:

Observation 1: The reuse of current EVM window based on 36.104 specification in NR creates several problems especially for high order modulation schemes (256QAM and 1024QAM) and high spectral utilization (above 90%).

Proposal 1: Window length ‘W’ used in EVM calculation shall be 3.5% of FFT length for various numerologies.

Proposal 2: It is proposed to adopt table 3 to table 7 for EVM window length ‘W’ for normal CP.

Proposal 3: It is proposed to adopt table 8 to table 9 for EVM window length ‘W’ for extended CP.

# References

- 3GPP TS 36.104, 	Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception

- R4-1711736 “WF on NR spectrum utilization”, ZTE

- R4-1713545, Consideration on EVM window length reduction for NR

- R4-1714024, On UE EVM for NR