| WI | # | Feature group | Components | Prerequisite feature groups | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) | Consequences if the feature is not supported by the UE | Type (See R4-17121 19) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Remarks | Responsible WG | Recommendation for TSG-RAN | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. NR RRM | 1-1 | Independent measurement gap configurations for FR1 and FR2 | 1) measurement gaps for FR1 and FR2 are configured independently |  | Yes | UE does not support independent gap configuration between FR1 and FR2 | Type 4 | No Need | No Need |  |  | RAN4 | Optional |  |
| 2. NR System parameters | 2-1 =>Follow WF | 60kHz of subcarrier spacing for FR1 | 1) whether UE supports 60kHz of subcarrier spacing for data channel in FR1 |  | Yes | UE does not support 60kHz of subcarrier spacing for data channel | Type 2 | No Need | Applicable only to FR1 |  |  | RAN4 | Optional |  |
|  | 2-2 => further discussion on LS from RAN2 received during this meeting | Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports | 1) FR1 channel bandwidths in TS38.101-1 Table 5.3.5-1 2) FR2 channel bandwidths in TS38.101-2 Table 5.3.5-1 |  | Yes | For FR1, all the bandwidths listed in TS38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC For FR2, UE does not support some UE channel bandwidths | Type 1 | No Need | No Need |  | Decision was made in RP-172832 RAN4 will discuss UE mandatory channel bandwidth for FR2 | RAN2/4 | Optional |  |
|  | 2-3 => More discussion | Simultaneous reception of data and SS block with different numerologies | 1) Whether UE supports simultaneous reception of data and SS block with different numerologies |  | Yes | UE does not support simultaneous reception of data and SS block with different numerologies | [Type 2] | No Need | No Need |  |  | RAN4 | Optional |  |
|  | 2-4 => Follow LS to RAN2 | Simultaneous reception and transmission for inter band CA or EN-DC (TDD-TDD or TDD-FDD) | 1) Simultaneous reception and transmission for inter CA or EN-DC (TDD-TDD or TDD-FDD) |  | Yes | UE does not support simultaneous reception and transmission for inter CA or EN-DC (TDD-TDD or TDD-FDD) | Type 3 | No Need | No Need |  | For band combination within FR1 (or FR2) bands Per band combination signaling | RAN4 | Optional |  |
|  | 2-5 => Follow WF | Mixed numerologies for CA | 1) Support simultaneous reception or transmission with different numerologies in CA |  |  |  |  |  |  |  | TBD |  |  |  |
|  | 2-6 => More discussion | Number of UE RX ports |  |  |  |  |  |  |  |  | FR1: UE supports 2 or 4 RX chains [RP-172788]. FR2: Number of transceiver chains for FR2 is FFS FFS if separate capability signalling is needed or can be coupled with number of MIMO layers E///: no need => this is proposed by intel but need time to discuss |  |  |  |
|  | 2-7 => More discussion | Number of UE TX ports |  |  |  |  |  |  |  |  | FFS if separate capability signalling is needed or can be coupled with number of MIMO layers E///: no need => this is proposed by intel but need time to discuss |  |  |  |
| 3. NR UE RF | 3-1 => More discussion with 3-2 | Single switched UL transmission in EN-DC | 1) single switched UL transmission in EN-DC |  | Yes | UE does not support single switched UL | Type 4 | No Need | Applicable only to FR1 |  | For UEs without dynamic LTE-NR power sharing capability, the support of single UL operation (Operation A with Case 1 in Slide 5 of RP-1722833) is mandatory with capability signaling. (Decision was made in RAN#78, RP-172833) | RAN4 | Optional |  |
|  | 3-2 => Type needs to be discussed | Dynamic transmission power sharing between NR and LTE in EN-DC | 1) dynamic transmission power sharing between NR and LTE in EN-DC |  | Yes | Support of single UL operation (operation A with Case 1) is mandatory | [Type 4] | No Need | Applicable only to FR1 |  | This feature is not mandatory in Rel.15. For UEs without this capability, the support of single UL operation (Operation A with Case 1 in Slide 5 of RP-1722833) is mandatory with capability signaling. (Decision was made in RAN#78, RP-172833). | RAN 1/4 | Optional |  |
|  | 3-3 => More discussion | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | 1) ~0us 2) <20us => what is “almost” 0us? |  | Yes | UE does not support UL subcarrier alignment between LTE and NR for EN-DC with LTE-NR coexistence in UL | Type 4 | No Need | Applicable only to FR1 |  |  | RAN4 | Mandatory only for UE supports EN-DC with LTE-NR coexistence in UL |  |
|  | 3-4 => More discussion with 3-3 | [Support of UL sharing from UE perspective] |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3-5 => proposed by intel but discussion is postponed | BWP switching delay |  |  |  |  |  |  |  |  | Optional “BWP switching delay needed” capability with per band granularity |  |  |  |
|  | 3-6 => proposed by intel but discussion is postponed | Non-contiguous intra-band CA frequency span |  |  |  |  |  |  |  |  | TBD |  |  |  |
|  | 3-7 => proposed by intel but discussion is postponed | PA calibration gap |  |  |  |  |  |  |  |  | TBD |  |  |  |

Additional list from RAN1 feature list

| WI | # | Feature group | Components | Prerequisite feature groups | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) | Consequences if the feature is not supported by the UE | Type (See R4-1712119) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Remarks | Responsible WG | Recommendation for TSG-RAN | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | x-x1 | 64QAM modulation for FR2 PDSCH | 64QAM modulation for FR2 PDSCH |  | Yes |  | NA |  | Applicable only to FR2 |  | RAN4 will check 64QAM modulation for FR2 => FFS: Mandatory without capability | RAN4 |  |  |
|  | x-x2 | pi/2-BPSK for PUSCH | pi/2-BPSK for PUSCH | 0-2 | Yes | pi/2-BPSK for PUSCH is not possible | [Type 3] |  |  |  | RAN4 will discuss if it is per band, common for all bands or FR1/2 QC, intel: per FR1/2 need time to check | RAN4 |  |  |
|  | x-x3 | 64QAM for PUSCH | 64QAM for PUSCH |  | Yes | 64QAM for PUSCH is not possible | [Type 3] |  |  |  | RAN4 will discuss if it is per band or common for all bands need time to check | RAN4 |  |  |
|  | x-x4 | 256QAM for PDSCH | 256QAM for PDSCH |  | Yes | 256QAM for PDSCH is not possible | [Type 3] |  |  |  | RAN4 will discuss if it is per band or common for all bands intel: per band for FR1, postpone for FR2 QC: per UE for FR1, for FR2 per band. optional for both FR1/2 E///: need operator view need time to check | RAN4 |  |  |
|  | x-x5 | 256QAM for PUSCH | 256QAM for PUSCH |  | Yes | 256QAM for PUSCH is not possible | [Type 3] |  |  |  | RAN4 will discuss if it is per band or common for all bands QC/intel: per band for FR1 intel: no need for FR2 in Rel.15 need time to check | RAN4 |  |  |
|  | x-x6 | Subcarrier spacings and FFT size in conjunction with supportable BW with normal CP | 1) 15kHz 2) 30 kHz 3) 60 kHz 4) 120 kHz |  | Yes |  | Type 4 [Type 3] |  |  |  | It is up to RAN4 decision Baseband processing (memory) in CA combination related as well as RF (SCS support is per band between sub6 and mmWave) => skipped since almost SCSs are mandatory except for 60kHz SCS. Mandating 60kHz SCS is now discussed in RAN4. | RAN4 |  |  |
|  | x-x7 | Extended CP | Extended CP | 0-10 (component 3; SCS60) | Yes |  | Type 4 | [No need] | [No need] |  | RAN4 to check => Need to check of RAN1 intention | RAN4 |  |  |
|  |  | pi/2-BPSK for PUCCH format 3/4 | pi/2-BPSK for PUCCH format 3/4 | 0-2 | Yes | pi/2-BPSK for PUCCH format 3/4 is not possible | Type 3 |  |  |  | RAN4 will discuss if it is per band, common for all bands, or FR1/2 QC, intel: per FR1/2 => need time to check |  |  |  |
|  | x-x8 | Non-contiguous UL CP-OFDM |  | 0-1 | Yes |  |  | No |  |  | It is up to RAN4 to decide => follow decision in UE RF session | RAN4 |  |  |
|  | x-x9 | [1-symbol GP for 120KHz SCS in unpaired spectrum] | 1) Slot formats with 1-symbol GP(s) for 120KHz SCS in unpaired spectrum |  | Yes |  | Type 4 | Applicable only to TDD | Applicable only to FR2 |  | RAN4 to check whether this feature is included in their list intel: need time to check => need time to check | RAN4 |  |  |
|  |  | DL MIMO layers (PDSCH MIMO layers) |  |  |  |  |  |  |  |  | - Maximum number of MIMO layers shall be supported => This is proposed by Intel but discussion is postponed. |  |  |  |
|  |  | UL MIMO layers (PUSCH MIMO layers) |  |  |  |  |  |  |  |  | - Maximum number of MIMO layers shall be supported => This is proposed by Intel but discussion is postponed. |  |  |  |
|  |  | Basic DL NR-NR CA operation | 1) Up to16 DL carriers 2) Same numerology across carrier for data/control channel [at a given time] |  | Yes |  | N.A. |  |  |  | This is conditioned on the support of DL CA band combination(s). The band combination definition is up to RAN4. => need time to check |  |  |  |
|  |  | Basic UL NR-NR CA operation | 1) Up to16 UL carriers 2) Same numerology across carrier for data/control channel [at a given time] 3) One PUCCH group 4) Single TAG | [6-4] | Yes |  | N.A. |  |  |  | This is conditioned on the support of UL CA band combination(s). The band combination definition is up to RAN4. => need time to check |  |  |  |
|  |  | 7.5kHz UL raster shift | 7.5kHz UL raster shift |  | Yes |  | Type 2 |  |  |  | => duplicated discussion between RAN1 and RAN4 | RAN4 |  |  |
|  |  | Supplemental uplink | Initial access and RRC connected operation on SUL carrier (incl 7.5kHz configurable shift) 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL | 6-14 | Yes | The UE will not be able to access or operate on a SUL carrier UE will not be able to perform the RACH/PUSCH/PUCCH/SRS operation in a band combination including SUL | Type 2 or 4 |  |  |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. => need time to check |  |  |  |
|  |  | Supplemental uplink with different numerology | Numerology other than that of associated DL | 6-15? | Yes | The UE will not be able to access or operate on a SUL carrier | Type 2 or 4 | Need Support not required for an FDD-only UE |  |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. => need time to check |  |  |  |
|  |  | Supplemental uplink with dynamic switch | DCI based selection of PUSCH carrier | 6-15, 6-16? | Yes |  | Type 2 or 4 | Need Support not required for an FDD-only UE |  |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. => need time to check |  |  |  |
|  |  | Simultaneous reception and transmission on different bands for each band combination |  |  |  |  | Type 2 | Yes |  |  | => duplicated discussion between RAN1 and RAN4 |  |  |  |