3GPP TSG-RAN WG4 Meeting AH-1801	R4-1801036

## San Diego, CA, USA, 22 – 26 Jan, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.104 | CR | CRNum | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104: correction of the conducted Rx in-band blocking requirement (7.4.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | WF agreements on the inband /OOB boundaries are hardly readable when appied in text. Therefore it is proposed to use ΔfOOB offset values, and to describe the conducted in-band blocking requirement based on parametrized approach in the text. Also correction of the unnecessary “basic limit” approach is proposed, i.e. based on the AAS discusions, the “basic limit” concept was supposed to be applied only to the requirements, which are subject to the emisisons scaling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 3.2: ΔfOOB definition introduction - 7.4.2: - Correction of the unnecessary “basic limit” approach - New table 7.4.2.2-0 with the ΔfOOB values for te 1-C and 1-H - Apply the parametrized description for blocking requirement based on the ΔfOOB offset - Simplification of the narrowband requirement in table 7.4.2.2-2 - Wording alignment with other blocking requirements - Reference corrections, editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification of the blocking requirement for various NR operating bands; wrong references. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 7.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | In the revision, the following improvements were considered: - Correction of the ΔfOOB definition - Inclusion of the upper 900MHz operating band limit in table 7.4.2.2-0 |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BWChannel	Channel bandwidth

BWConfig	Transmission bandwidth configuration, expressed in MHz, where BWConfig = NRB x SCS x 12 kHz.

f 	Separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency

fmax 	f_offsetmax minus half of the bandwidth of the measuring filter

ΔfOBUE 	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

EISminsens		The EIS declared for the minimum OSDD

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

f_offset	Separation between the channel edge frequency and the centre of the measuring filter

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

NRXU,active	The number of active receiver units. The same as the number of demodulation branches to which compliance is declared for chapter 8 performance requirements.

NRXU,counted	The number of active receiver units that are taken into account for unwanted emission scaling, as calculated in subclause 7.1.

NRXU,countedpercell	The number of active receiver units that are taken into account for unwanted emissions scaling per cell, as calculated in subclause 7.6. The number is defined in subclause 7.1.

NTXU,counted	The number of active transmitter units , as calculated in subclause 6.1, that are taken into account for conducted TX power limit in subclause 6.2, and for unwanted emissions scaling.

NTXU,countedpercell	The number of active transmitter units  that are taken into account for emissions scaling per cell, as calculated in subclause 6.6. The number is defined in subclause 6.1.

Pmax,c,AC		Maximum carrier output power measured per antenna connector, or per TAB connector, and corresponding to the declared rated carrier output power (Prated,c,TABC, or Prated,c,sys)

Pmax,c,TABC	The maximum carrier output power per TAB connector

Pmax,t,AC		Maximum total output power measured per antenna connector, or per TAB connector, and corresponding to the declared rated total output power (Prated,t,TABC, or Prated,t,sys)

Pmax,c, TRP		Maximum carrier TRP output power measured at the RIB(s), and corresponding to the declared rated carrier TRP output power (Prated,c,TRP)

Pmax,t, TRP		Maximum total TRP output power measured at the RIB(s), and corresponding to the declared rated total TRP output power (Prated,t,TRP)

Prated,c,sys	The sum of Prated,c,TABC for all TAB connectors for a single carrier

Prated,c,TABC	The rated carrier output power per TAB connector

Prated,t,TABC	The rated total output power per TAB connector

Prated,c,TRP	Rated carrier TRP output power declared per RIB

Prated,t,TRP	Rated total TRP output power declared per RIB

------------------------------ Next modified section ------------------------------

### 7.4.2	In-band blocking

#### 7.4.2.1	General

The in-band blocking characteristics is a measure of the receiver’s ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

#### 7.4.2.2	Minimum requirement for BS type 1-C and BS type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel

, with a wanted and an interfering signal coupled to BS antenna input using the parameters in tables 7.4.2.2-1, 7.4.2.2-2 and 7.4.2.2-3 for general blocking and narrowband blocking requirements. The reference measurement channel for wanted signal is identified in subclause 7.2.2 for each channel bandwidth and further specified in annex A.

The in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For BS type 1-C and BS type 1-H the in-band blocking requirement applies from FUL_low - ΔfOOB to FUL_high + ΔfOOB

, excluding the downlink frequency range of the operating band. The ΔfOOB for or BS type 1-C and BS type 1-H is defined in table 7.4.2.2-0.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C and at the TAB connector for BS type 1-H.

Table 7.4.2.2-0: ΔfOOB offset for NR operating bands

| BS type | Operating band characteristics | ΔfOOB [MHz] |
| --- | --- | --- |
| BS type 1-C | 200 MHz ≥ FUL_high – FUL_low | 20 |
|  | 200 MHz < FUL_high – FUL_low ≤ 900 MHz | 60 |
| BS type 1-H | 100 MHz > FUL_high – FUL_low | 20 |
|  | 100 MHz ≤ FUL_high – FUL_low ≤ 900 MHz | 60 |

[For a BS operating in non-contiguous spectrum within any operating band, the in-band blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in tables 7.4.2.2-1 and 7.4.2.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For a BS capable of multi-band operation, the blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in tables 7.4.2.2-1 and 7.4.2.2-3.]

Table 7.4.2.2-1: Base station general blocking requirement

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | PREFSENS + 6 dB | Wide Area: -43 Medium Range: -38 Local Area: -35 | 7.5 | 5 MHz NR signal SCS: 15 kHz |
| [25, 30, 40, 50, 60, 70, 80,90, 100] | PREFSENS + 6 dB | Wide Area: -43 Medium Range: -38 Local Area: -35 | 30 | 20 MHz NR signal SCS: 15 kHz |

Table 7.4.2.2-2: Base station narrowband blocking requirement

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] |
| --- | --- | --- |
| 5, 10, 15, 20, [25, 30, 40, 50, 60, 70, 80,90, 100] (NOTE) | PREFSENS + 6 dB | Wide Area: -49 Medium Range: -44 Local Area: -41 |
| NOTE: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth |  |  |

Table 7.4.2.2-3: Base station narrowband blocking interferer frequency offsets

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset to the band edge of the wanted carrier [kHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [342,5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz NR signal, 1 RB SCS: 15 kHz |
| 10 | [347.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | [352.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | [342.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20 MHz NR signal, 1 RB SCS: 15 kHz |
| 30 | [562.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 40 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 50 | [552.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 60 | [562.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 70 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 80 | [552.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 90 | [562.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 100 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |

#### 7.4.2.3	Void

#### 7.4.2.4	Void

------------------------------ End of modified sections ------------------------------