3GPP TSG-RAN WG4 AH-1801	R4-1801184

San Diego, USA, 22 – 26 January 2018

Source:	Ericsson

Title:	TP to TR 37.863-02-01: TP for DC_66A_(n)71B

Agenda item:	4.1.2.3

Document for:	Approval

# 1. Introduction

This is a text proposal for 37.863-02-01 for DC_66A_(n)71B.

# 2. Text Proposal for TR 37.863-02-01

##### ---Start of changes---

## 6.x	DC_66A_(n)71B

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_(n)71B | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 50 MHz | 80 MHz | 100 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] | Total Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_(n)71B | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 20 | 40 |
|  | 71 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 20 |  |
|  | n71 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

The DC_71b_n71b part is a contiguous allocation thus only uplink harmonic analysis is needed.

Table 6.x.3-1: DC_71b_n71b UL harmonics

| Uplink band |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 71 | 663 | 698 | 1326 | 1396 | 1989 | 2094 |

From Table 6.x.3-1 it can be seen that uplink harmonics 3rd harmonic overlaps with bands 3, 25, 34 and 70.

Table 6.x.3-2 lists Band 66 +Band 71/n71 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.x.3-2: Band 66 and Band 71/n71 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 |  | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 |  | 1396 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 |  | 2094 | 5130 | 5340 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 1117 | 1012 |  | 2373 | 2478 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 454 |  | 314 | 2722 | 2897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
|  | 3036 |  | 3176 | 4083 | 4258 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
|  | 2234 |  | 2024 | 4746 | 4956 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
|  | 209 | 384 |  | 4432 | 4677 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
|  | 3699 |  | 3874 | 5793 | 6038 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ |  | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
|  | 6457 |  | 6142 | 1082 | 872 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
|  | 7503 |  | 7818 | 4362 | 4572 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
|  | 4014 |  | 3734 | 1326 | 1571 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
|  | 6456 |  | 6736 | 5409 | 5654 |

Based on Table 6.x.3-2;

- 2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 77 and 78.

- 3rd order harmonic may fall into Rx frequencies of bands 2, 25, 34, 36, 46 and 70.

- 2nd order IMD may fall into Rx frequencies of band 30 and 40.

- 3rd order IMD may fall into Rx frequencies of band 77.

- 4th order IMD may fall into own Rx frequencies of band 66 as well as Rx frequencies of band 1, 4, 10, 23, 34, 43, 47, 48, 65, 77, 79 and 79.

- 5th order IMD may fall into Rx frequencies of band 5, 6, 8, 11, 18, 19, 21, 24, 26, 32, 43, 45, 46, 50, 74, 75, 76, 77, 78 and 79.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.x.3-3 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.x.3-3: 2UL Band 66 +Band n71 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD4, IMD5 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, IMD5 |

Table 6.x.3-4 lists the protected bands required for the dual connectivity configuration.

Table 6.x.3-4: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A_(n)71B | E-UTRA Band 4, 5, 7,10, 13, 14, 17, 22, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

### 6.x.4	∆TIB and ∆RIB values

For DC_66A_(n)71B, the TIB,c and RIB values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66A_(n)71B | 66 | 0.3 |
|  | 71 | 0.3 |
|  | n71 | 0.3 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66A_(n)71B | 66 | 0 |
|  | 71 | 0 |
|  | n71 | 0 |

### 6.x.5	MSD

Table 6.x.5-1 lists the MSD required for the dual connectivity configuration DC_66A_(n)71B.

Table 6.x.5-1: MSD for the DC_66A_(n)71B configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_66A_(n)71B | 66 | 1750 | 5 | 25 | 2150 | TBD | FDD | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A |  |  |

##### ---End of changes---