3GPP TSG-RAN WG4 Meeting AH-1801	R4-1800343

San Diego, CA, USA, 22 – 26 January 2018

Agenda Item:	4.6.5.2.2.

Source:	Ericsson

Title:	CSI-RS based measurement reporting time requirements

Document for:	Discussion

# 1	Introduction

RAN1/RAN2 agreed to use CSI-RS for the measurement reference symbols for both intra- and inter-frequency measurements as well as SS/PBCH blocks.

TS 38.331 [1] defines the CSI-RS based inter-/intra-frequency measurements as follows:

- CSI-RS based intra-frequency measurements: measurements at CSI-RS(s) resource(s) of configured neighbour cell(s) whose bandwidth(s) are within the bandwidth(s) of the CSI-RS resource(s) on the serving cell(s) configured for measurements and having the same subcarrier spacing of the CSI-RS resource(s) on the serving cell(s) configured for measurements.

- CSI-RS based inter-frequency measurements: measurements at CSI-RS(s) resource(s) of configured neighbour cell(s) whose bandwidth(s) are not within the bandwidth(s) or having different subcarrier spacing compared to the CSI-RS resource(s) on the serving cell(s) configured for measurements.

TS 38.331 also describes the network configuration for CSI-RS based measurements as follows:

The network may configure the UE to report the following measurement information based on CSI-RS resources:

- Measurement results per CSI-RS resource.

- Measurement results per cell based on CSI-RS resource(s).

- CSI-RS resource measurement identifiers.

In this contribution we discuss the CSI-RS resource indication used for CSI-RS based measurements

# 2	CSI-RS based measurements

When UE is in RRC_CONNECTED, the network may configure SS/PBCH (SSB) based and/or CSI-RS based measurement configuration. The CSI-RS based measurements are similar to the SSB based measurement; UE measures the measurement quantities, e.g., RSRP/RSRQ/SINR based on the specified CSI-RS resource(s). If the reporting condition is satisfied, UE reports the measurement result as event reporting or measurement resource index, depending on the configuration.

For the CSI-RS based measurements, for each measurement resource index (or resource id), UE should know the measured CSI-RS resource information such as time/frequency domain scheduling, resource mapping, and CSI-RS sequence generation. Moreover, RAN1 has agreed that CSI-RS measurement resources are associated with NR cell ID. In the appendix, we list the necessary CSI-RS resource information agreed by RAN1 although RAN2 is still working for the detailed the measurement resource information elements.

From the RAN4 RRM requirement point of view, our interest is the CSI-RS based measurement time. Based on the RAN1 agreement, we can consider two scenarios 1) the network provides the associated SSB information or 2) not. Note UE need to decode PSS/SSS for the associated physical cell id acquisition regardless of the scenarios.

When the network provides the associated SSB information, UE should identify the associated SSB index for time reference, and it is the same definition as TSSB_time_index discussed in the SSB based measurement. Therefore UE needs to acquire PBCH-DMRS sequence for FR1, and decode PBCH for FR2.

When the network does not provide the associated SSB information, on the other hand, according to RAN1 agreement, UE may assume that the carrier is synchronized with the serving cell. We think this scenario is also applicable to the case SSB index is known to UE or can be simply calculated, e.g., it is enough to detect the index for only one SSB for the same cell.

Observation: there are two scenarios for CSI-RS based measurement: one for the case the network provides the associated SSB information; another for the case the network does not provide the associated SSB information or the SSB index is known to UE.

Based on our observation, the measurement time for CSI-RS based measurement should be specified as follows:

Case 1: the network does not provide the associated SSB information or the SSB index is known to UE

Tidentify_without_ssb = TPSS/SSS_snyc + TCSI-RS_measurement_period

Case 2: the network provides the associated SSB information

Tidentify_with_ssb = TPSS/SSS_snyc + TCSI-RS_measurement_period + TSSB_time_index

where TPSS/SSS_sync is the time period used in PSS/SSS detection, TSSB_time_index is the time period used to acquire the index of the SSB being measured, and TCSI-RS_measurement_period is the measurement period of CSI-RS based measurement.

For cell identification time TPSS/SSS_snyc and SSB time index acquisition time TSSB_time_index, we can reuse the results for the SSB based measurement. For measurement time, TCSI-RS_measurement_period, RAN4 need study the necessary measurement time.

# 3	Conclusions

Observation: there are two scenarios for CSI-RS based measurement: one for the case the network provides the associated SSB information; another for the case the network does not provide the associated SSB information or the SSB index is known to UE.

Proposal: CSI-RS based measurement time is specified as follows:

Case 1: the network does not provide the associated SSB information or the SSB index is known to UE

Tidentify_without_ssb = TPSS/SSS_snyc + TCSI-RS_measurement_period

Case 2: the network provides the associated SSB information

Tidentify_with_ssb = TPSS/SSS_snyc + TCSI-RS_measurement_period + TSSB_time_index

# 4	References

3GPP TS38.331 “NR; Radio Resource Control (RRC); Protocol specification”, v1.0.0.

3GPP TS38.211 “NR; Physical channels and modulation”, v2.0.0.

# Appendix

## A.1	CSI-RS based measurement related agreements from RAN1#91 chairman note

Agreements:

- For each CSI-RS resource, at most one associated SSB can be configured

- If associated SSBs are configured for CSI-RS, maximum N1=96 number of CSI-RS resources can be configured per frequency layer

  - M>=1 number of CSI-RS resources per associated SSB can be configured

- If associated SSBs are not configured for CSI-RS, maximum N2>=1 number of CSI-RS resources can be configured per frequency layer

  - In this case, UE may assume that the carrier is synchronized with the serving cell.

  - FFS UE is not required to perform measurement based on CSI-RS if the corresponding cell ID is not detected

Agreements:

- For carriers with SSB,

  - Transmission BW is removed from the RRC parameter set.

  - Agree on the following table for the CSI-RS configuration parameters:

| Parameter Name | Description | Candidate values | Commonality of the configured value(s) across multiple resources |
| --- | --- | --- | --- |
| Cell_ID | Physical Cell ID for CSI-RS | 0, 1, …, 1007 | Common value is assigned across all the resources configured for a cell |
| slotConfig | Contains periodicity and slot offset for periodic/semi-persistent CSI-RS FFS slot offset details (no additional RRC impact) | Periodicity: {5, 10, 20, 40} msec Offset: 0, 1, …, P-1 slots, where periodicity P is in terms of slots in the CSI-RS numerology | Configured per resource |
| Sequence-Generation | Sequence generation parameter for CSI-RS, i.e., scrambling ID | 0-1023 | Configured per resource |
| Common-PRB-Grid-offset | Information to define common PRB grid for CSI-RS sequence generation Corresponds to an offset (in terms of number of subcarriers in CSI-RS numerology) between PRB 0 for common PRB indexing and a reference location (i.e., corresponds to RRC parameter DL-BWP-loc defined for BWP) – the reference location is the lowest PRB of the cell-defining SSB | [0,1,2, …, >276*4] | A common value is assigned across all the resources configured for per cell |
| Measurement-BW | Allowed measurement BW for CSI-RS | BW size: {24, 48, 96, 192, 268} PRBs in CSI-RS numerology Starting PRB index: {0, 1, …, [251](=274-24+1)]} PRBs with respect to PRB0 in CSI-RS numerology | A common set of values are assigned across all the resources configured per cell |
| Carrier-info | Provides frequency location information for inter-frequency measurement | Absolute frequency value, RAN2 to fill in, if not provided elsewhere | A common value is assigned across all the resources configured for a frequency carrier |
| RE-Mapping-Pattern | RE mapping pattern for a CSI-RS resource | Adopt the parameter values agreed in BM | Configured per resource |
| Density | Frequency domain density for the 1-port CSI-RS | {1,3} | A common value is assigned across all the resources configured per cell |
| Numerology | Numerology for CSI-RS | {15,30,60} for sub6GHz {60,120,240} for over6GHz | A common value is assigned across all the resources configured per frequency carrier |
| Associated-SSB | For each CSI-RS resource, at most one associated SSB can be configured Note: If the associated-SSB is signaled, UE is not required to monitor CSI-RS resource(s) when the UE cannot detect the associated SSB | {0, 1, …, L-1} | Configured per resource This field is optional per frequency layer |
| QCLed-SSB | The CSI-RS resource is either QCL’ed not QCL’ed with the associated SSB in spatial parameters | {yes, no} | Configured per resource This field is conditionally indicated if associated-SSB is configured |

Agreements:

- Up to RAN2 to indicate to the UE the associated SS block on a carrier for different carrier(s) without SSB