3GPP TSG-RAN4 Meeting AH#1807 					R4-1808688

Montreal, Canada, 2nd July – 6th July, 2018

Title:	Discussion on SCell activation requirements in FR2

Source:	CATT

Agenda Item:	5.2.8.3

Document for:	Discussion

- Introduction

In RAN4#87 meeting, RAN4 had extensive discussion on SCell known condition and activation delay requirements, and the CR [1] for Scell activation/deactivation delay requirements in FR1 was agreed. In this contribution, we further discuss the SCell known conditions in FR2 and SCell activation delay requirements in FR2.

- Discussion

In last RAN4 meeting, we discussed the condition of known cell in [2] and agreed that MIB reading is not necessary for SCell activation. The related agreement was captured as follows:

Agreement:

MIB reading is not necessary for SCell activation

And the side condition for FR1 known cell was agreed in [1], the Scell in FR1 is known if UE has sent a valid measurement report for Scell being activated and the SSB measured remains detectable during the last max([5] measCycleSCell,  [5] DRX cycles)ms. In FR2, beam sweeping shall be considered in FR2, and the SSBs corresponding to RX beams of SCell being activated shall be remain detectable during the last max([5] measCycleSCell, [5] DRX cycles)ms. so, We think the same approach can be reused for FR2.

Proposal 1: The side condition defined for FR1known SCell can be reused with FR2 known SCell.

Proposal 2: The side condition for SCell known can be updated as follows:

SCell is known if it has been meeting the following conditions:

-	During the period equal to max([5] measCycleSCell, [5] DRX cycles)ms before the reception of the SCell activation command:

- the UE has sent a valid measurement report for SSB measured of SCell being activated and

- the SSB measured remains detectable according to the cell identification conditions.

-	the SSB measured during the period equal to max([5] measCycleSCell, [5] DRX cycles) ms also remains detectable during the SCell activation delay according to the cell identification conditions.

Otherwise SCell is unknown.

In [3], the SCell activation delay Tactivation_time for FR1 is defined as follows:

Tactivation_time is the SCell activation delay. If the SCell is known and belongs to FR1, Tactivation_time is:

-	[3ms+ 1*SMTC periodicity+4*OFDM symbol], if the SCell measurement cycle is equal to or smaller than [160ms].

-	[3ms+2*SMTC periodicity+4*OFDM symbol], if the SCell measurement cycle is larger than [160ms].

If the SCell is unknown and belongs to FR1, Tactivation_time is:

-	[3ms+ 4*SMTC periodicity+4*OFDM symbol] provided the SCell can be successfully detected on the first attempt.

As we discussed in handover delay requirements in [4], it was agreed that there is no need to differentiate known/unknown when defining handover delay requirements. And we think that we can keep the same principle for defining SCell activation requirements. Thus, since the beam sweeping shall be considered in FR2, the SCell being activated can be assumed as unknown cell for specifying the delay requirement in FR2.

Proposal 3: It is proposed not to differentiate known/unknown SCell when defining SCell activation delay requirement.

Proposal 4: The SCell activation delay Tactivation_time for FR2 is defined as [3ms+ N1*4* SMTC periodicity+4*OFDM symbol]

- Conclusion

In this contribution, we further discuss the SCell known conditions in FR2 and SCell activation delay requirements in FR2, and provide the proposals as follows:

Proposal 1: The side condition defined for FR1known SCell can be reused with FR2 known SCell.

Proposal 2: The side condition for SCell known can be updated as follows:

SCell is known if it has been meeting the following conditions:

-	During the period equal to max([5] measCycleSCell, [5] DRX cycles)ms before the reception of the SCell activation command:

- the UE has sent a valid measurement report for SSB measured of SCell being activated and

- the SSB measured remains detectable according to the cell identification conditions.

-	the SSB measured during the period equal to max([5] measCycleSCell, [5] DRX cycles) ms also remains detectable during the SCell activation delay according to the cell identification conditions.

Otherwise SCell is unknown.

Proposal 3: It is proposed not to differentiate known/unknown SCell when defining SCell activation delay requirement.

Proposal 4: The SCell activation delay Tactivation_time for FR2 is defined as [3ms+ N1*4* SMTC periodicity+4*OFDM symbol]

- Reference

[1]	R4-1808522,	“CR to 38.133 on NR Scell activation and deactivation delay”,	 CATT

[2]	R4-1807958,	“Ad hoc minutes on NR RLM”, Huawei, HiSilicon

[3]	TS38.133 V15.2.0

[4]	R4-1808485,	“CR on TS38.133 for intra-NR handover”, Huawei, HiSilicon