3GPP TSG-RAN WG4 Meeting #AH1807 	R4-1808727

Montreal, Canada, July 2 – 6, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | CRNum | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on interruption due to BWP switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requirements for interruption due to BWP switching for EN-DC and NR CA need to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section 8.2.1.X and 8.2.2.2.X are added to specify interruption due to BWP switching requirements for EN-DC and NR CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 8.2.1.2.X, 8.2.2.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1>

##### 8.2.1.2.X	Interruptions at BWP reconfiguration

The requirements in this clause shall apply for the UE configured with only PSCell or with PSCell and one or more SCells.

When BWP switch occurs on the NR PSCell/SCell in EN-DC, the UE is allowed to

-	an interruption on E-UTRA PCell/SCell:

-	of up to X2 slot, if the NR PSCell/SCell with BWP switching is in FR1 and it is not in the same band as the E-UTRA PCell, or

-	of up to Y2 slot + SMTC duration if the NR SCell with BWP switching is in FR1 and it is in the same band as the E-UTRA PCell;

-	an interruption on any other serving NR PSCell/SCell:

-	of up to X2 slot, if the NR PSCell/SCell with BWP switching is in the same FR but not in the same band as the NR PSCell/SCell being interrupted, or

-	of up to Y2 slot + SMTC duration if the NR PSCell/SCell with BWP switching is in the same band as the NR PSCell/SCell being interrupted.

The values of X2 and Y2 are defined in Table 8.2.1.2.4-1.

< END OF CHANGE #1>

< START OF CHANGE #2>

##### 8.2.2.2.X	Interruptions at BWP reconfiguration

The requirements in this clause shall apply for the UE configured with only PCell or with PCell and one or more SCells.

When BWP switch occurs on the NR SCell in NR CA, the UE is allowed to an interruption on the other NR serving cell

-	of up to the duration shown in Table 8.2.2.2.2-1, if the NR serving cell with BWP switching is in the same FR but not in the same band as the NR serving cell being interrupted, or

-	of up to the duration shown in Table 8.2.2.2.2-2, if the NR serving cell with BWP switching is in the same band as the NR serving cell being interrupted.

< END OF CHANGE #2>