3GPP TSG RAN WG4 Meeting AH-1807		R4- 1808768

Montreal, Canada, 2nd – 6th July 2018

Source: 	NTT DOCOMO, INC.

Title: 	Link level evaluation results on SS RSRP accuracy

Agenda item:	5.3.1

Document for:	Discussion

- Introduction

At the last RAN4 meeting, simulation assumption for link level evaluation on RSRP, RSRQ, SINR estimation accuracy were agreed [1]. In this contribution, we provide evaluation results on SSB based RSRP accuracy.

- Evaluation results

Link level evaluation results on SS-RSRP measurement based on agreed simulation assumption are shown in this section. Evaluation assumptions we selected are summarized in Appendix.

Results on 4 GHz carrier frequency

Figure 1 shows RSRP estimation error CDF curves for SCS = 15 kHz, and Figure 2 shows RSRP that for SCS = 30 kHz, respectively. 5%-ile, 50%-ile, and 95%-ile CDF values of RSRP estimation error are summarized in Table I.

Figure 1: RSRP estimation error CDF curves (SCS=15 kHz, left: EPA5, right: ETU30)

Figure 2: RSRP estimation error CDF curves (SCS=30 kHz, EPA5)

Table I: 5%-ile, 50%-ile and 95%-ile CDF values of RSRP error for SCS = 15 kHz and 30 kHz

| RSRP Error (dB) | 5% CDF | 50% CDF | 95% CDF |
| --- | --- | --- | --- |
| 5 MHz, SCS=15 kHz, EPA5 (sync) | -1.82 | -0.10 | 1.49 |
| 5 MHz, SCS=15 kHz, ETU30 (sync) | -2.57 | -0.10 | 1.59 |
| 10MHz, SCS=30 kHz, EPA5 (sync) | -2.93 | -0.22 | 1.48 |

Results on 30 GHz carrier frequency

Figure 3 and Figure 4 show RSRP estimation error CDF curves for SCS = 120 kHz and 240 kHz. 5%-ile, 50%-ile and 95%-ile CDF values of RSRP estimation error are summarized in Table II.

Figure 3: RSRP estimation error CDF curves (left: SCS=120 kHz, right: SCS=240 kHz, CDL-C (3 km/h))

Figure 4: RSRP estimation error CDF curves (left: SCS=120 kHz, right: SCS=240 kHz, CDL-C (30 km/h))

Table II: 5%-ile, 50%-ile and 95%-ile CDF values of RSRP error for the case of SCS = 120 kHz and 240 kHz

| RSRP Error (dB) | 5% CDF | 50% CDF | 95% CDF |
| --- | --- | --- | --- |
| 50 MHz, SCS=120 kHz, CDL-C, 3 km/h (sync) | -2.46 | -0.34 | 1.32 |
| 50 MHz, SCS=240 kHz, CDL-C, 3 km/h (sync) | -2.22 | -0.20 | 1.19 |
| 50 MHz, SCS=120 kHz, CDL-C, 30 km/h (sync) | -2.78 | -0.64 | 0.69 |
| 50 MHz, SCS=240 kHz, CDL-C, 30 km/h (sync) | -3.62 | -1.20 | 0.49 |

- Conclusion

In this contribution, we provided evaluation results on SS-RSRP estimation accuracy based on agreed link level simulation assumption.

# Reference

- 3GPP, R4-1806551, Ericsson, “Link-level simulation assumptions for SSB-based measurement accuracy,” May 2018

# Appendix

Table A-I and A-II show the agreed simulation assumption for link level evaluation on SS-RSRP, RSRQ, SINR estimation accuracy [1]. Some parameters we selected are highlighted by red in the tables.

Table A-I: General parameters

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 4 GHz | 30 GHz |
| System bandwidth | 5 MHz; 10 MHz | 50 MHz; 100 MHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes | No / Yes |
| DRX | No | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions | 1 tx or single layer transmissions |
| UE receive antennas | Case A: 2 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx (with rx beamforming Note1) | Case A: 2 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx (with rx beamforming Note1) |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 5 | 5 |
| SS burst set configuration in Cell 1 and Cell 2: |  |  |
| - Subcarrier spacing | 15 kHz; 30 kHz | 120 kHz; 240 kHz |
| - Number of SS blocks per SS burst set | 1 | 1 |
| - SS burst periodicity | 5 ms | 5 ms |
| - Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| - Actual SS block transmissions | always transmitted | always transmitted |
| NOTE1: For Case B, companies are encouraged to state their assumptions on beamforming and rx antenna configuration together with the provided simulation results |  |  |

Table A-II: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS and NR-SSS sequences | - | To be indicated by companies | To be indicated by companies |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | 3 ms |
| SNR (beamforming gain is included for Case B) | dB | 6 dB 3.3 dB | 1 dB |
| Es/IoT (calculated from SNR) | dB | N/A | -5.97 dB -3.97 dB |
| Propagation conditions | - | * For 4 GHz: AWGN, EPA5, ETU30 (for 15 kHz SCS), ETU70 (for 15 kHz SCS), TDL-A/B/C Note (delay spread: 30 ns and 300 ns; UE speed: 3 km/h and 30 km/h) * For 30 GHz: CDL-C, optional TDL-A/B/C Note (delay spread: 30 ns and 300 ns; UE speed: 3 km/h and 30 km/h) |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |