3GPP TSG RAN WG4 Meeting NR-AH#1807		R4-1808770

Montreal, Canada, 2nd – 6th July, 2018

Source: 	NTT DOCOMO, INC.

Title: 	General principle for NR RRM test cases

Agenda item:	5.3.3.1

Document for:	Discussion

- Introduction

The discussion on NR RRM performance has already started from April meeting. In this contribution, we provide our views on general principle for NR RRM test cases.

- Discussion

2.1 NR RRM test case list

In general, RAN4 should specify all of test cases for each core requirement, except for some requirements which cannot be tested, e.g., cell phase synchronization accuracy. The baseline of NR RRM test case list can reuse that of LTE, however some test cases for new features, e.g., Link recovery procedures and L1-RSRP measurement, need to be added. Table 1 shows our views on which test cases are needed at least.

Table 1: Proposed test cases for NR RRM

| Corresponding requirements |  | comments |
| --- | --- | --- |
| RRC_IDLE/Inactive state mobility | Cell reselection | for SA |
| RRC_CONNECTED state mobility | Handover | for SA |
|  | RRC Re-establishment | for SA |
|  | Random access |  |
|  | RRC Connection Release with Redirection | for SA |
| Timing | UE transmit timing |  |
|  | Timing advance |  |
| Signalling characteristics | Radio Link Monitoring |  |
|  | Link reconfiguration | New test cases for NR |
|  | Interruption |  |
|  | PSCell addition and release Delay |  |
|  | SCell Activation and Deactivation Delay |  |
| Measurement Procedure | Intra-frequency measurements |  |
|  | Inter-frequency measurements |  |
|  | Inter-RAT measurements |  |
|  | L1-RSRP measurements | New test cases for NR |
|  | Monitoring of Multiple Layers |  |
|  | Monitoring of Multiple SSBs | New test cases for NR |
|  | SFTD measurement (before and after NR PSCell is configured) | New test cases for NR |

Proposal 1:

Test cases for NR RRM in Table 1 should be defined at least, including following new test cases from LTE.

- Link recovery procedures for beam management

- L1-RSRP measurements

- Monitoring of Multiple SSBs

- SFTD measurement before PSCell configured.

  - Both interruption-based and gap-based solution are needed.

2.2 Basic parameters

Unlike LTE, there are a lot of configurable parameters for NR, e.g., subcarrier spacing, bandwidth and the number of SSBs to be measured, and some core requirements are different depending on such parameters, especially subcarrier spacing. In general, all of configurable parameters should be included in each test case no matter whether such parameters affect each core requirement or not since each operator would utilize different configuration.

In addition, some special cases which don’t happen in LTE case should be considered, e.g., the full overlapping or partial overlapping cases between SMTC, measurement gap and RLM. Since each collision case has different measurement delay requirements and the principle of measurement procedure, some test cases are needed to ensure the appropriate procedure on such aspects.

Proposal 2:

At least following parameters and cases needed to be considered to define each test case:

- Subcarrier spacing

  - In general, all of subcarrier spacing should be included for each test cases.

- The number of SSBs to be measured

  - Following parameters would also be affected.

    - SMTC

    - the number of RLM-RSs

- Collision cases between SMTC, measurement gap and RLM

  - The requirements when gap sharing factor are configured would also be tested.

- Conclusion

In this contribution, we provided our view on general principle for NR RRM test cases. Our proposals are as follows:

Proposal 1:

Test cases for NR RRM in Table 1 should be defined at least, including following new test cases from LTE.

- Link recovery procedures for beam management

- L1-RSRP measurements

- Monitoring of Multiple SSBs

- SFTD measurement before PSCell configured.

  - Both interruption-based and gap-based solution are needed.

Proposal 2:

At least following parameters and cases needed to be considered to define each test case:

- Subcarrier spacing

  - In general, all of subcarrier spacing should be included for each test cases.

- The number of SSBs to be measured

  - Following parameters would also be affected.

    - SMTC

    - the number of RLM-RSs

- Collision cases between SMTC, measurement gap and RLM

  - The requirements when gap sharing factor are configured would also be tested.