3GPP TSG-RAN WG4 Meeting AH1807	R4-1808811

Montreal, Canada, 2 – 6 July 2018

Agenda Item:	4.1.2.2.2

Source:	Ericsson

Title:	CQI reporting tests for DMRS-based PDSCH

Document for:	Discussion

# 1	Introduction

RAN4#87 agreed with the way forward on sTTI demodulation and CQI reporting test [1]. In this contribution, we discuss the CQI to MCS table and show the simulation results.

- Consider wideband sPDSCH scheduling with PUSCH 1-1 with EPA5 channel for CQI reporting performance requirement with both CRS-based TM4 and DRMS-based TM9 sPDSCH. - Compute CQI2MCS mapping table based on slot/subslot index. - Companies are encouraged to provide: - CQI2MCS mapping table - Simulation results of CQI reporting - Proposal of test points

# 2	Discussion

## 2.1	SPDCCH setup

Since Slot/Subslot-PDSCH is scheduled SPDCCH, and SPDCCH PRB uses the PDSCH region, we need consider the SPDCCH parameter. As same as CRS-based wideband CQI reporting tests [2], we have investigated the SPDCCH performance with parameters in Table 1, and the simulation results are shown in Figure 1. From the simulation results, the required SNR points of SPDCCH with AL4 are larger than 2dB to satisfy 1% BLER. Considering the possible test points for CQI reporting tests, we propose to set AL8 for SPDDCH.

Proposal 1: Configure AL8 for SPDCCH for DMRS-based CQI test.

Table 1	SPDCCH parameters for Wideband CQI test.

|  | SPDCCH parameters |
| --- | --- |
| Reference symbol | DMRS |
| Transmission type | Localized |
| Number of OFDM symbols | 2 |
| Number of PRBs | 16 |
| Allocated PRBs | {0, 1, … 15} |
| Rate matching mode | Mode 1 |
| SPDCCH L1 Reuse Indication | Not configured |
| Total number of SCCEs | 4 (16 SREGs) or 8 (32 SREGs) |

Figure 1	Simulation results of SPDCCH for wideband CQI tests.

## 2.2	CQI to MCS table

Table 2 shows the current simulation assumption for DMRS-based wideband CQI reporting test.

Table 2	Parameters for wideband CQI reporting test.

|  | Test 1 |
| --- | --- |
| Bandwidth | 10MHz |
| TTI length | Slot-based / Subslot-based |
| Number of PDCCH symbols | 2 |
| Cyclic prefix | Normal |
| Subframe configuration | Non-MBSFN |
| Propagation channel | EPA5 |
| Antenna configuration | High (2 x 2) |
| Transmission modes | TM9 |
| Beamforming model | TS 36.101 B.4.3 |
| CRS reference signals | Antenna port 0 |
| CSI reference signals | Antenna ports 15 and 16 |
| CSI-RS periodicity and subframe offset | 5ms / 1 |
| CSI-RS reference signal configuration | 0 |
| CodeBookSubsetRestrction bitmap | 000001 |
| Reporting interval | 5ms |
| CQI delay | n+4 slots for slot-based n+4 or n+6 subslots for subslot-based |
| Reporting mode | PUSCH 1-1 |
| PDSCH transmission bandwidth | 50 RB |
| Number of HARQ transmissions | 1 |
| Duplex mode | FDD / TDD |

Table 3 and Table 4 show our proposal on the CQI to MCS table used for the wideband CQI for slot-based transmission and subslot-based transmission, respectively. We calculate the available resource elements based on the assumption:

- SF#0 and SF#5 are DTX as same as the existing wideband CQI reporting test. In this case, we only need to consider the subframes without PSS/SSS/PBCH and SI transmission.

- Reduce the REs used for SPDCCH transmission (AL8 = 16 RBs with two OFDM symbols)

- CFI=2

- 1 CRS port

- 2 DMRS ports (CSI-RS configuration 0).

- 2 NZP-CSI-RS ports

Since we set 2 CSI-RS ports with CSI-RS configuration 0, CSI-RS is transmitted in slot 0 for the slot-based transmission and in subslot 2 for the subslot-based transmission.

Table 3	CQI-to-MCS table for DMRS-based Slot-PDSCH

|  |  |  | Non CSI-RS |  | 2 CRS-RS |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Slot 0 | Slot 1 | Slot 0 | Slot 1 |
|  |  | PRB | 50 | 50 | 50 | 50 |
|  |  | Available REs | 2600 | 3348 | 2500 (CSI-RS) | 3348 |
| CQI index | Code rate | Modulation |  |  |  |  |
| 1 | 0.08 | 2 | 0 | 0 | 0 | 0 |
| 2 | 0.12 | 2 | 0 | 0 | 0 | 0 |
| 3 | 0.19 | 2 | 1 | 2 | 1 | 2 |
| 4 | 0.3 | 2 | 3 | 4 | 3 | 4 |
| 5 | 0.44 | 2 | 5 | 7 | 5 | 7 |
| 6 | 0.59 | 2 | 7 | 9 | 7 | 9 |
| 7 | 0.37 | 4 | 10 | 12 | 10 | 12 |
| 8 | 0.48 | 4 | 12 | 14 | 12 | 14 |
| 9 | 0.6 | 4 | 14 | 16 | 13 | 16 |
| 10 | 0.46 | 6 | 17 | 19 | 17 | 19 |
| 11 | 0.55 | 6 | 18 | 21 | 18 | 21 |
| 12 | 0.65 | 6 | 20 | 23 | 20 | 23 |
| 13 | 0.75 | 6 | 22 | 26 | 22 | 26 |
| 14 | 0.85 | 6 | 24 | 27 | 23 | 27 |
| 15 | 0.93 | 6 | 25 | 28 | 24 | 28 |

Table 4	CQI-to-MCS table for DMRS-based Subslot-PDSCH

|  |  |  | Non CSI-RS |  |  |  |  | 2 CSI-RS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Subslot 1 | Subslot 2 | Subslot 3 | Subslot 4 | Subslot 5 | Subslot 1 | Subslot 2 | Subslot 3 | Subslot 4 | Subslot 5 |
|  |  | PRB | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  |  | Available REs | 1500 | 1000 | 612 | 1000 | 1500 | 1500 | 900 (CSI-RS) | 612 | 1000 | 1500 |
| CQI index | Code rate | Modulation |  |  |  |  |  |  |  |  |  |  |
| 1 | 0.08 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0.12 | 2 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| 3 | 0.19 | 2 | 4 | 2 | 0 | 2 | 4 | 4 | 1 | 0 | 2 | 4 |
| 4 | 0.3 | 2 | 7 | 4 | 2 | 4 | 7 | 7 | 3 | 2 | 4 | 7 |
| 5 | 0.44 | 2 | 9 | 6 | 3 | 6 | 9 | 9 | 5 | 3 | 6 | 9 |
| 6 | 0.59 | 2 | 9 | 8 | 5 | 8 | 9 | 9 | 7 | 5 | 8 | 9 |
| 7 | 0.37 | 4 | 14 | 11 | 10 | 11 | 14 | 14 | 10 | 10 | 11 | 14 |
| 8 | 0.48 | 4 | 16 | 13 | 10 | 13 | 16 | 16 | 12 | 10 | 13 | 16 |
| 9 | 0.6 | 4 | 16 | 15 | 11 | 15 | 16 | 16 | 14 | 11 | 15 | 16 |
| 10 | 0.46 | 6 | 22 | 18 | 17 | 18 | 22 | 22 | 17 | 17 | 18 | 22 |
| 11 | 0.55 | 6 | 25 | 20 | 17 | 20 | 25 | 25 | 19 | 17 | 20 | 25 |
| 12 | 0.65 | 6 | 28 | 22 | 17 | 22 | 28 | 28 | 20 | 17 | 22 | 28 |
| 13 | 0.75 | 6 | 28 | 24 | 18 | 24 | 28 | 28 | 22 | 18 | 24 | 28 |
| 14 | 0.85 | 6 | 28 | 26 | 19 | 26 | 28 | 28 | 24 | 19 | 26 | 28 |
| 15 | 0.93 | 6 | 28 | 27 | 20 | 27 | 28 | 28 | 25 | 20 | 27 | 28 |

# 3	Simulation results

According the existing wideband CQI reporting requirements, the test metrics consist of three factors:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to δ

Table 5 and Table 6 show our simulation results of wideband CQI reporting with slot-PDSCH and subslot-PDSCH, respectively, with regard to the metrics above. We list the test points around the median CQI index 4 (1/3 QPSK), 8 (1/2 16QAM), and 13 (3/4 64QAM). Figure 2 and Figure 3 show the median CQI and throughput ratio values for reference. As we discussed earlier, since the existing wideband CQI test case sets SNR above 0dB, we propose to set two sets of SNR test points {3, 4} and {12, 13}. The reason we propose the test points corresponding to median CQI 12 is to consider higher throughput ratio values. For the test metrics, our simulation results show we can reuse the existing metrics, i.e., α=20%, γ=1.05, and δ=0.02.

Proposal 2: Set two sets of test points {3, 4} and {12, 13} for both slot-based and subslot-based wideband CQI reporting test.

Proposal 3: Reuse the existing criteria, i.e., α=20%, γ=1.05, δ=0.02.

Table 5	Simulation results of wideband CQI with slot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| -4 | 4 | 38.9 | 0.15 | 0.97 |
| -3 | 4 | 43.1 | 0.12 | 1.16 |
| 3 | 8 | 40.6 | 0.09 | 1.40 |
| 4 | 8 | 40.9 | 0.09 | 1.31 |
| 12 | 12 | 36.2 | 0.09 | 1.40 |
| 13 | 13 | 36.3 | 0.08 | 1.56 |
| 14 | 13 | 32.8 | 0.07 | 1.39 |
| 15 | 13 | 37.0 | 0.07 | 1.28 |

Table 6	Simulation results of wideband CQI with subslot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| -4 | 4 | 36.4 | 0.34 | 1.25 |
| -3 | 4 | 36.8 | 0.31 | 1.30 |
| 4 | 8 | 37.7 | 0.18 | 1.27 |
| 5 | 8 | 37.2 | 0.17 | 1.24 |
| 12 | 12 | 37.7 | 0.16 | 1.31 |
| 13 | 12 | 36.2 | 0.16 | 1.22 |
| 14 | 12 | 38.7 | 0.17 | 1.15 |
| 15 | 13 | 32.8 | 0.17 | 1.23 |
| 16 | 13 | 34.2 | 0.19 | 1.13 |

Figure 2	Median CQI.

Figure 3	Throughput ratio between the follow CQI and Fixed median CQI.

# 4	Conclusion

Proposal 1: Configure AL8 for SPDCCH for DMRS-based CQI test.

Proposal 2: Set two sets of test points {3, 4} and {12, 13} for both slot-based and subslot-based wideband CQI reporting test.

Proposal 3: Reuse the existing criteria, i.e., α=20%, γ=1.05, δ=0.02.

# 5	References

R4-1808475, “Way forward on sTTI UE demodulation and CSI requirements”, Qualcomm, Ericsson, Huawei.

R4-1808810, “CQI reporting tests for CRS-based PDSCH”, Ericsson.

# Appendix

Table 7	TBS table for slot/subslot-PDSCH of 50PRB.

|  |  |  |  | Slot-PDSCH |  | Subslot-PDSCH |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IMCS | Qm | ITBS | TBS with 50PRB | TBSscaled | TBSslot | TBSscaled | TBSsubslot |
| 0 | 2 | 0 | 1384 | 692 | 696 | 230.7 | 224 |
| 1 | 2 | 1 | 1800 | 900 | 904 | 300 | 296 |
| 2 | 2 | 2 | 2216 | 1108 | 1096 | 369.3 | 376 |
| 3 | 2 | 3 | 2856 | 1428 | 1416 | 476 | 472 |
| 4 | 2 | 4 | 3624 | 1812 | 1800 | 604 | 600 |
| 5 | 2 | 5 | 4392 | 2196 | 2216 | 732 | 744 |
| 6 | 2 | 6 | 5160 | 2580 | 2600 | 860 | 872 |
| 7 | 2 | 7 | 6200 | 3100 | 3112 | 1033.3 | 1032 |
| 8 | 2 | 8 | 6968 | 3484 | 3496 | 1161.3 | 1160 |
| 9 | 2 | 9 | 7992 | 3996 | 4008 | 1332 | 1320 |
| 10 | 4 | 9 | 7992 | 3996 | 4008 | 1332 | 1320 |
| 11 | 4 | 10 | 8760 | 4380 | 4392 | 1460 | 1480 |
| 12 | 4 | 11 | 9912 | 4956 | 4968 | 1652 | 1672 |
| 13 | 4 | 12 | 11448 | 5724 | 5736 | 1908 | 1928 |
| 14 | 4 | 13 | 12960 | 6480 | 6456 | 2160 | 2152 |
| 15 | 4 | 14 | 14112 | 7056 | 6968 | 2352 | 2344 |
| 16 | 4 | 15 | 15264 | 7632 | 7736 | 2544 | 2536 |
| 17 | 6 | 15 | 15264 | 7632 | 7736 | 2544 | 2536 |
| 18 | 6 | 16 | 16416 | 8208 | 8248 | 2736 | 2728 |
| 19 | 6 | 17 | 18336 | 9168 | 9144 | 3056 | 3112 |
| 20 | 6 | 18 | 19848 | 9924 | 9912 | 3308 | 3368 |
| 21 | 6 | 19 | 21384 | 10692 | 10680 | 3564 | 3624 |
| 22 | 6 | 20 | 22920 | 11460 | 11448 | 3820 | 3880 |
| 23 | 6 | 21 | 25456 | 12728 | 12576 | 4242.7 | 4264 |
| 24 | 6 | 22 | 27376 | 13688 | 13536 | 4562.7 | 4584 |
| 25 | 6 | 23 | 28336 | 14168 | 14112 | 4722.7 | 4776 |
| 26 | 6 | 24 | 30576 | 15288 | 15264 | 5096 | 5160 |
| 27 | 6 | 25 | 31704 | 15852 | 15840 | 5284 | 5352 |
| 28 | 6 | 26 | 36696 | 18348 | 18336 | 6116 | 6200 |