# 3GPP TSG-RAN WG4 Meeting AH-1807 LTE/NR perf                                  R4-1808821

# Montreal, Canada, 2th – 6th July, 2018

Source: 	ZTE Corporation

Title: 	TP to TS 38.141-1: Test configurations for multi-carrier(Sections 4.8.3)

Agenda Item:	5.1.3

Document for:	Approval

1	Introduction

In the #86bis meeting,  the WF [1] for NR BS test configurations was agreed, where the following five NRTCs were agreed:

NRTC1: NR contiguous spectrum operation

NRTC2: Contiguous CA occupied bandwidth

NRTC3: NR non-contiguous spectrum operation

NRTC4: NR multi-band test configuration for full carrier allocation

NRTC5: NR multi-band test configuration with high PSD per carrier

Since the TS38.141-1 spec is mainly focus on the conductive test requirements, and the generation for NRTCs are use the similar principle as E-UTRA(or Rel-13 AAS), and also the test configuration for each requirements for multi-carrier and multi-band should be specified.

In this contribution, we provide a TP to TS38.141-1[2] on the test configurations (NRTC1~5) for each requirements multi-carrier in section 4.8.3 based on contribution of test configuration generated in[3]

2	Reference

[1] R4-1805872, WF on Test configuration for NR BS conformance testing, Huawei, Ericsson, Nokia

[2] 38.141-1 v0.3.0

[3] R4-1808820, TP to TS38.141-1: Test configuration(Sections 4.7) , ZTE Corporation

--------------Start of TP  -------------

4.8.3	Test configurations for multi-carrier

The applicable test configurations are specified in the tables below for each the supported RF configuration, which shall be declared according to subclause 4.6. The generation and power allocation for each test configuration is defined in subclause 4.7. This subclause contains the test configurations for single-band connector operating with multiple carriers (MC).

For a NR BS declared to be capable of single carrier operation only, a single carrier (SC) shall be used for testing.

For a single-band connector declared to support multi-carrier and/or CA operation in contiguous spectrum operation in single band only, the test configurations in the second column of table 4.8.3-1 shall be used for testing.

For a single-band connector declared to support multi-carrier and/or CA operation in contiguous and non-contiguous spectrum in single band and where the parameters in the manufacture's declaration according to subclause 4.6 are identical for contiguous (C) and non-contiguous (NC) spectrum operation, the test configurations in the third column of table 4.8.3-1 shall be used for testing.

For a single-band connector declared to support multi-carrier and/or CA in contiguous and non-contiguous spectrum in single band and where the parameters in the manufacture's declaration according to subclause 4.6 are not identical for contiguous and non-contiguous spectrum operation, the test configurations in the fourth column of table 4.8.3-1 shall be used for testing.

Table 4.8.3-1: Test configurations for single-band connector capable of both contiguous and non-contiguous spectrum in multi-carrier and/or CA operation in contiguous spectrum in single band only

| single-band connector test case | Contiguous spectrum capable single-band connector | C and NC capable single-band connector with identical parameters | C and NC capable single-band connector with different parameters |
| --- | --- | --- | --- |
| 6.2 Base station output power | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| 6.3 Output power dynamics |  |  |  |
| 6.3.1 RE Power control dynamic range | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| 6.3.2 Total power dynamic range | SC | SC | SC |
| 6.4 Transmit ON/OFF power | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| 6.5 Transmitted signal quality | - | - | - |
| 6.5.1 Frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| 6.5.2 Error Vector Magnitude | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| 6.5.3 Time alignment error | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| 6.6 Unwanted emissions | - | - | - |
| 6.6.2 Occupied bandwidth | SC, NRTC2 (Note) | SC, NRTC2 (Note) | SC, NRTC2 (Note) |
| 6.6.3 Adjacent Channel Leakage power Ratio (ACLR) | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| 6.6.3 Cumulative ACLR requirement in non-contiguous spectrum (CACLR) | - | NRTC3 | NRTC3 |
| 6.6.4 Operating band unwanted emissions | NRTC1 | NRTC1, NRTC3 | NRTC1, NRTC3 |
| 6.6.5 Transmitter spurious emissions | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| 6.7 Transmitter intermodulation | NRTC1 | Same TC as used in 6.6 | Same TC as used in 6.6 |
| 7.2 Reference sensitivity level | SC | SC | SC |
| 7.3 Dynamic range | SC | SC | SC |
| 7.4 In-band selectivity and blocking |  |  |  |
| 7.4.1 Adjacent Channel Selectivity(ACS) | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| 7.4.2 In-band Blocking | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| 7.4.2 Narrowband blocking | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| 7.5 Out-of-band Blocking | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| 7.6 Receiver spurious emissions | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| 7.7 Receiver intermodulation | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| 7.8 In-channel selectivity | SC | SC | SC |
| Note: NRTC2 is only applicable when contiguous CA is supported. |  |  |  |

--------------End of TP ------------