# 3GPP TSG-RAN WG4 Meeting AH-1807 LTE/NR perf                           R4-1808823

# Montreal, Canada, 2th – 6th July, 2018

Source: 	ZTE Corporation

Title: 	TP to TS 38.141-2: Applicability of requirements (Sections 4.7)

Agenda Item:	5.1.4.1

Document for:	Approval

1	Introduction

There is a common clause title named “Applicability of requirements” in TS38.141-1, TS38.141-2 and TS37.145-2  with different clause number, shown in table 1.

Table 1. Clause titled “Applicability of requirements” in TS38.141-1[1], TS38.141-1[2] and TS37.145-2 [3]

| TS38.141-1 | TS38.141-2 | TS37.145-2 |
| --- | --- | --- |
| 4.8 Applicability of requirements | 4.7 Applicability of requirements | 5 Applicability of requirements |
| 4.8.1 General | 4.7.1 General | 5.1 General |
| 4.8.2 Requirement set applicability | 4.7.2 Requirement set applicability |  |
| 4.8.3 Test configurations for multi-carrier | No this subclause | ## 5.3 Test configurations for multi-carrier capable AAS BS in operating bands where one RAT capability sets are supported |
| 4.8.4 Test configurations for multi-band | No this subclause | ## 5.4 Test configurations for AAS BS operating bands with multi-band dependencies |

In the TS38.141-1, TS38.141-2 and TS37.145-2, for the subclause titled “Requirement set applicability”, the requirement applicability for each requirement set is defined. For each requirement, the applicable requirement subclause in the specification is identified.

In TS38.141-1, there are two subclauses containing the applicability table the test configuration for each conduct requirement for multi-carrier (contiguous and non-contiguous cases) and multi-band, respectively. The information in the tables defined in these two subclauses are very important for the NR BS conformance test and it can be seen as the guild line to select the right test configuration for each requirements to guarantee the requirements are tested in the right way.

Also, in TS37.145-2, there are also some tables defined under clauses 5 containing the applicability table the test configuration for each OTA requirement, similar with TS38.141-1.

However, in TS38.141-2, the related subclauses are missing, which means there may no place to capture such important information in the specification, as TS38.141-1 and TS37.145-2 did.

In general, the conformance test depend on NR BS capabilities such as multi-carrier operation and multi-band operation, or NB-IoT capability if possible, which means the test configuration selected for a certain BS RF requirement may be different.

Therefore the information of the test configuration for each requirement should be defined explicit in the TS to avoid confusion or to avoid the duplicate test.  It is proposed:

Proposal: Add two new subclauses titled “Test configurations for multi-carrier ” and “Test configurations for multi-band ” in section 4.7 in TS38.141-2.

Therefore, this contribution provide a TP to TS38.141-2[4] to add two new subclauses, i.e. 4.7.3 and 4.7.4, titled “Test configurations for multi-carrier ” and “Test configurations for multi-band ” in section 4.7 to capture the the test configuration for each requirement for multi-carrier (contiguous and non-contiguous cases) and multi-band, respectively.

2	Reference

[1] 38.141-1 v0.3.0

[2] 38.141-2 v0.2.0

[3] TS37.145-2

--------------Start of TP  -------------

4.7	Applicability of requirements

Editor’s note: NR BS aspects related to the TS 37.145-2 radiated MSR conformance will be addressed after single RAT specification aspects in the present specification will be settled.

4.7.1	General

### 4.7.2	Requirement set applicability

In table 4.7.2-1, the requirement applicability for each requirement set is defined. For each requirement, the applicable requirement subclause in the specification is identified. Requirements not included in a requirement set is marked not applicable (NA).

Table 4.7.2-1: Requirement set applicability

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O |
| Radiated transmit power | 6.2 | 6.2 | 6.2 |
| OTA base station output power | NA | 6.3 | 6.3 |
| OTA output power dynamics |  | 6.4 | 6.4 |
| OTA transmit ON/OFF power |  | 6.5 | 6.5 |
| OTA transmitted signal quality |  | 6.6 | 6.6 |
| OTA occupied bandwidth |  | 6.7.2 | 6.7.2 |
| OTA ACLR |  | 6.7.3 | 6.7.3 |
| OTA out-of-band emission |  | 6.7.4 | 6.7.4 |
| OTA transmitter spurious emission |  | 6.7.5 | 6.7.5 |
| OTA transmitter intermodulation |  | 6.8 | NA |
| OTA sensitivity | 7.2 | 7.2 | NA |
| OTA reference sensitivity level | NA | 7.3 | 7.3 |
| OTA dynamic range |  | 7.4 | NA |
| OTA in-band selectivity and blocking |  | 7.5 | 7.5 |
| OTA out-of-band blocking |  | 7.6 | 7.6 |
| OTA receiver spurious emission |  | 7.7 | 7.7 |
| OTA receiver intermodulation |  | 7.8 | 7.8 |
| OTA in-channel selectivity |  | 7.9 | 7.9 |
| Radiated performance requirements |  | 8 | 8 |

4.7.3	Test configurations for multi-carrier

Editor’s note: this section contains the applicability table for the text configuration for each requirement for multi-carrier (contiguous and non-contiguous cases).

4.7.4	Test configurations for multi-band

Editor’s note: this section contains the applicability table for the text configurations for each requirement for multi-band

--------------End of TP ------------