3GPP TSG-RAN WG4 Meeting #AH1807	R4-1808839

# Montreal, Canada, 2 – 6 July, 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on introduction of interruption requirement for BWP switching in TS38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | BWP switching on one serving cell will cause interruption in other serving cells in EN-DC or NR CA, but currently the corresponding interruption requirements are missung |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce interruption requirement for BWP switching |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interruption requirement for BWP switching is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1, new 8.2.1.2.7, 8.2.2.1, new 8.2.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

8.2.1.1	Introduction

This section contains the requirements related to the interruptions on PSCell, and SCell, when

E-UTRA PCell transitions between active and non-active during DRX, or

E-UTRA PCell transitions from non-DRX to DRX, or

E-UTRA SCell in MCG or SCell in SCG is added or released, or

E-UTRA SCell in MCG or SCell in SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA MCG or NR SCG, or

a downlink bandwidth part (BWP) and/or an uplink BWP is switching in PSCell or any SCell

The requirements shall apply for E-UTRA-NR DC with an E-UTRA PCell.

This section contains interruptions where victim cell is PSCell or SCell belonging to SCG. Requirements for interruptions requirements when the victim cell is E-UTRA PCell or E-UTRA SCell belonging to MCG are specified in [TS 36.133].

----------------------NEXT CHANGE----------------------------

8.2.1.2.7	Interruptions at BWP switch

The requirements in this section applies for a UE configured with more than one BWP on any of PSCell and activated SCells in SCG.

When BWP switch involving bandwidth, center frequency or SCS changing occurs on a downlink BWP and/or uplink BWP in PSCell or in any SCell, an interruption on the serving cell on which BWP switch occurs shall not exceed the BWP switch delay defined in section 8.5.

When BWP switch involving bandwidth, center frequency or SCS changing occurs on a downlink BWP and/or uplink BWP in PSCell or in any SCell, an interruption on the serving cell on which BWP switch does not occur shall not exceed K number of slots in Table 8.2.1.2.7-1.

Table 8.2.1.2.7-1 Interruption lengths K in slots due to BWP switch

|  | NR Slot length (ms) | Interruption length K (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |

----------------------NEXT CHANGE----------------------------

8.2.2.1	Introduction

This section contains the requirements related to the interruptions on PCell and activated SCell if configured, when up to TBD SCells are configured, deconfigured, activated or deactivated, or when a downlink bandwidth part (BWP) and/or an uplink BWP is switching in PCell or any SCell.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

Editor’s Note:	The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command. How to specify this is FFS.

----------------------NEXT CHANGE----------------------------

8.2.2.2.5	Interruptions at BWP switch

The requirements in this section applies for a UE configured with more than one BWP on any of PCell and activated SCells.

When BWP switch involving bandwidth, center frequency or SCS changing occurs on a downlink BWP and/or uplink BWP in PCell or in any SCell, an interruption on the serving cell on which BWP switch occurs shall not exceed the BWP switch delay defined in section 8.5.

When BWP switch involving bandwidth, center frequency or SCS changing occurs on a downlink BWP and/or uplink BWP in PCell or in any SCell, an interruption on the serving cell on which BWP switch does not occur shall not exceed L number of slots in Table 8.2.2.2.5-1.

Table 8.2.2.2.5-1 Interruption lengths L in slots due to BWP switch

|  | NR Slot length (ms) | Interruption length L (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |

----------------------END OF CHANGES----------------------------