3GPP TSG-RAN WG4 AH-1807                                         R4-1808912

Montreal, Canada, July 2nd -6th, 2018

Source: 	Samsung

Title: 	Email discussion summary for NR UE simulation assumption and simulation cases

Agenda Item:	5.4.1

Document for:	Information

Introduction

In order to facilitate simulation alignment of NR UE performance between companies for July ad-hoc, open issues for simulation assumptions and candidate initial simulation cases for July Ad-hoc alignment purpose were discussed in email between June 6 and June 18. Email discussion output was summarized in this contribution.

Summary of email discussion

Companies were discussed below open issues, and recommendations reached for July Ad-hoc alignment purpose.

Issue 1: PBCH overhead & configuration:

A default configuration proposed: Periodicity: 20ms with slot =0, L =1; Frequency location: In the centre of BW; SS Block pattern: Case A for 15kHz, Case C for 30kHz, case D for 120kHz, case E for 240kHz SCS.

For PDSCH scheduling in PBCH slot:

- Skip scheduling PDSCH for PBCH slot i.e. slot 0 per 20ms (Intel, Huawei)

Recommendation: Skip scheduling PDSCH for PBCH slot i.e. slot 0 per 20ms

Issue 2: FRC with MCS selection

Recommendation: Below MCS proposed

- QPSK, MCS4 (Table 1), CR 0.3008

- 16QAM, MCS13 (Table 1), CR 0.4785

- 64QAM, MCS24 (Table 1), CR 0.7539

- 256QAM, MCS24 (Table 2),CR 0.8213

Issue 3: Number of HARQ process

Companies’ preference summarized in below, no consensus can be reached during email discussion.

- FDD:

  - 4 (Qualcomm, Intel, Samsung)

  - 8 (NTT DoCoMo)

  - 10 (Huawei)

- TDD:

  - 8 HARQ processes for DDDSU and 7D1S2U and 4 HARQ processes for DSSU (Intel, Qualcomm)

  - 16 for initial simulation alignment purpose (Ericsson)

  - 10 HARQ processes for slot format DDDSU and 16 HARQ processes for slot format 7D1S2U (Huawei)

  - 16 for TDD (at least for 7D1S2U in FR1 and DDDSU in FR2) (NTT DoCoMo)

Recommendation: companies bring results for TDD based on their preferred HARQ process numbers (at least no less than 8) and indicated in contributions.

Issue 4: FDM of DMRS and data: (layer 1/layer 2 test)

- Both Layer 1 and layer 2 (Intel, Huawei?)

- Only Layer 2(NTT DoCoMo)

- Only Layer 1 (Qualcomm)

Recommendation: both FDM and non-FDM can be considered based on companies' preference

Issue 5: Channel model assumption of FR2 for July ad-hoc simulation purpose

- Option 1:  TDL-A 30ns with 10Hz  (Ericsson, Intel)

- Option 2: CDL-A 30ns with 900Hz (Qualcomm)

Recommendation: companies can bring results based their preferred channel model and indicated detailed assumption in contributions, two candidate options recommended by companies.

List of initial simulation cases for July Ad-hoc

Initial simulation cases for July Ad-hoc alignment purpose were recommended and list as below for FR1 FDD, TDD and FR2.

Note: only for initial alignment purpose in July Ad-hoc, not exclude any other test cases with other configurations for introducing performance requirements.

Table 1 List of simulation cases for FR1 FDD with 10 MHz + 15 kHz

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Channel Model |
| --- | --- | --- | --- | --- | --- |
| 1.1a,1.1b | 10MHz/15kHz | 2Tx 2Rx Low 2Tx 4Rx Low | QPSK MCS 4 | Layer 1 | TDL-C 300ns, 100Hz |
| 1.2a,1.2b | 10MHz/15kHz | 2Tx 2Rx Low 2Tx 4Rx Low | 256QAM MCS 24 | Layer 1 | TDL-A 30ns, 10Hz |
| 1.3a,1.3b | 10MHz/15kHz | 2Tx 2Rx Low 2Tx 4Rx Low | 16QAM MCS 13 | Layer 2 | TDL-A 30ns, 10Hz |
| 1.4a,1.4b | 10MHz/15kHz | 2Tx 2Rx Low 2Tx 4Rx Low | 64QAM MCS 24 | Layer 2 | TDL-A 30ns, 10Hz |
| 1.5a,1.5b | 10MHz/15kHz | 4Tx 2Rx Low 4Tx 4Rx Low | 16QAM MCS 13 | Layer 2 | TDL-A 30ns, 10Hz |
| 1.6 | 10MHz/15kHz | 4Tx 4Rx Low | 16QAM MCS 13 | Layer 4 | TDL-A 30ns, 10Hz |

Table 2 List of simulation cases for FR1 TDD covering different combinations of MIMO Tx Rx, MCS and layers

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Channel Model |
| --- | --- | --- | --- | --- | --- |
| 2.1a,2.1b | 40MHz/30kHz | 2Tx 2Rx Low 2Tx 4Rx Low | QPSK MCS 4 | Layer 1 | TDL-C 300ns,100Hz |
| 2.2a,2.2b | 40MHz/30kHz | 2Tx 2Rx Low 2Tx 4Rx Low | 256QAM MCS 24 | Layer 1 | TDL-A 30ns, 10Hz |
| 2.3a,2.3b | 40MHz/30kHz | 2Tx 2Rx Low 2Tx 4Rx Low | 16QAM MCS 13 | Layer 2 | TDL-A 30ns, 10Hz |
| 2.4a,2.4b | 40MHz/30kHz | 2Tx 2Rx Low 2Tx 4Rx Low | 64QAM MCS 24 | Layer 2 | TDL-A 30ns, 10Hz |
| 2.5a,2.5b | 40MHz/30kHz | 4Tx 2Rx Low 4Tx 4Rx Low | 16QAM MCS 13 | Layer 2 | TDL-A 30ns, 10Hz |
| 2.6 | 40MHz/30kHz | 4Tx 4Rx Low | 16QAM MCS 13 | Layer 4 | TDL-A 30ns, 10Hz |
| Note: TDD with 40MHz/30kHz and DL-UL configuration as DDDSU (default),S = 12D+1Gp+1U |  |  |  |  |  |

Table 3 List of simulation cases for FR1 TDD covering different CHBW and SCS, and different DL-UL configurations

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Channel Model |
| --- | --- | --- | --- | --- | --- |
| 3.1a,3.1b | 10MHz/15kHz | 2Tx 2Rx Low 2Tx 4Rx Low | QPSK MCS 4 | Layer 1 | TDL-C 300ns, 100Hz |
| 3.2a,3.2b | 20MHz/30kHz | 2Tx 2Rx Low 2Tx 4Rx Low | QPSK MCS 4 | Layer 1 | TDL-C 300ns, 100Hz |
| 3.3a,3.3b | 40MHz/30kHz with DL-UL 7D1S2U | 2Tx 2Rx Low 2Tx 4Rx Low | QPSK MCS 4 | Layer 1 | TDL-C 300ns, 100Hz |
| 3.4a,3.4b | 40MHz/30kHz With DL-UL DSSU | 2Tx 2Rx Low 2Tx 4Rx Low | QPSK MCS 4 | Layer 1 | TDL-C 300ns, 100Hz |

Table 4 List of simulation cases for FR2 TDD covering different MIMO Tx Rx, MCS and layers, CHBW and SCS, and different DL-UL configurations

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Channel Model |
| --- | --- | --- | --- | --- | --- |
| 4.1 | 100MHz/120kHz | 1Tx 2Rx Low | QPSK MCS 4 | Layer 1 | TBD |
| 4.2 | 100MHz/120kHz | 1Tx 2Rx Low | 64QAM MCS 24 | Layer 1 | TBD |
| 4.3 | 100MHz/120kHz | 2Tx 2Rx Low | 16QAM MCS 13 | Layer 2 | TBD |
| 4.4 | 100MHz/60kHz | 1Tx 2Rx Low | QPSK MCS 4 | Layer 1 | TBD |
| 4.5 | 200MHz/120kHz | 1Tx 2Rx Low | QPSK MCS 4 | Layer 1 | TBD |
| 4.6 | 100MHz/120kHz With DSDSU, S =10D+2Gp+2U | 1Tx 2Rx Low | QPSK MCS 4 | Layer 1 | TBD |
| Note 1:Except case 20 in table, all other cases as DDDSU |  |  |  |  |  |