3GPP TSG-RAN WG4 Meeting #AH1807                     	R4-1808917

Montreal, Canada, July 2nd –6th, 2018

Agenda item:            5.2.4.4

Source:	MediaTek Inc.

Title:	Discussion on requirements for beam management

Document for:          Discussion

- Introduction

In the previous meeting, the beam management related requirement has been discussed, including beam failure detection (BFD), candidate beam detection (CBD), and beam reporting (BR). The corresponding agreements are listed in the way forward [1].

In this paper, our views on PDCCH parameters for BFD and L1-RSRP measurement are discussed in Section 2 and Section 3. And PDCCH link level simulation results is provided in Section 4, and the one-shot measurement accuracy of L1-RSRP is provided in Section 5.

- SINR Level of BFD and L1-RSRP side condition

It agreed that the hypothetic PDCCH parameters for BFD will reuse the parameters for RLM OOS without power boosting, and it also agreed that the L1 evaluation period for beam failure instance evaluation will reuse the requirements for RLM in-sync evaluation.

From the PDCCH link level simulation results in Section 4, the required SINR with PDCCH parameters of RLM OOS without 4 dB power boost is given as:

- -7.33 dB for static channel

- -5.43 dB for ETU42 km/hr channel

which is lower than the required SINR with PDCCH parameters of RLM INS and it also means the BFD SINR estimation is less accurate than that of RLM INS, while the same evaluation period is re-used.

Observation 1: SINR estimation of BFD is less accurate than the SINR estimation of RLM INS, if we reuse the PDCCH parameters of RLM OOS without 4 dB power boost.

Based on the simulation result, the SINR levels of scenarios are illustration in Figure 1. From Figure 1, it can observe that, if the L1-RSRP side condition for CBD is too close to Qout_LR, it may happen that the suggested candidate beam could be still triggering another beam failure event, and it may cause non-preferred ping-pong effect between candidate beam suggestion and beam failure triggering.

Observation 2: If the gap between L1-RSRP side condition for CBD and Qout_LR is insufficient, the suggested candidate beam could be still triggering BFD.

In order to reduce the probability of ping-pong effect, it proposes:

Proposal 1: Sufficient gap between L1-RSRP side condition for CBD and BFD should be provided.

Figure 1: Illustration on SINR levels

Since the required SINR for BFD under ETU42 km/hr channel is around -5.5 dB, it proposes that the L1-RSRP side condition for CBD is [-2dB to 0dB], in order to provide sufficient gap between L1-RSRP side condition for CBD and required SINR of BFD, by considering the BFD SINR estimation error and L1-RSRP measurement error.

Proposal 2: L1-RSRP side condition for CBD is [-2dB to 0dB].

- Measurement Accuracy for L1-RSRP

One-shot measurement accuracy is shown in Section 6. Once the side condition of L1-RSRP has been determined, the measurement accuracy can be decided accordingly. One open issue could be whether L1-RSRP for CBD and L1-RSRP for beam reporting share the same requirement.

For beam reporting, the RS can be aperiodic, while only the periodic RS can be applied to CBD. Since aperiodic  RS may rely on one-shot measurement and one-shot measurement is the most challenging case, so we think requirement for L1-RSRP could be based on one-shot measurement.

Besides, for specification simplicity and progress, it could be beneficial to have the same requirement for L1-RSRP for beam reporting and candidate beam detection.

Proposal 3: L1-RSRP for beam reporting and L1-RSRP for candidate beam detection share the same accuracy requirement and side condition.

Proposal 4: Measurement accuracy of L1-RSRP is based on one-shot measurement.

- RX beam sweeping aspect for BFD evaluation period and L1-RSRP measurement period in FR2

It has been agreed the conditions on Rx beam sweeping operation for RLM are reused for BFD, so the following text is proposed to capture the aspect.

Proposal 5: Text proposal for Rx beam sweeping operation for BFD, i.e. no RX beam scaling if

- the SSB configured for BFD is spatially QCLed and TDMed to CSI-RS resources configured for BM, and the QCL association is known to UE;

- if the CSI-RS resource configured for BFD is spatially QCLed and TDMed to CSI-RS resources configured for BM or SSBs configured for BM, and the QCL association is known to UE;

UE RX beams sweeping should be always allowed for L1-RSRP measurement. Otherwise, UE can not determine its RX beam not only for L1-RSRP but also for other tasks relying the RX beam information from beam management, e.g. RLM, BFD.

Proposal 6: RX beam scaling for L1-RSRP measurement period is always needed.

- Simulation result of PDSCH parameters for BFD with 10% BLER

The PDCCH link level simulation results has been provided in the previous meeting [2] and the results of 10% PDCCH BLER are listed for reference.

Table 1: Required SNR for 10% PDCCH BLER

| Required SINR to achieve 10% PDCCH BLER (unit: dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Static Channel |  |  |  |  |  |  |
| DCI format | DCI format 1-0 |  |  | DCI format 1-1 |  |  |
| Payload size inc. CRC | 56 | 58 | 60 | 90 | 92 | 94 |
| AL = 4 | -5.24 | -5.12 | -4.96 | -3.43 | -3.36 | -3.28 |
| AL = 8 | -7.44 | -7.33 | -7.23 | -5.85 | -5.78 | -5.67 |
| AL = 16 | -9.50 | -9.40 | -9.30 | -8.10 | -8.01 | -7.93 |
| EPA 3Km/hr |  |  |  |  |  |  |
| DCI format | DCI format 1-0 |  |  | DCI format 1-1 |  |  |
| Payload size inc. CRC | 56 | 58 | 60 | 90 | 92 | 94 |
| AL = 4 | -2.73 | -2.60 | -2.35 | -0.65 | -0.55 | -0.45 |
| AL = 8 | -5.40 | -5.28 | -5.08 | -3.60 | -3.49 | -3.42 |
| AL = 16 | -7.69 | -7.55 | -7.41 | -6.06 | -5.99 | -5.89 |
| EPA 42Km/hr |  |  |  |  |  |  |
| DCI format | DCI format 1-0 |  |  | DCI format 1-1 |  |  |
| Payload size inc. CRC | 56 | 58 | 60 | 90 | 92 | 94 |
| AL = 4 | -2.96 | -2.79 | -2.52 | -0.81 | -0.75 | -0.63 |
| AL = 8 | -5.51 | -5.38 | -5.22 | -3.72 | -3.63 | -3.55 |
| AL = 16 | -7.76 | -7.63 | -7.50 | -6.14 | -6.06 | -5.99 |
| ETU 3Km/hr |  |  |  |  |  |  |
| DCI format | DCI format 1-0 |  |  | DCI format 1-1 |  |  |
| Payload size inc. CRC | 56 | 58 | 60 | 90 | 92 | 94 |
| AL = 4 | -2.79 | -2.62 | -2.40 | -0.64 | -0.57 | -0.45 |
| AL = 8 | -5.45 | -5.33 | -5.18 | -3.67 | -3.58 | -3.48 |
| AL = 16 | -7.96 | -7.84 | -7.72 | -6.36 | -6.28 | -6.20 |
| ETU 42Km/hr |  |  |  |  |  |  |
| DCI format | DCI format 1-0 |  |  | DCI format 1-1 |  |  |
| Payload size inc. CRC | 56 | 58 | 60 | 90 | 92 | 94 |
| AL = 4 | -2.78 | -2.60 | -2.38 | -0.59 | -0.50 | -0.43 |
| AL = 8 | -5.56 | -5.43 | -5.31 | -3.78 | -3.69 | -3.62 |
| AL = 16 | -7.96 | -7.83 | -7.69 | -6.32 | -6.27 | -6.19 |

- Simulation result of measurement accuracy of L1-RSRP

The one shot measurement accuracy of SSB and CSI-RS with D=3 are listed in this session for reference. Note it is not capturing the RF impairment.

Table 2: SSB, 90% RSRP accuracy (dB)

| SNR | STATIC | EPA5 | ETU70 |
| --- | --- | --- | --- |
| 0 | 0.81 | 1.01 | 2.38 |
| -2 | 1.00 | 1.24 | 2.45 |
| -4 | 1.23 | 1.52 | 2.63 |
| -6 | 1.50 | 1.91 | 2.86 |

Table 3: CSI-RS (D=3), 90% RSRP accuracy (dB)

| SNR | STATIC | EPA5 | ETU70 |
| --- | --- | --- | --- |
| 0 | 1.12 | 1.66 | 2.65 |
| -2 | 1.41 | 1.88 | 2.82 |
| -4 | 1.81 | 2.27 | 3.11 |
| -6 | 2.31 | 2.76 | 3.34 |

- Conclusion

In the contribution, we discuss the beam management related requirement, including BFD and L1-RSRP. We have the following observations and proposals:

Observation 1: SINR estimation of BFD is less accurate than the SINR estimation of RLM INS, if we reuse the PDCCH parameters of RLM OOS without 4 dB power boost.

Observation 2: If the gap between L1-RSRP side condition for CBD and Qout_LR is insufficient, the suggested candidate beam could be still triggering BFD.

Proposal 1: Sufficient gap between L1-RSRP side condition for CBD and BFD should be provided.

Proposal 2: L1-RSRP side condition for CBD is [-2dB to 0dB]

Proposal 3: L1-RSRP for beam reporting and L1-RSRP for candidate beam detection share the same accuracy requirement and side condition.

Proposal 4: Measurement accuracy of L1-RSRP is based on one-shot measurement.

Proposal 5: Text proposal for Rx beam sweeping operation for BFD, i.e. no RX beam scaling if

- the SSB configured for BFD is spatially QCLed and TDMed to CSI-RS resources configured for BM, and the QCL association is known to UE;

- if the CSI-RS resource configured for BFD is spatially QCLed and TDMed to CSI-RS resources configured for BM or SSBs configured for BM, and the QCL association is known to UE;

Proposal 6: RX beam scaling for L1-RSRP measurement period is always needed.

- Reference

[1] R4-1808444, “Way forward on Beam management requirements in NR”, Huawei.

[2] R4-1803679, “PDCCH Link Level Simulation Results”, MediaTek.