TSG-RAN Working Group 4 (Radio) meeting AH-1807	R4-1809095

# Montreal, Canada, 2nd July – 6th July 2018

Source:	Ericsson

Title:	TP to TR 37.843: Sparse sampling for spurious emissions

Agenda item:	4.2.1.1.5.1

Document for:	Approval

# Introduction

The currently proposed TRP methods in TR 37.843 clause 10.9 are based on spherical or orthogonal cuts sampling. When testing Spurious Emissions, an error-free bullet-proof TRP estimation method will most probably result in extremely long measurement time. However, when the radiation pattern emerges from uncorrelated sources the needed angular sampling can be relaxed.

In this contribution we propose a methodology to quantify the integration error for TRP calculation, resulting from sparse sampling from uncorrelated or partial correlated sources. The sources of the EIRP patterns of the analysis are generated by selecting a random correlation chosen from a correlation interval and using random rotations. The statistical source distributions are integrated on sparse grids and compared to numerically converged values on dense grids. The final result is a systematic correction factor that can be added to the integrated TRP value (the raw grid value) to ensure an over-estimated TRP value by 95% confidence.

# Discussion

In the case of spurious emissions for electrically large Equipments Under Test (EUTs)  the following observations are made:

Observation 1: The emissions do not necessarily radiate through the antenna elements. To be on the conservative side, the entire EUT dimensions should be considered when calculating the reference angular steps (in case of considering the equal angle grid).

Observation 2: The angular variation of the emissions is dominated by the array factor, i.e., the spatial distribution of the sources. Hence, emissions can be modelled by the array factor of a set of point sources, i.e., for simplicity the element pattern is isotropic.

Observation 3: The emission lobes will be narrow and hard to locate for some emissions, especially in the higher part of the spurious emissions domain. By studying random rotations of the sources of emissions, the need to align the measurement grid to the emission pattern can be removed.

Observation 4: In practice, the emissions are likely to have a low degree of correlation, except for harmonic emissions. For harmonic emissions a beam sweeping test signal is proposed FFS. For non-harmonic emissions, a correlation distributed uniformly in a given interval from 0 to  is used.

To quantify the degree of sparse sampling relative to the reference angular steps a Sparsity Factor (SF) is introduced. For a spherical equal angles grid

Where

Note: For other spherical grids the Sparsity Factor needs to be defined and statistically evaluated.

The method to estimate ΔTRP for large antennas (D>4λ) is based on generating a set of statistical samples of TRP values calculated on a set of randomly generated emission sources of a given electrical size

The final TRP values are then calculated by using a given angular grid and Eq. (3). The CDF of these TRP values are then created and the CDF is shifted so that the 5th percentile corresponds to 0 dB TRP error. This shift is used as ΔTRP, see Figure 1.

In this investigation each statistical sample is generated as follows:

- Set up a quadratic uniform linear NxN array in the yz-plane, with horizontal and vertical element separation

Reduce the number of elements N to make sure that the element separation is larger or equal to a half wave length.

- Rotate the source points by an Euler zyz-rotation [3] with angles a, b, and c which are uniformly chosen on the intervals [0,360], [0,90] and [0,360] degrees, respectively. The rotated position of source n is denoted

- Pick a random correlation value ρ from the uniform distribution between 0 and , and generate source weights as

Where  are picked from a normal distribution with zero mean and unit standard deviation.

- Normalize the weights to TRP=1 by using a full sphere grid with sparsity factor 0.25.

- Generate EIRP values on the desired grid by using an array factor

- Calculate the TRP value as an appropriate mean value of the EIRP values for two/three cut grids and using full sphere numerical integration for the full sphere case.

A publicly available Matlab code [1] is available. https://se.mathworks.com/matlabcentral/fileexchange/67143-sparse-sampling-analysis-tool?s_tid=srchtitle and more results can be found in Ref [2].

The resulting 5th percentile values of the empirical CDFs are used as a practical lower bound on the TRP calculated directly from the points in the actual grid. If the 5th percentile value is positive, there is no need for a systematic correction, that means ΔTRP=0. On the other hand, if the 5th percentile value is negative, a systematic correction is needed to guarantee TRP overestimation with 95% confidence. For this purpose, the absolute value of the 5th percentile is used as systematic correction factor, denoted ΔTRP (see Fig. 4). Some simulation results are found in Figures 2 (a) – (c) for two different choices of max correlation and the three grid types: full-sphere, three-cut, and two-cuts.

Figure 1: Determination of ΔTRP based on the 5th percentile values. The dashed curve depicts the empirical CDF found from the statistical analysis, and the solid curve depicts the corrected CDF corresponding to over-estimation with 95% confidence.

Figure 2: Correction factor  for 95% confidence overestimation of the TRP for three different electrical sizes  and correlation intervals.

Figure 3: Full sphere correction factors  for max correlation up to 0.75and D/λ=10.

Figure 4: Three cuts correction factors  for max correlation up 0.75and D/λ=10.

Figure 5: Two cuts correction factors  for max correlation up to 0.75 and D/λ=10.

The above descriptions and results are included in the draft CR for TR 37.843 in R4-1809096