3GPP TSG-RAN WG4 Meeting AH-1807                	R4-1809170

Montreal, Canada, 2 – 6 July 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for SPUSCH

Agenda item:	4.1.1.1

Document for:	Discussion

Introduction

In this contribution we introduce our simulation results for SPUSCH using the parameters as agreed in [1].

Simulation assumptions

Simulation assumptions as agreed for SPUSCH in [1] are shown below.

Table 1: Simulation assumptions for SPUSCH

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2, 4 |
| Propagation condition | EPA5 |
| Cyclic prefix | Normal |
| Reference receiver | MRC |
| Channel bandwidth | [5 MHz], 10 MHz, 15 MHz, 20 MHz |
| TTI length | Subslot TTI |
| Number of OFDM-symbols | Subslot TTI: 2 and 3 |
| Fraction of maximum throughput | 70 % |
| Maximum number of HARQ transmissions | 4 |
| DMRS configuration | IFDMA is not configured |
| DMRS sharing pattern | RDD DD DD RD DD RDD (agreed in RAN4#87) |
| Number of HARQ processes | 16 |
| RV sequence | 0, 2, 3, 1, 0, 2, 3, 1 |
| Modulation | 16QAM |

Additionally, the following FRCs were tentatively agreed in the RAN4#86bis meeting:

# Table A.4-1 FRC parameters for performance requirements (16QAM 3/4)

| Reference channel | Ax-1 |  |  | Ax-2 |  | Ax-3 |  | Ax-4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 24 |  |  | 48 |  | 72 |  | 100 |  |
| DFT-OFDM Symbols per subframe | 1 | 2 |  | 1 | 2 | 1 | 2 | 1 | 2 |
| Modulation | 16QAM |  |  | 16QAM |  | 16QAM |  | 16QAM |  |
| Code rate | 3/4 |  |  | 3/4 |  | 3/4 |  | 3/4 |  |
| Payload size (bits) | 872 |  | 1736 | 1736 | 3496 | 2536 | 5160 | 3624 | 7224 |
| Transport block CRC (bits) | 24 |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Total number of bits per sub-frame | 1152 |  | 2304 | 2304 | 4608 | 3456 | 6912 | 4800 | 9600 |
| Total symbols per sub-frame | 288 |  | 576 | 576 | 1152 | 864 | 1728 | 1200 | 2400 |

Simulation results

Simulation results for SPUSCH using the parameters from Table 1 and FRCs as in Table A.4-1 are shown below in Table 2. Results are slightly updated from our last meeting results.

Table 2: Initial simulation results for SPUSCH.

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix | FRC | Fraction of maximum throughput | SNR, dB |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | EPA 5Hz Low | Ax-1 | 70% | 10.28 |
|  |  |  |  | Ax-2 | 70% | 10.35 |
|  |  |  |  | Ax-3 | 70% | 10.49 |
|  |  |  |  | Ax-4 | 70% | 10.77 |
|  | 4 | Normal | EPA 5Hz Low | Ax-1 | 70% | 6.73 |
|  |  |  |  | Ax-2 | 70% | 6.65 |
|  |  |  |  | Ax-3 | 70% | 6.66 |
|  |  |  |  | Ax-4 | 70% | 6.98 |

Conclusion

In this contribution we have presented simulation results for SPUSCH.

- References

- R4-1805495, Simulation assumptions for sPUCCH and sPUSCH, Ericsson, 3GPP TSG-RAN WG4 #86bis, Melbourne, April 2018.