3GPP TSG-RAN WG4 Meeting AH-1807                	R4-1809180

Montreal, Canada, 2 – 6 July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on BWP switching delay requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2017-06-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requirements for BWP switching delay need to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section 8.x is added to define BWP switching delay requirements for DCI- and timer-based BWP switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 8.x BWP Switching Delay

8.x.1 Introduction

This section defines requirements for the delay within which the UE shall be able to complete BWP switching on PCell or any activated SCell in standalone NR, and PSCell or any activated SCell in SCG in EN-DC.

The requirements in this section apply for a UE supporting EN-DC and configured with at least PSCell, and for a UE supporting standalone NR and configured with at least PCell.

8.x.2 BWP Switching Delay Requirement for DCI-Based BWP Switching

When uplink and/or downlink BWP is switched on PCell, PSCell or any of the activated SCells by DCI-based BWP switching, the UE shall be capable to complete the BWP switching upon the specified conditions.

For DCI-based BWP switching, after receiving BWP switching command in slot n, the UE shall be capable to receive and transmit on the new BWP no later than in slot n + TBWP_switching_DCI, where:

TBWP_switching_DCI is up to:

- A1 slots for Type 1 capable UE and B1 slots for Type 2 capable UE, if BWP switching involves changing the center frequency and/or bandwidth of the BWP, with or without changing the SCS or other baseband parameters,

- A2 slots for Type 1 capable UE and B2 slots for Type 2 capable UE, if BWP switching involves changing only the SCS or other baseband parameters.

Table 8.x.2-1 BWP switching delay A1, A2, B3 and B4 for DCI-based BWP switching

|  | NR Slot length (ms) | Interruption length (slots) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | A1 | A2 | B1 | B2 |
| 0 | 1 | 1 | 1 | 3 | 2 |
| 1 | 0.5 | 2 | 2 | 5 | 3 |
| 2 | 0.25 | 3 | 2 | 9 | 5 |
| 3 | 0.125 | 6 | 4 | 17 | 8 |

Editor’s note: It is FFS whether UE can be scheduled without any scheduling restrictions immediately after BWP switch has been completed.

8.x.2 BWP Switching Delay Requirement at Inactivity Timer Expiration

When inactivity timer expires on PCell, PSCell or any of the activated SCells, the UE shall be capable to complete the BWP switching upon the specified conditions.

If inactivity timer expires in slot n, the UE shall be capable to receive and transmit on the new BWP no later than in slot n + TBWP_switching_timer, where:

TBWP_switching_timer is up to:

- A3 slots for Type 1 capable UE and B3 slots for Type2 capable UE, if BWP switching involves changing the center frequency and/or bandwidth of the BWP, with or without changing the SCS or other baseband parameters,

- A4 slots for Type 1 capable UE and B4 slots for Type2 capable UE, if BWP switching involves changing only the SCS or other baseband parameters.

Table 8.x.2-2 BWP switching delay A1, A2, B3 and B4 for BWP switching due to inactivity timer expiration

|  | NR Slot length (ms) | Interruption length (slots) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | A3 | A4 | B3 | B4 |
| 0 | 1 | 1 | 1 | 2 | 1 |
| 1 | 0.5 | 2 | 1 | 4 | 2 |
| 2 | 0.25 | 3 | 2 | 8 | 4 |
| 3 | 0.125 | 5 | 4 | 16 | 8 |

Editor’s note: It is FFS whether UE can be scheduled without any scheduling restrictions immediately after BWP switch has been completed.