3GPP TSG-RAN WG4 Meeting AH-1807	R4-1809227

## Montreal, Canada, 2 – 6 July, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 37.145-2: Descriptions of the OTA declarations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-06-13 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | On top of the Draft CR to the TS 37.145-2 agreed during the email approval process after RAN4#87 in R4-1808502, in this Cat. B Draft CR we are completing the manufacturer’s declarations for OTA AAS BS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 3.3: adding missing abbreviations - 4.10: - Clarification notes for D9.x, D10.x and the new D11.x declaration sets. - Adding missing descriptions for the Rel-15 declarations - Addition of declaration descriptions for D11.x declarations - Reschuffling and renumbering of the declarations in table 4.10-2 - Removal of “beam RAT support” declaration, as its covered indirectly by the lsit of operating bands, which are RAT-specific. - D11.4: the “Equivalent passive antenna gain” declaration remaned into “Equivalent passive BS gain”, following the terminology used already in TS 37.105, annex B (“Calculation of EIRP based on fixed assumption of passive antenna gain”). - D11.16: removal of redundant decaration of “Total maximum number of supported carriers”. - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OTA AAS BS conformance testing would not be feasible due to missing declarations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAS BS	Active Antenna System Base Station

AoA	Angle of Arrival

BC	Band Category

BER	Bit Error Rate

CA	Carrier Aggregation

DTT	Digital Terrestrial Television

EIRP	Equivalent Isotropic Radiated Power

EIS	Equivalent Isotropic Sensitivity

FDD	Frequency Division Duplex

FRC	Fixed Reference Channel

ITU	International Telecommunication Union

ITU-R	Radio communication Sector of the ITU

MB-MSR	Multi-Band Multi-Standard Radio

MBT	Multi-Band Testing

MC	Multi-Carrier in a Single RAT

OBW	Occupied Band Width

OSDD	OTA Sensitivity Directions Declaration

OTA	Over The Air

RAT	Radio Access Technology

RB	Resource Block (for E-UTRA)

RDN	Radio Distribution Network

RF	Radio Frequency

RoAoA	Range of Angles of Arrival

SBT 	Single Band Testing

SC	Single-Carrier

TAB	Transceiver Array Boundary

TDD	Time Division Duplex

TRP	Total Radiated Power

UE	User Equipment

------------------------------ Next modified section ------------------------------

## 4.10	Manufacturer declarations

The AAS BS declarations categories D9.x and D10.x listed in table 4.10-1 are required to be provided by the manufacturer for the radiated requirements testing of the hybrid AAS BS or the OTA AAS BS.

For the hybrid AAS BS declarations required for the conducted requirements testing, refer to 3GPP TS 37.145-1 [9], subclause 4.10.

NOTE 1: 	D9.x declarations are related to the radiated Tx requirements, while D10.x declarations are related to the radiated Rx requirements.

NOTE 2: 	From Rel-15 onwards, additional D11.x declarations are introduced in table 4.10-2 for OTA AAS BS, in order to easily distinguish from the Rel-13/14 OTA declarations which are also applicable for hybrid AAS BS. Declarations in table 4.10-2 are applicable to OTA AAS BS only.

Table 4.10-1: Hybrid AAS BS and OTA AAS BS manufacturer declarations for radiated test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D9.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the AAS BS enclosure. |
| D9.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the AAS BS enclosure. |
| D9.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams, the minimum requirement to declare for conformance are the beams with the highest intend EIRP for each of the beams widths below: 1) Narrowest intended BeWθ, narrowest intended BeWϕ (possible when narrowest intended BeWθ) at the reference beam direction. 2) Narrowest intended BeWϕ, narrowest intended BeWθ (possible when narrowest intended BeWϕ) at the reference beam direction. 3) Widest intended BeWθ, widest intended BeWϕ (possible when widest intended BeWθ at the reference beam direction. 4) Widest intended BeWϕ, widest intended BeWθ (possible when widest intended BeWϕ) at the reference beam direction. 5) BeWθ and BeWϕ which provide highest intended EIRP of all possible beams at the reference beam direction. NOTE 1: Depending on the capability of the system some of these beams may be the same. When selecting the above five beam widths for declaration, all beams that the AAS BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. |
| D9.4 | Operating band support | UTRA or E-UTRA operating band(s) supported by AAS BS. Supported bands declared for every beam in D9.3. NOTE 2: these operating bands are related to their respective sigle-band RIBs. NOTE 3: this declaration in-directly provide information on the RAT’s supported by the AAS BS. |
| D9.6 | E-UTRA channel bandwidth support | E-UTRA channel bandwidth supported. Declared for each beam (D9.3) and each E-UTRA operating band (D9.4). |
| D9.7 | Reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D9.3). |
| D9.8 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam identified in D9.3. NOTE 4: In Rel-13/14 version of this specification, this declaration was called “EIRP accuracy directions set”. |
| D9.9 | Maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam identified in D9.3. |
| D9.10 | Rated beam EIRP | The rated EIRP level per carrier at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D9.9), as well as the reference beam direction pair (D9.7). Declared for every beam identified in D9.3. |
| D9.11 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam identified in D9.3. |
| D9.12 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D9.4-D9.11) made for the beams are identical. |
| D9.13 | Parallel beams | List of beams which have been declared equivalent (D9.12) and can be generated in parallel using independent RF power resources. Independent power resources means that the beams are transmitted from mutually exclusive transmitter units. |
| D9.14 | Number of carriers at maximum EIRP | The number of carriers per operating band the AAS BS is capable of generating at maximum EIRP declared each RAT (and multi-RAT) for every beam identified in D9.3. |
| D9.16 | Operating bands with multi-band dependencies | List of operating band combinations which are declared if those are generated using transceiver units supporting operation in multiple operating bands (i.e. multi-band transceiver units) through common active electronic component(s). NOTE 5: these operating bands are related to their respective multi-band RIBs. |
| D9.17 | Maximum radiated Base Station RF Bandwidth | Maximum Base Station RF Bandwidth in the operating band, declared for each supported operating band ( D9.4). |
| D9.18 | Maximum radiated Base Station RF Bandwidth for contiguous operation | Maximum Base Station RF Bandwidth for contiguous spectrum operation, declared for each supported operating band identified in D9.4. |
| D9.19 | Maximum radiated Base Station RF Bandwidth for non- contiguous operation | Maximum Base Station RF Bandwidth for non-contiguous spectrum operation, declared for each supported operating band (D9.4). |
| D9.20 | Inter-band CA bands | Declared inter-band CA bands supported per operating band (D9.4). |
| D9.21 | CA only operation | Declared per operating band (D9.4). |
| D9.22 | Multi-carrier HSPA only operation | Declared per each supported UTRA operating band (D9.4). |
| D9.23 | Reduced number of supported carriers at maximum EIRP in multi-RAT operations | Declared for each supported operating band (D9.4). |
| D9.24 | Reduced maximum EIRP at the total number of supported carriers in multi-RAT operations | Declared for each supported operating band (D9.4). |
| D9.25 | Radiated capability set (RCSA) | The manufacturer shall declare the supported radiated capability set(s) according to table 4.9-1 for each supported operating band (D9.4). |
| D9.26 | Maximum Radio Bandwidth of the operating band with multi-band dependencies | Largest Radio Bandwidth that can be supported by the operating bands with multi-band dependencies. Declared for each supported operating band which has multi-band dependencies (D9.16). |
| D9.27 | Total number of supported carriers for operating bands with multi-band dependencies | Total number of supported carriers for operating bands declared to have multi-band dependencies (D9.16). |
| D9.28 | Contiguous or non-contiguous spectrum support | Ability of AAS BS to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier in an operating band. |
| D9.29 | Contiguous and non-contiguous parameters identical | Contiguous and non-contiguous operation is possible then parameters are the same. |
| D10.1 | OSDD identifier | A unique identifier for the OSDD. |
| D10.2 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD identified in D10.1. NOTE 2: As each identified OSDD has a declared minimum EIS value (D10.6), multiple operating bands can be only declared if they have the same minimum EIS declaration. |
| D10.3 | OSDD RAT support | RAT(s) supported by the OSDD for each supported operating band, declared for every OSDD identified in D10.1. NOTE 3: If the OSDD supports multiple RAT's with different minimum EIS value (D10.6) if all other parameters are the same then different EIS values for different RATS and signal BW's may be declared for an OSDD. |
| D10.4 | OTA sensitivity E-UTRA supported channel bandwidth | The E-UTRA channel bandwidths supported by each OSDD. |
| D10.5 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD |
| D10.6 | Minimum EIS | The minimum EIS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared per supported RAT and E-UTRA supported channel BW for the OSDD (D10.4). The lowest EIS value for all the declared OSDD’s is called minSENS, while its related range of angles of arrival is called minSENS RoAoA. NOTE 4: If the AAS BS is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. |
| D10.7 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D10.9) for each OSDD. |
| D10.8 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD. |
| D10.9 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D10.8). NOTE 5: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. |
| D10.10 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D10.11). |
| D10.11 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. |

Table 4.10-2: OTA AAS BS manufacturer declarations for radiated test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D11.1 | AAS BS requirements set | Declaration of either hybrid AAS BS architecture conforming to the hybrid requirement set, or OTA AAS BS architecture conforming to the OTA requirement set. |
| D11.2 | OTA coverage range | Declared as OTA AAS BS common range of directions within which selected TX OTA requirements are intended to be met. NOTE 1: OTA coverage range is used for conformance testing of such OTA Tx requirements as occupied bandwidth, frequency error, TAE or EVM. |
| D11.3 | The rated carrier OTA BS power, Pmax,c,TRP | Pmax,c,TRP is declared as TRP OTA power per carrier, declared per supported operating band, per supported RAT. |

| D11.4 | Equivalent passive BS gain | Declared value of the equivalent gain (to the OTA AAS BS) of the passive BS, used for translation of regional regulatory requirements for unwanted emissions (i.e. EIRP value using an assumed passive BS gain). NOTE 2: referring to TS 37.105 [6] annex B on Calculation of EIRP based on fixed assumption of passive antenna gain, value of 17 dBi was used, with the assumption of antenna losses equal zero, i.e. directivity being equall to the BS antenna gain. |
| --- | --- | --- |
| D11.5 | Spurious emission category | Declare the OTA AAS BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [16]. |
| D11.6 | Geographic area support | The manufacturer shall declare the regions the OTA AAS BS may operate in. e.g. CEPT. |
| D11.7 | Band 20/XX or Band 32/XXXII support, operating in geographical areas allocated to broadcasting (DTT) | If the OTA AAS BS supports Band 20/XX or Band 32/XXXII, the manufacturer shall declare if the OTA AAS BS may operate in geographical areas allocated to broadcasting (DTT). |
| D11.8 | Band 20 or Band XX support, emission level for channel N (PEM,N) | If the OTA AAS BS supports Band 20 or Band XX and has been declared to operate in geographical areas allocated to broadcasting (DTT; declaration D11.7), the emission level for channel N (as defined in annex G of 3GPP TS 36.104 [4]) shall be declared. |
| D11.9 | Band 20 or Band XX support, maximum output power in 10 MHz (P10MHz) | If the OTA AAS BS supports Band 20 or Band XX and has been declared to operate in geographical areas allocated to broadcasting (DTT; declaration D11.7), the maximum output power in 10 MHz (annex G of 3GPP TS 36.104 [4]) shall be declared. |
| D11.10 | Band 32 or Band XXXII support, declared emission level in Band 32/XXXII (PEM,B32,ind) | If the OTA AAS BS supports Band 32 or Band XXXII and has been declared to operate in geographical areas allocated to broadcasting (DTT; declaration D11.7), the emission level in Band 32/XXXII (PEM,B32,ind, ind = a, b, c, d, e) shall be declared. |
| D11.11 | Co-existence with other systems | The manufacturer shall declare whether the OTA AAS BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or PHS operating in another operating band are deployed. |
| D11.12 | Co-location with other base stations | The manufacturer shall declare whether the OTA AAS BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD and/or E-UTRA operating in another operating band. |
| D11.14 | Single or multiple carrier | OTA AAS BS capability to operate with a single carrier (only) or multiple carriers. Declared per supported operating band, per RAT, per RIB. |
| D11.15 | Maximum number of supported carriers per operating band | Maximum number of supported carriers. Declared per supported operating band, per RAT, per RIB. |
| D11.16 | Total maximum number of supported carriers | Maximum number of supported carriers for all supported operating bands. Declared per RIB. |
| D11.16 | Other band combination multi-band restrictions | Declare any other limitation under simultaneous operation in the declared band combinations (D9.16), which have any impact on the test configuration generation. |
| D11.17 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by an OTA AAS BS in a particular operating band. Declared per single band RIB (D11.13). |
| D11.18 | Maximum supported power difference between carriers | Maximum supported power difference between carriers in each supported operating band. Declared per RIB. |
| D11.19 | Maximum supported power difference between carriers is different operating bands | Maximum supported power difference between any two carriers in any two different supported operating bands. Declared per operating bands combination (D9.16, D11.16). . |
| D11.21 | UTRA FDD MIMO support | Number of ‘antennas' supported by the UTRA FDD MIMO mode (i.e. 2 or 4). Declared per supported UTRA FDD operating band (D9.4). NOTE 3: The concept of "antenna 2", "antenna 3" and "antenna 4" is described in 3GPP TS 25.104 [2]. |
| D11.22 | UTRA Inner loop power control dynamic range | Power control dynamic range for UTRA inner loop power control. Declared per supported UTRA FDD operating band, per RIB. |
| D11.23 | Inter-band CA or inter-band HSDPA | Declaration of operating band combinations supporting inter-band CA or multi-band HSDPA. Declared per operating band combination (D9.16, D11.16). |
| D11.24 | Intra-band contiguous CA or intra-band contiguous HSDPA | Declaration of operating band(s) supporting intra-band contiguous CA, or intra-band contiguous HSDPA. Declared per operating band with CA support. |
| D11.25 | Intra-band non-contiguous CA or intra-band non-contiguous HSDPA | Declaration of operating band(s) supporting intra-band non-contiguous CA, or intra-band non-contiguous HSDPA. Declared per operating band with CA support. |
| D11.26 | OTA REFSENS RoAoA | Range of angles of arrival associated with the OTA REFSENS. |
| D11.27 | OTA REFSENS receiver target reference direction | Reference direction inside the OTA REFSENS RoAoA (D11.26). |
| D11.28 | OTA REFSENS conformance test directions | The following four OTA REFSENS conformance test directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. |

------------------------------ End of modified section ------------------------------