3GPP TSG-RAN WG4 AH1807	R4- 1809243

Montreal, Canada 2nd –6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843 - OTA ACLR measurement in a Near Field Test Range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MVG Industries |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-07-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding measurement procedure for OTA ACLR measurement in Near Field Test setup |  |  |  |  |  |  |  |  |
| Summary of change: |  | The addition of text for OTA ACLR measurement procedure for in-band TRP requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test procedure is needed for OTA ACLR requirement with respect to the Near Field test method. If not included, then Near Field test method is excluded from this OTA requirement for conformance requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 10.4.2 OTA ACLR

## 10.4.2.4 Near Field

### 10.4.2.4.1 Description

The system setup is depicted in section 10.2.2.4.1. In case of OTA ACLR measurement, the NF to FF transform is not needed since ACLR is based on TRP.

### 10.4.2.4.2 Calibration

Stage 1 – Calibration:

Calibration shall be done by following the procedure in 10.2.2.4.2.

### 10.4.2.4.3 Procedure

Stage 2 - Measurement:

The testing procedure consists of the following steps:

- Configure TX branch and carrier according to the required test configuration

- Measure in Near Field ACLR and CALCR for the frequency offsets both side of carrier frequency for both polarizations. In multiple carrier case only offset frequencies below the lowest and above the offsets highest carrier frequency used shall be measured.

  - Power is measured Near Field – no NF to FF transform is applied to

  - Full sphere power is measured with using a defined sampling grid (refer to Annex X for proper measurement grids)

- Calculate TRP by using the formula in [2]

### 10.4.2.4.4	MU assessment

### 10.4.2.4.4.1	MU Budget

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
|  | Stage 2:, EIRP near field radiation pattern measurement and EIRP near field DUT power measurement |  |
| 1 | Axes Intersection | B4-1 |
| 2 | Axes Orthogonality | B4-2 |
| 3 | Horizontal Pointing | B4-3 |
| 4 | Probe Vertical Position | B4-4 |
| 5 | Probe H/V pointing | B4-5 |
| 6 | Measurement Distance | B4-6 |
| 7 | Amplitude and Phase Drift | B4-7 |
| 8 | Amplitude and Phase Noise | B4-8 |
| 9 | Leakage and Crosstalk | B4-9 |
| 10 | Amplitude Non-Linearity | B4-10 |
| 11 | Amplitude and Phase Shift in rotary joints | B4-11 |
| 12 | Channel Balance Amplitude and Phase | B4-12 |
| 13 | Probe Polarization Amplitude and Phase | B4-13 |
| 14 | Probe Pattern Knowledge | B4-14 |
| 15 | Multiple Reflections | B4-15 |
| 16 | Room Scattering | B4-16 |
| 17 | DUT support Scattering | B4-17 |
| 18 | Positioning | B4-21 |
| 19 | Probe Array Uniformity | B4-22 |
| 20 | Mismatch of receiver chain (i.e. between receiving antenna and measurement receiver) | B4-23 |
| 21 | Insertion loss of receiver chain | B4-24 |
| 22 | Uncertainty of the absolute gain of the probe antenna | B4-25 |
| 23 | MU of TE derived from conducted specification | E |
| 24 | Measurement repeatability - Positioning Repeatability | B4-27 |
|  | Stage 1: Calibration measurement |  |
| 25 | Uncertainty of network analyser | E |
| 26 | Mismatch of receiver chain | B4-29 |
| 27 | Insertion loss of receiver chain | B4-30 |
| 28 | Mismatch in the connection of the calibration antenna | B4-31 |
| 29 | Influence of the calibration antenna feed cable | B4-32 |
| 30 | Influence of the probe antenna cable | B4-33 |
| 31 | Uncertainty of the absolute gain of the calibration antenna | E |
| 32 | Short term repeatability | B4-35 |

### 10.4.2.4.4.2	MU Value

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Axes Intersection | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 2 | Axes Orthogonality | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 3 | Horizontal Pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 4 | Probe Vertical Position | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 5 | Probe H/V pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 6 | Measurement Distance | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 7 | Amplitude and Phase Drift | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 8 | Amplitude and Phase Noise | 0.02 | 0.02 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 9 | Leakage and Crosstalk | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 10 | Amplitude Non-Linearity | 0.04 | 0.04 | Gaussian | 1.00 | 1 | 0.04 | 0.04 |
| 11 | Amplitude and Phase Shift in rotary joints | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 12 | Channel Balance Amplitude and Phase | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 13 | Probe Polarization Amplitude and Phase | 0.0001 | 0.0001 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 14 | Probe Pattern Knowledge | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 15 | Multiple Reflections | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 16 | Room Scattering | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| 17 | DUT support Scattering | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 18 | Positioning | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 19 | Probe Array Uniformity | 0.055 | 0.055 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 20 | Mismatch of receiver chain | 0.284 | 0.284 | U-Shaped | 1.41 | 1 | 0.20 | 0.20 |
| 21 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 22 | Uncertainty of the absolute gain of the probe antenna | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 23 | MU of TE derived from conducted specification | 0.41 | 0.56 | Gaussian | 1.00 | 1 | 0.41 | 0.56 |
| 24 | Measurement repeatability - positioning repeatability | 0.15 | 0.15 | Gaussian | 1.00 | 1 | 0.15 | 0.15 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 25 | Network analyzer | 0.13 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 |
| 26 | Mismatch of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 27 | Insertion loss of receiver chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 28 | Mismatch in the connection of the calibration antenna | 0.02 | 0.02 | U-Shaped | 1.41 | 1 | 0.01 | 0.01 |
| 29 | Influence of the calibration antenna feed cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 30 | Influence of the probe antenna cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 31 | Reference antenna | 0.50 | 0.25 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 32 | Short term repeatability | 0.088 | 0.088 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.59 | 0.7 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.16 | 1.38 |

MU for OTA ACLR is derived from the below equation:

MU OTA ACLR=  MU for OTA Near Field Power density + TRP systematic Error

TRP systematic error is reported in Annex X