3GPP TSG-RAN WG4 Meeting AH1807	R4-1809254

## Montreal, Canada, 02nd – 06th July, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843 - OTA Tx IMD in a Near Field Test Range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MVG Industries |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-07-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding measurement procedure for OTA Co-location Tx IMD in a Near Field Test setup |  |  |  |  |  |  |  |  |
| Summary of change: |  | The addition of text for OTA Co-location Tx IMD measurement procedure for co-location requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test procedure is needed for OTA Co-location Tx IMD measurement with respect to the Near Field test method. If not included, then Near Field test method is excluded from this OTA requirement for conformance requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.X.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 10.6.X.4	Near Field

##### 10.6.X.4.1	General

This method measures the OTA transmitter intermodulation in a Near Field Test Range. An example of a Near Field measurement setup for TxIMD measurements is shown in Figure 10.6.X.4.1-1

# Figure 10.6.X.4.1-1: Example of a Near Field measurement setup for TxIMD measurement

##### 10.6.X.4.2	Calibration

Calibration for wanted signal power level is the same as in subclause 10.2.2.4.2.

Additionally, the losses in the signal chain between co-location reference antenna conducted input(s) and measurement equipment need to be calibrated out:

1) Cable and matching loss calibration for co-location reference antenna

- For each polarization supported by the co-location reference antenna

a)	Measure co-location reference antenna reflection coefficient separately at the antenna's connector with a network analyser (or equivalent measurement equipment) to obtain ΓANT.

- Measure cable loss from point C to input of co-location reference antenna, call this L ANT↔C which is the equivalent of 20log|S21| from the use of a network analyser.

- Calculate the combined total path loss from C to co-location reference antenna using the following equating

Lcal = L ANT↔C + 10log(1 - |ΓANT|2)

##### 10.6.X.4.3 	Procedure

- Place AAS BS and co-location reference antenna as specified in subclause 4.15, 3GPP TS 37.145-2 [24].

- Connect Measurement antenna and co-location reference antenna to the measurement equipment.

- Configure measurement equipment to transmit the blocking signal through each polarization of the co-location measurement antenna at appropriate level for the BS class as specified in section 6.X.X

- Configure all OTA AAS BS TX branches according to a maximum power requirement for a single carrier.

- Set the AAS BS  to transmit the test signal according to appropriate test model.

- Measure unwanted emissions power spectral density

- Compute TRP

Additional factors to be considered in measurement of unwanted emissions are that in some measurement positions co-location reference antenna will be in the signal path between EUT and range antenna. These measurement positions shall be excluded from the results. It is not straightforward how to separate if emissions are coming from the co-location reference antenna or EUT. This will further impact measurement uncertainty.

##### 10.6.X.3.4 	MU assessment

###### 10.6.X.3.4.1 	MU Budget

###### 10.6.X.3.4.2 	MU Value

#### 10.6.X.x	Summary

{editors note: Last section at this level summarises MU estimates for all chambers and concludes final agreed MU}