3GPP TSG-RAN4 Meeting AH#2 	R4-1808775

# Montreal, Canada, 2nd to 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR37.843 MU of OTA Receiver intermodulation for IAC and CATR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-07-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The text capturing detailed procedures in CATR for receiver intermodulation is missing. For IAC, the MU budget does not align with the agreed method |  |  |  |  |  |  |  |  |
| Summary of change: |  | All changes highlighted in green. Conformance test procedure and MU budget are included. IAC MU updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Detailed test procedure descriptions not fully clear and correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 10.3.6 	OTA receiver intermodulation

#### 10.3.6.1	General

{editors note: general description of the test requirement placed here – non chamber specific}

#### 10.3.6.2	In-door anechoic chamber

##### 10.3.6.2.1	General

This method measures the receiver intermodulation in an in-door anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the transmitting antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of transmitting antenna and λ is the wavelength. The measurement system setup is as depicted in figure 10.3.6.2.1-1.

##### Figure 10.3.6.2.1-1: In-door Anechoic Chamber measurement system setup for  OTA receiver intermodulaiton

##### 10.3.6.2.2	Calibration

Calibration shall be done with the procedure shown in 37.842 [4] subclause 10.3.1.1.2.3 for the frequencies of wanted and unwanted signals with power combiner/coupler. Calculate the calibration value for wanted and unwanted signal from A to D, from A to E, and from A to F.

LWanted_cal, A→D:  Calibration value for wanted signal between A and D in figure 10.3.6.2.1-1.

LUnanted_cal, A→E:  Calibration value for unwanted signal between A and E in figure 10.3.6.2.1-1.

LUnanted_cal, A→F:  Calibration value for unwanted signal between A and F in figure 10.3.6.2.1-1.

##### 10.3.6.2.3 	Procedure

1)	Uninstall the reference antenna and install the AAS BSwith its manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with testing system.

2)	Set the AAS BS to be satisfied with the RoAoA desired by the requirement covering conformance testing receiving direction.

3)	Rotate the AAS BS to make the conformance testing receiving direction aligned with the boresight of the reference antenna at the calibration stage.

[4)	For FDD AAS BS start BS transmission at the required condition. ]

5)	Set the test signal mean power at the RF signal source generator for wanted signal as the required level plus LWanted_cal, A→D  and the reference measurement channel.

6) Set the test signal mean power at the RF signal source generators for unwanted signal as the required level plus LUnwanted_cal, A→E  or plus LUnwanted_cal, A→F and the intereferece signal channel.

6)	Measure the throughput according to annex E of the 3GPP TS 36.141 [??].

7)	Repeat the above steps 1)~6) per conformance testing direction.

##### 10.3.6.2.4 	MU assessment

###### 10.3.6.2.4.1 	MU Budget

The MU is estimated based on MU in the conducted specification 36.141, the MU for EIS in section 10.3.3 and an estimated uncertainty for matching of test equipment and DUT or test antenna.

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect

Where

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUinterfer=MUinterfer_conducted2-MUmatching_conducted2

MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

The uncertainty budget behind the EIS MU is captured in section 10.3.3.  The 2 sigma matching uncertainty is 0.196dB for   f ≦ 3GHz and 0.314dB for  3GHz < f ≦ 4.2 GHz .

The ACLR effect from the interference signal is assumed to be 0.4dB, as in 36.141. This is under the assumption that no additional PA is needed for the test signal in the OTA test setup.

###### 10.3.6.2.4.2 	MU Value

The MU for RX IM is calculated as follows. Note that all uncertainties in the table are 1.96σ values.

Table 10.3.6.2.4.2-2 Test system uncertainty of RX IM for in-door anechoic chamber

| Frequency range | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz |  |
| --- | --- | --- | --- |
| MUwanted_conducted | ±0.7 dB | ±1.0 dB |  |
| MUCWinterfer_conducted | ±0.5 dB | ±0.7 dB |  |
| MUMODinterfer_conducted | ±0.7 dB | ±1.0 dB |  |
| MUmatching_conducted | ±0.196 dB | ±0.314 dB |  |
| MUwanted | ±0.672dB | ±0.949 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
| MUCWinterfer | ±0.460 dB | ±0.626 dB | MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2 |
| MUMODinterfer | ±0.672dB | ±0.949 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
| MUEIS_OTA | 1.22 dB | 1.25 dB |  |
| MUTestEquipment_OTA | 0.9 dB | 0.9 dB |  |
| MUmatching_OTA | ±0.196 dB | ±0.314 dB |  |
| MUchamber_OTA | ±0.8 dB | ±0.807 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment2-MUmatching_OTA2 |
| ACLR effect | 0.4 dB |  |  |
| MURXIM_OTA | 2.0 dB | 2.5 dB | MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect |

#### 10.3.6.3	CATR

##### 10.3.6.3.1	General

This method measures the receiver intermodulation in compact antenna test range with a sufficient separation between reference point of the AAS BS and the transmitting antenna by the range antenna reflector. The measurement system setup is as depicted in figure 10.3.6.3.1-1.

##### Figure 10.3.6.3.1-1: CATR measurement system setup for receiver intermodulation

##### 10.3.6.3.2 	Calibration

Calibration shall be done with the procedure shown in 37.842 [4] subclause 10.3.1.1.3.3 for the frequencies of wanted and 2 interfering signals with power combiners/couplers. Calculate the calibration value for wanted and interfering signals from A to B as well as from A to E and A to H.

LWanted_cal, A→B:  Calibration value for wanted signal between A and B in figure 10.3.6.3.1-1.

Linterferer1_cal, A→E:  Calibration value for interfering signal between A and E in figure 10.3.6.3.1-1.

Linterferer2_cal, A→H:  Calibration value for interfering signal between A and H in figure 10.3.6.3.1-1.

##### 10.3.6.3.3 	Procedure

- Set up AAS BS in place of SGH from calibration stage. Align AAS BS with boresight of transmitting antenna.

- Configure RF signal generator for wanted signal to set the signal at the required level plus LA→B and at reference measurement channel.

- Configure RF signal generator for interferer1 to set a modulated signal at the required level plus LA→E and at reference measurement channel. The interferer’s center frequency offset should also fulfil the requirement.

- Configure RF signal generator for interferer2 to set a CW signal at the required level plus LA→H and at reference measurement channel. The interferer’s center frequency offset should also fulfil the requirement.

- Measure the BER or throughput of the wanted signal according to annex E of the 3GPP TS 36.141 [??].

- Repeat steps all above per involved band for multi-band RIB(s).

##### 10.3.6.3.4 	MU assessment

###### 10.3.6.3.4.1 	MU Budget

###### The MU is estimated based on MU in the conducted specification 36.141, the MU for EIS in section 10.3.3 and an estimated uncertainty for matching of test equipment and DUT or test antenna is in section 10.3.5.

###### 10.3.6.3.4.2 	MU Value

Test system uncertainty is obtained by follwing equations:

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect

Where

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUMODinterfer=MUMODinterfer_conducted2-MUmatching_conducted2

MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

And we assume  MUmatching_conducted= MUmatching_OTA

Test system uncertainty for receiver intermodulation is shown in Table 10.3.6.3.4.2-1. Note that all uncertainties in table are 1.96σ values.

Table 10.3.6.3.4.2-1 Test system uncertainty of receiver intermodulation for CATR

| Frequency range |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RX IM | MUwanted_conducted | ±0.7 dB |  |  | ±1.0 dB |  |  |  |
|  | MUCWinterfer_conducted | ±0.5 dB |  |  | ±0.7 dB |  |  |  |
|  | MUMODinterfer_conducted | ±0.7 dB |  |  | ±1.0 dB |  |  |  |
|  | MUmatching_conducted | ±0.294 dB |  |  | ±0.294 dB |  |  |  |
|  | MUwanted | ±0.635 dB |  |  | ±0.956 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |  |  |
|  | MUCWinterfer | ±0.404 dB |  |  | ±0.635 dB | MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2 |  |  |
|  | MUMODinterfer | ±0.635 dB |  |  | ±0.956 dB | MUMODinterfer=MUMODinterfer_conducted2-MUmatching_conducted2 |  |  |
|  | MUEIS_OTA | 1.33 dB |  |  | 1.40 dB |  |  |  |
|  | MUTestEquipiment_OTA | 0.9 dB |  |  | 0.9 dB |  |  |  |
|  | MUmatching_OTA | ±0.294 dB |  |  | ±0.294 dB |  |  |  |
|  | MUchamber_OTA | ±0.933 dB |  |  | ±1.03 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2 |  |  |
|  | ACLR effect | 0.4 dB |  |  |  |  |  |  |
|  | MURXIM_OTA | 2.0 dB |  |  | 2.6 dB | MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect |  |  |

#### 10.3.6.x	Summary

The different OTA test chamber MU are summarised below:

Table 10.3.6.x-1: Test system specific measurement uncertainty values for the OTA receiver intermodulation

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 2.0 | 2.5 |
| Compact Antenna Test Range | 2.0 | 2.6 |
| … |  |  |
| … |  |  |
| Common maximum accepted test system uncertainty | 2.0 | 2.6 |