3GPP TSG-RAN WG4 Meeting #AH-1807	R4-1809436

Montreal, Quebec, Canada, 2nd – 6th July 2018

Source:	Nokia, Nokia Shanghai Bell, NTT DoCoMo

Title:	Way forward on TRP systematic error and measurement uncertainty budget

Agenda item:	4.2.1.1.4.1

Document for:	Approval

# 1	Introduction

During the 3GPP RAN4 #AH-1807 meeting, contributions on TRP systematic error values [1]-[3] and TRP measurement uncertainty values [4]-[7] were discussed and presented.

This way forward aims at capturing any agreements and open issues at this meeting in an attempt to make progress following online and offline discussions.

# 2	Discussion

## 2.1 	TRP systematic error

According to Rel-15 TR 37.843, TRP systematic error is defined as

TRPsyserr=TRPEstimate- TRPReference

where TRPReference is the theoretical TRP and TRPEstimate is the numerically estimated TRP using one of the agreed measurement grids.

Table 1 lists the proposed TRP systematic error values for in band TRP from different companies for frequency ranges, f ≤ 6 GHz.

| Reference | TRP systematic error (dB) |
| --- | --- |
| Huawei [1] | 0.5 |
| ZTE [8] | 0.5 (Rayleigh) |
| ZTE [8] | 0.8 (space vector) |
| Ericsson [?] | 0.5 |
| MVG Industries [10] | 0.3 |
| Nokia [3] | 1.9 |

Table 1: Summary of proposed TRP systematic error values for in band TRP from different companies for frequency ranges, f ≤ 6 GHz

The next step is to determine what methodology is used reach a final agreeable TRP systematic error value given the different values, which is listed as one of the open issues in the way forward on TRP systematic error and MU [9]. One methodology is to calculate the average of the TRP systematic error values in Table 1.

Proposal 1: the final TRP systematic error, which is needed for calculating TRP measurement uncertainty, is based on the average of TRP systematic error values contributed by different companies.

Proposal 2: the TRP systematic error = 0.75 dB for in band TRP (eAAS and NR FR1)  in the frequency range, f ≤ 6 GHz.

Note:  if SE appears in the TS then it should be rounded to 0.8dB

## 2.2 TRP measurement uncertainty

The contributions from different companies are as follows:

NTT DoCoMo [4]:

- for absolute OTA ACLR, OTA OBUE and OTA SEM in an indoor anechoic chamber:

  - [ MUper-point=0.88 dB, f ≤3 GHz                     1.06 dB, 3 GHz<f ≤4.2 GHz   ]

note: the “uncertainty of the RF Power Measurement Equipment” value will be revised and MUper-point will be re-calculated.

Ericsson [5]-[7]:

- for OTA BS maximum output power and OTA OBUE in CATR:

  - the per point MU is the same the Rel-13 AAS BS EIRP MU value, that is

MUper-point=0.99 dB, f ≤3 GHz                     1.17 dB, 3 GHz<f ≤4.2 GHz

- for OTA ACLR ratio in CATR:

  - the total OTA ACLR (ratio) MU is MUTRP-total=0.58 dB, f ≤3 GHz                     0.77 dB, 3 GHz<f ≤4.2 GHz

Huawei [1]:

- for in band TRP requirements in CATR:

  - adopt the Rel-13 AAS BS EIRP MU, that is MUper-point=0.99 dB, f ≤3 GHz                     1.17 dB, 3 GHz<f ≤4.2 GHz

  - methodology for combining MUper-point and TRPsyserr is the root of the sum of the squares (RSS)

  - MUTRP-total= 1.4 dB, f ≤3 GHz                       1.55 dB, 3 GHz<f ≤4.2 GHz

MVG [2]:

- for in band TRP requirements in near field test range:

  - methodology for combining MUper-point and TRPsyserr is the root of the sum of the squares (RSS)

Nokia [3]:

- for in band TRP requirements in CATR:

  - adopt the Rel-13 AAS BS EIRP MU and additionally the following uncertainty contributors attributed to TRP measurement:

    - DUT offset from phase centre of axis of rotation

    - Polarization alignment

    - Test system frequency flatness

  - methodology for combining MUper-point and TRPsyserr is the root of the sum of the squares (RSS)

Based on the above contributions, the following proposals can be made:

Proposal 3: adopt Rel-13 EIRP MU as the MU per point for in band TRP test requirements (OTA BS maximum output power, absolute OTA ACLR, OTA SEM and OTA OBUE) for frequency ranges, f ≤ 3 GHz, 3 GHz < f ≤ 4.2 GHz

Proposal 4: methodology for calculating the total TRP MU is by combining MUper-point and TRPsyserr as the root of the sum of the squares (RSS), i.e.,

MUTRP_total=MUper-point2+TRPsyserr2

Proposal 5: in addition to the MU per point, adopt the following uncertainty contributor and revise the existing CATR contributor “Misalignment DUT & pointing error”:

| Uncertainty contributor | Description |
| --- | --- |
| Test system frequency flatness | This uncertainty is coming from frequency interpolation error caused by a finite frequency resolution during the calibration stage. |

CATR:

| TRP uncertainty budget |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 9 | Test system frequency flatness | [0.25] | [0.25] | Gaussian | 1 | 1 | [0.25] | [0.25] |

## 2.3	Open issues

To complete the work in TT for TRP requirements for OTA AAS BS, NR BS type 1-O and NR BS type 2-O, the following open issues need to be addressed at the next RAN4 meeting in August 2018:

- Determine TRP systematic error for:

  - in band TRP requirements for NR BS type 2-O taking into account the following assumptions:

      - beamwidth = TBD

      - frequency range = [24.25 – 29.5 GHz] and [37 – 40 GHz] (other options are not precluded)

  - out of band TRP requirements for OTA AAS BS, NR BS type 1-O and 2-O taking into account the following assumptions:

      - beamwidth = TBD

      - frequency range: 30 MHz – 4 GHz, 4 GHz – 19 GHz, 19 GHz – 26 GHz, 26 GHz – 60 GHz (other options are not precluded)

Companies are also free to choose any of the agreed measurement grids or other techniques to determine the TRP systematic error.

- Determine total MU value for in band and out of band TRP for NR BS type 1-O (4.2 GHz < f ≤ 6 GHz) and 2-O.

- Determine total MU value for out of band TRP for OTA AAS BS, NR BS type 1-O and 2-O.

# 3	Way forward

Based on the discussions in the document, the following agreements can be made:

Proposal 1: the final TRP systematic error, which is needed for calculating TRP measurement uncertainty, is based on the average of TRP systematic error values contributed by different companies.

Proposal 2: the TRP systematic error = 0.75 dB for in band TRP in the frequency range, f ≤ 6 GHz.

Proposal 3: adopt Rel-13 EIRP MU as the MU per point for in band TRP test requirements (OTA BS maximum output power) for frequency ranges, f ≤ 3 GHz and 3 GHz < f ≤ 4.2 GHz

Open issue: absolute OTA ACLR, OTA SEM and OTA OBUE the MU should be considered for next meeting

Proposal 4: methodology for calculating the total TRP MU is by combining MUper-point and TRPsyserr as the root of the sum of the squares (RSS), i.e.,

MUTRP_total=MUper-point2+TRPsyserr2

Proposal 5: in addition to the MU per point, consider to adopt the following uncertainty contributor and revise the existing CATR contributor “Misalignment DUT & pointing error”:

| Uncertainty contributor | Description |
| --- | --- |
| Test system frequency flatness | This uncertainty is coming from frequency interpolation error caused by a finite frequency resolution during the calibration stage. |

| TRP uncertainty budget |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | [0.3] | [0.3] | Rectangular | √3 | 1 | [0.174] | [0.174] |
| 9 | Test system frequency flatness | [0.25] | [0.25] | Gaussian | 1 | 1 | [0.25] | [0.25] |

To complete the work in TT for TRP requirements for OTA AAS BS, NR BS type 1-O and NR BS type 2-O, the following open issues need to be addressed at the next RAN4 meeting in August 2018:

- Determine TRP systematic error for:

  - in band TRP requirements for NR BS type 2-O taking into account the following assumptions:

      - beamwidth/aperture = companies to submit SE based on FR2 aperture estimations using [Step size = lambda/d]

      - frequency range = [24.25 – 29.5 GHz] and [37 – 40 GHz] (other options are not precluded)

  - out of band TRP requirements for OTA AAS BS, NR BS type 1-O and 2-O taking into account the following assumptions:

      - beamwidth/aperture = companies to submit SE based on FR2 aperture estimations using [Step size = lambda/d]

      - frequency range: 30 MHz – 4 GHz, 4 GHz – 19 GHz, 19 GHz – 26 GHz, 26 GHz – 60 GHz (other options are not precluded)

Companies are also free to choose any of the agreed measurement grids or other techniques to determine the TRP systematic error.

- Determine total MU value for in band and out of band TRP for NR BS type 1-O (4.2 GHz < f ≤ 6 GHz) and 2-O.

- Determine total MU value for out of band TRP for OTA AAS BS, NR BS type 1-O and 2-O.

# References

- R4-1809205, Discussion on TRP systematic error and MU, Huawei

- R4-1809261,	In-band TRP Systematic Error for Spherical Equal Angle Grid, MVG Industries

- R4-1809270,	On TRP systematic errors for in band TRP test requirements, Nokia, Nokia Shanghai Bell

- R4-1809164,	MU of OTA OBUE, SEM, ACLR absolute requirement, NTT DoCoMo

- R4-1808651,	MU framework and values for OBUE for CATR, Ericsson

- R4-1808652,	MU framework and values for OTA Base station output power for CATR, Ericsson

- R4-1809118,	MU framework and values for ACLR for CATR, Ericsson

- R4-1806879, Discussion on the TRP measurement, ZTE

- R4-1808377, Way forward on TRP systematic error and MU, Nokia, Nokia Shanghai Bell

- R4-1805107, Overview of In-band TRP uncertainty versus sampling grid, MVG