3GPP TSG-RAN WG4 Meeting #AH-1807	R4-1809450

## Montreal, Canada, 2 - 6 July, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843: Tx IMD test method |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-06-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CATR test method for Tx IMD to be used for MU evaluation is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of CATR test method for Tx IMD to be used for MU evaluation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CATR test method for Tx IMD to be used for MU evaluation remains missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 10.6.4  	OTA Transmitter intermodulation

#### 10.6.4.1 	General

#### 10.6.4.2 	In-door anechoic chamber

##### 10.6.4.2.1 	General

[This method measures the transmitter intermodulation in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength.] The measurement system setup is as depicted in figure 10.6.x.2.1-1.

{editor’s note: the separation is [] because the out-of-band spurious emission test may not satisfy the condition that the sparation is no less than 2D2/λ.}

Figure 10.6.4.2.1-1: In-door Anechoic Chamber measurement system setup for transmitter intermodulation

{editor’s note: Diagrams needs to be updated to show the co-location reference antenna and AAS BS are on the same position  and also the PA associated with the interferer signal. }

##### 10.6.4.2.2 	Calibration

Calibration shall be done with the procedure shown in 10.2.2.2.2

Calibration of the emission measurements are procedure shown in 10.2.2.2.2.

For calibration of [co-location reference antenna] part is FFS.

##### 10.6.4.2.3 	Procedure

1)	Uninstall the reference antenna and install the AAS BS with the manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with the testing system.

2)	Set the [co-location reference antenna] as specified in 3GPP TS37.145-2 sub-clause 4.15 [24], [at the declared worst case side.]

3) 	Set the signal generator to generate the interfering signal to achieve the required level at the input port of [co-location reference antenna].

4)	Perform required ACLR, OBUE (for E-UTRA), SEM (for UTRA), occupied bandwidth, spurious emission tests according to the corresponding testing procedures.

##### 10.6.4.2.4 	MU assessment

###### 10.6.4.2.4.1 	MU Budget

###### 10.6.4.2.4.2 	MU Value

#### 10.6.4.3	CATR

##### 10.6.4.3.1	General

This method measures the OTA transmitter intermodulation in a compact antenna test range chamber. CATR can be used only within the frequency range where the test facility is functional.

##### 10.6.4.3.2	Calibration

Calibration for wanted signal power level is the same as in subclause 10.2.2.3.2.

Additionally, the losses in the signal chain between [co-location reference antenna] conducted input(s) and measurement equipment need to be calibrated out:

1) Cable and matching loss calibration for co-location reference antenna

- For each polarization supported by the co-location reference antenna

a)	Measure co-location reference antenna reflection coefficient separately at the antenna's connector with a network analyser (or equivalent measurement equipment) to obtain ΓANT.

- Measure cable loss from point F to input of co-location reference antenna, call this L ANT↔F which is the equivalent of 20log|S21| from the use of a network analyser.

- Calculate the combined total path loss from F to co-location reference antenna using the following equating

Lcal = L ANT↔F + 10log(1 - |ΓANT|2)

##### 10.6.4.3.3 	Procedure

- Set up AAS BS (DUT) in place of SGH from the calibration stage, ensure that the [co-location reference antenna] and DUT are positioned as described in 3GPP TS 37.145-2, subclause 4.15 [24], [co-location reference antenna] placed at the declared worst-case side.

- Place Range antenna in boresight direction (reference direction) at far-field distance, aligned in both polarizations with the AAS BS.

- Connect Range antenna and co-location reference antenna to the measurement equipment.

- Configure measurement equipment to transmit the blocking signal through each polarization of the [co-location reference antenna] at appropriate level for the BS class as specified in section 6.X.X

- Configure all OTA AAS BS TX branches according to a maximum power requirement for a single carrier.

- Set the DUT to transmit the test signal according to appropriate test model.

- Measure unwanted emissions according to subclauses 10.4 and 10.5

##### 10.6.4.3.4 	MU assessment

###### 10.6.4.3.4.1 	MU Budget

###### 10.6.4.3.4.2 	MU Value

#### 10.6.4.x	Summary

## 10.6.5 	OTA co-location blocking

------------------------------ End of modified sections ------------------------------