3GPP TSG-RAN WG4 Meeting #AH-1807	R4-1809489

Montreal, Quebec, Canada, 2nd – 6th July 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TS 38.141-2 – OTA unwanted emissions – General (6.7.1)

Agenda item:	5.1.4.1

Document for:	Approval

# 1	Introduction

This is a revision of R4-1809267, which provides a text proposal to Section 6.7.1 General of TS 38.141-2 [1].

Changes are highlighted in yellow.

Constructive comments are encouraged.

# 4	References

- R4-1809266, Draft TS 38.141-2 (v0.2.0), Huawei

# 5	Text proposal

TS 38.141-2 v0.2.0

--------------Start of text proposal-------------

### 6.7.1	General

OTA unwanted emissions consist of so-called out-of-band emissions and spurious emissions according to ITU definitions ITU-R SM.329 [5]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the BS channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The OTA out-of-band emission requirements for BS type 1-O are specified in terms of Adjacent Channel Leakage power Ratio (ACLR) and operating band unwanted emissions (OBUE). The OTA operating band unwanted emissions define all unwanted emissions in each supported downlink operating band plus the frequency ranges ΔfOBUE above and ΔfOBUE below each band. OTA unwanted emissions outside of this frequency range are limited by an OTA spurious emissions requirement.

The OTA out-of-band emissions for BS type 2-O are specified in terms of ACLR and spectrum emission mask (SEM). The SEM limits in FR2 are defined from ΔfOBUE below the lowest frequency of each supported downlink operating band up to ΔfOBUE above the highest frequency of each supported downlink operating band.

The maximum offset of the operating band unwanted emissions mask from the operating band edge is ΔfOBUE. The value of ΔfOBUE is defined in table 6.7.1-1 for BS type 1-O and BS type 2-O for the NR operating bands.

Table 6.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE [MHz] |
| --- | --- | --- |
| BS type 1-O | FDL_high – FDL_low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL_high – FDL_low ≤ 900 MHz | 40 |
| BS type 2-O | FDL_high – FDL_low < 3250 MHz | 1500 |

The OTA unwanted emission requirements are applied per cell for all the configurations supported by BS type 1-O.  Requirements for OTA unwanted emissions are captured using TRP, directional requirements or co-location requirements as described per requirement.

There is in addition a requirement for OTA occupied bandwidth.

--------------end of text proposal-------------