3GPP TSG-RAN WG4 Meeting AH-1807                                                       R4-1809330

Montreal ,Canada, 2nd  – 6th July, 2018

Agenda Item:   5.4.1.5

Source:        Samsung

Title:          Views on PMI test cases

Document for:  Discussion

Introduction

A dedicated specification was assigned for NR UE performance requirements as 38.101-4. The target competition time for Rel-15 NR performance part is RAN#82 in Dec 2018. In this contribution, preliminary views for UE PMI requirements were provided.

Discussion

For PMI reporting, 4 types of codebook structure were introduced for different applicable scenarios (SU-MIMO/MU-MIMO, single panel/multi-panels):

- Type I–Single panel codebook: similar as LTE eFD-MIMO Class A codebook (single Beam and co-phasing selection)

- Type I–Multi panel codebook: further extended with inter-panel co-phasing selection for multi panels Ng = (2 or 4)

- Type II codebook: similar as LTE eFD-MIMO Advanced codebook with multi-beam linear combinations

- Type II Port Selection Codebook

Different codebook structures correspond to different deployment scenario and beamforing scheduled schemes. Only Type 1-single panel codebook was mandatory in Rel-15. It’s agreed in last RAN4 meeting, in Rel-15 only focused on single panel codebook for introducing PMI test cases.

For Type I–Single panel codebook, test metric and beam-forming mode for LTE Rel13 FDMIMO Class A PMI test cases can be reused as a starting point.

Test metric: Similar as LTE, relative throughput ratio between following PMI and random PMI under fixed reference channel can be used. Regarding test point, FFS pending on performance difference for the introduced test cases.

Proposal 1: Reusing Test metric as LTE, relative throughput ratio between followed PMI and random PMI under FRC

- FFS for test point

Number of ports: for Type I–Single panel codebook, {2, 4, 8, 12, 16, 24, 32} ports supported in Rel-15. Also following UE feature list, FR1 mandatory supported 8Tx wideband codebook and 4Tx ports for sub-band codebook, for other number of ports belong to UE capability. We proposed to introduce test cases based on UE capability: i.e. 8 ports/32 ports for single PMI test cases and 4 ports/16 ports multiple PMI test cases

In order to construct Type I–Single panel codebook, below RRC parameters need to be configured:

- Number of ports in vertical/horizontal directions: (N1,N2)

- Oversampling rate: (O1,O2)

- CodebookMode: 1/2

Proposal 2: Introducing below PMI test cases:

- Single PMI test case:

  - Test 1 a: 8 ports, ‘periodic or ‘semi-persistent’ CSI, (N1,N2) = (2,2) and (O1,O2) = (4,4), CodebookMode 1/2 (Rank1)

  - Test 1 b: 32ports, ‘periodic or ‘semi-persistent’ CSI, (N1,N2) = (4,4) and (O1,O2) = (4,4), CodebookMode 1/2 (Rank1)

  - Note1: pending on UE capability of supported port number, pass test 1a or 1b.

  - Note 2: Pending on UE capability of supporting ‘semi-persistent’ CSI report and NZP-CSI resources, to configure CSI and NZP CSI-RS resources type either periodic or persistent.

- Multiple PMI test case:

  - Test 2a: 4 ports, aperiodic CSI report, (N1,N2) = (2,1) and (O1,O2) = (4,-), CodebookMode 1 (Rank2)

  - Test 2b: 16 ports, aperiodic CSI report, (N1,N2) = (4,2) and (O1,O2) = (4,4), CodebookMode 1 (Rank2)

  - Note: pending on UE capability of supported port number, pass test 2a or 2b.

Beam forming model: similar as LTE, beam steering approach can be used to randomize main beam directions in both horizontal and vertical directions.

Proposal 3: beam steering approach can be used to randomize main beam directions

A detailed test set-up proposed in table below for Type-Single panel codebook test

| Parameter | Unit | Single PMI Test 1a | Single PMI Test 1b | Multiple PMI test 2a | Multiple PMI test 2b |
| --- | --- | --- | --- | --- | --- |
| System parameter |  |  |  |  |  |
| Bandwidth/SCS | MHz/kHz | 10MHz/15kHz (FDD) 40MHz/30kHz (TDD) | 10MHz/15kHz (FDD) 40MHz/30kHz (TDD) | 10MHz/15kHz (FDD) 40MHz/30kHz (TDD) | 10MHz/15kHz (FDD) 40MHz/30kHz (TDD) |
| Transmission Scheme |  | 1 | 1 | 1 | 1 |
| Duplex Mode |  | FDD &TDD | FDD &TDD | FDD &TDD | FDD &TDD |
| DLUL configuration (TDD) |  | FFS | FFS | FFS | FFS |
| Propagation channel |  | TDL A (30 ns), 10Hz | TDL A (30 ns), 10Hz | TDL C (300ns), 10Hz | TDL C (300ns), 10Hz |
| Correlation and antenna configuration |  | 8 ports (N1,N2) = (2,2) 8*2 XP High 8*4 XP High | 32 ports (N1,N2) = (4,4) 32*2 XP High 32*4 XP High | 4 ports (N1, N2) = (2,1) 4*2 XP High 4*4 XP High | 16 ports (N1, N2) = (4,2) 16*2 XP High 16*4 XP High |
| CSI-RS resource setting |  |  |  |  |  |
| ResourceConfigType |  | Periodic or persistent | Periodic or persistent | aperiodic | aperiodic |
| CSI-RS/CSI-IM periodicity and slot offset |  | FFS | FFS | FFS | FFS |
| NZP CSI-RS ports |  | 8 port {3000,….,3007} | 32 port {3000,….,3031} | 4 port {3000,…,3003} | 16 port {3000,…,3015} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | CDM4,density 1 (row 9) | FD-CDM2 ,density 0.5 (row 16) | FD-CDM2 density 1 (row 4) | CDM4,density 1 (row 12) |
| CSI-IM configuration |  | FFS | FFS | FFS | FFS |
| CSI report setting |  |  |  |  |  |
| reportConfigType |  | Periodic or persistent | Periodic or persistent | aperiodic | aperiodic |
| (N1,N2) |  | (2,2) | (4,4) | (2,1) | (4,2) |
| (O1,O2) |  | (4,4) | (4,4) | (4,1) | (4,4) |
| CodebookMode |  | 1/2 | 1/2 | 1 | 1 |
| Reporting interval |  | FFS | FFS | FFS | FFS |
| Codebook Restriction |  | FFS | FFS | FFS | FFS |
| Reporting granularity |  | Single PMI reporting | Single PMI reporting | Multiple PMI reporting | Multiple PMI reporting |
| Physical channel for reporting |  | PUSCH | PUSCH | PUSCH | PUSCH |
| Rank &MSCS |  | Rank1 | Rank1 | Rank2 | Rank2 |

Followed above configurations, we provided initial simulation results under: FDD 10MHz &15kHz

- 8Tx single PMI – MCS13 Rank1

- 4Tx multiple PMI-MCS 13 Rank1

Figure 1: Absolute TP vs SNR for 8Tx single PMI

Figure 2: Relative throughput ratio for 8Tx single PMI

Figure 3: Absolute TP vs SNR for 4Tx multiple PMI

Figure 4: Relative throughput ratio for 4Tx mutilple PMI

Conclusion

In this contribution, preliminary views for UE PMI requirements (Type I single panel codebook) were provided.

Proposal 1: Reusing Test metric as LTE, relative throughput ratio between followed PMI and random PMI under FRC for Type 1 single panel codebook

- FFS for test point

Proposal 2: Introducing below PMI test cases :

- Single PMI test case:

  - Test 1 a: 8 ports, ‘periodic or ‘semi-persistent’ CSI, (N1,N2) = (2,2) and (O1,O2) = (4,4), CodebookMode 1/2 (Rank1)

  - Test 1 b: 32ports, ‘periodic or ‘semi-persistent’ CSI, (N1,N2) = (4,4) and (O1,O2) = (4,4), CodebookMode 1/2 (Rank1)

  - Note1: pending on UE capability of supported port number, pass test 1a or 1b.

  - Note 2: Pending on UE capability of supporting ‘semi-persistent’ CSI report and NZP-CSI resources, to configure CSI and NZP CSI-RS resources type either periodic or persistent.

- Multiple PMI test case:

  - Test 2a: 4 ports, aperiodic CSI report, (N1,N2) = (2,1) and (O1,O2) = (4,-), CodebookMode 1 (Rank2)

  - Test 2b: 16 ports, aperiodic CSI report, (N1,N2) = (4,2) and (O1,O2) = (4,4), CodebookMode 1 (Rank2)

  - Note: pending on UE capability of supported port number, pass test 2a or 2b.

Proposal 3: beam steering approach can be used to randomize main beam directions

Furthermore, a detailed test set-up was proposed and initial simulation results provided.

Reference

[1] R4-1808027, “Way Forward on NR UE performance requirements”, Ericsson